HP 3000 Simulator User’s Guide
9-May-2025

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published 1993-2012, written by Robert M Supnik
Copyright © 1993-2012, Robert M Supnik
Copyright © 2012-2025, J. David Bryan

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL THE AUTHORS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

Except as contained in this notice, the names of the authors shall not be used in advertising or otherwise to promote the sale, use, or other dealings in this Software without prior written authorization from the authors.

1	Introduction	5

1.1	The HP 3000 Computer System	5

1.2	Simulator Files	6

2	Simulator Features	9

2.1	Hardware-Equivalent Actions	10

2.2	Simulator-Specific Commands	11

2.2.1	Numeric Display and Entry	11

2.2.2	Symbolic Display and Entry	11

2.2.3	Memory Addressing	13

2.2.3.1	EXAMINE, DEPOSIT, IEXAMINE, and IDEPOSIT	14

2.2.3.2	BREAK and NOBREAK	14

2.2.3.3	RUN and GO	14

2.2.4	START, LOAD, DUMP, and BOOT	14

2.2.4.1	Cold Load and Dump Commands for the Series III	14

2.2.4.2	Cold Load and Dump Commands for the Series 58	15

2.2.4.3	Cold Loading with the BOOT Command	15

2.2.5	POWER FAIL and POWER RESTORE	15

2.2.6	TRACE	16

2.2.7	Device Configuration	16

2.2.8	Enabling and Disabling Devices	16

2.2.9	I/O Interface Assignments	16

2.2.10	SAVE and RESTORE	17

2.3	Realistic, Calibrated, and Optimized Timing	18

2.4	The Simulation Console and the System Console	19

2.5	Tracing Simulator Operations	19

2.5.1	Trace Overview	20

2.5.2	Trace Commands	20

2.5.2.1	TRACE MEMORY	20

2.5.2.2	TRACE TRIGGER	21

2.5.2.3	TRACE STORE	22

2.5.2.4	TRACE WINDOW	22

2.5.2.5	TRACE EXECUTE	23

2.5.2.6	TRACE HALT	23

2.5.2.7	TRACE LIST	23

2.5.2.8	SHOW TRACE	24

2.5.3	Obsolete Commands	24

3	Processor Device Simulations	25

3.1	Central Processing Unit	25

3.1.1	System Halt	27

3.1.2	Idling	28

3.1.3	Simulation Stops	28

3.1.4	Tracing	29

3.1.5	Registers	35

3.2	Memory Control and Logging interface	36

3.2.1	Tracing	36

3.3	I/O Processor	37

3.4	Selector Channel	38

3.5	Multiplexer Channel	40

3.6	Channel Program Processor	41

3.6.1	Tracing	42

3.6.2	Registers	43

4	Programmed I/O Device Simulations	44

4.1	30032B Asynchronous Terminal Controller	44

4.1.1	Terminal Data Interface	44

4.1.2	Terminal Control Interface	48

4.2	30033A Selector Channel Maintenance Board	50

4.3	30135A System Clock	51

4.4	30340A Intermodule Bus Adapter	53

5	Selector Channel I/O Device Simulations	56

5.1	30229B Disc Interface with Eight 7905/7906/7920/7925 Drives	56

5.1.1	Device Options	57

5.1.2	Unit Options	58

5.1.3	Diagnostic Support	58

5.1.4	BOOT Command	59

5.1.5	Tracing and Registers	60

6	Multiplexer Channel I/O Device Simulations	62

6.1	30209A Line Printer Controller with One 2607/13/17/18 Line Printer	62

6.1.1	Device Options	63

6.1.2	Unit Options	64

6.1.3	Vertical Format Unit	65

6.1.4	Tracing and Registers	67

6.2	30215A Tape Controller with Four 7970B/E Drives	69

6.2.1	Device Options	70

6.2.2	Unit Options	71

6.2.3	BOOT Command	71

6.2.4	Tracing and Registers	72

7	Intermodule Bus I/O Device Simulations	74

7.1	31262A General I/O Channel	74

7.1.1	Device Options	74

7.1.2	Tracing	76

7.1.3	Registers	78

7.2	31264A Asynchronous Data Communications Channel	78

7.2.1	Device Options	80

7.2.2	Unit Options	81

7.2.3	Tracing	82

7.2.4	Registers	85

7.3	CS/80 Disc and Tape Drives	85

7.3.1	Configuration	88

7.3.2	Device Options	88

7.3.3	Unit Options	89

7.3.4	BOOT Command	90

7.3.5	Tracing	90

7.3.6	Registers	92

7.4	7970E HP-IB Tape Controller with four Magnetic Tape Drives	92

7.4.1	Device Options	93

7.4.2	Unit Options	94

7.4.3	BOOT Command	95

7.4.4	Tracing	95

7.4.5	Registers	98

7.5	26069A HP-IB to Differential Translator with One 2611/13/17/18/19 Line Printer	98

7.5.1	Device Options	100

7.5.2	Unit Options	101

7.5.3	Vertical Format Unit	102

7.5.4	Tracing	104

7.5.5	Registers	105

# Introduction

This manual documents the features and operation of the HP 3000 simulator.  It is intended for use in conjunction with the SIMH Users' Guide and SIMH Users’ Guide Supplement manuals, which describe how to compile and run the simulator, as well as the general commands that may be entered at the Simulator Control Program (SCP) prompt.

Commands and command options presented in this guide use the following syntax formatting:

| Format | Interpretation |
| --- | --- |
| KEYWORD | A string that must be entered literally, either in upper or lower case |
| <class> | A name from the indicated class, e.g., a device name or unit number |
| [optional] | An optional parameter that might be a keyword or class value |
| {alternate ¦ alternate} | An item selected from a set of alternates |
| item... | An item that may be repeated zero or more additional times |

This simulator is based on SIMH — the Computer History Simulator.

## The HP 3000 Computer System

Hewlett-Packard sold the HP 3000 family of general-purpose business computers from 1972 through 2001.  There are two major divisions within this family: the classic 16-bit, stack-oriented CISC machines, and the Precision Architecture 32-bit, register-oriented RISC machines that succeeded them.  All machines run versions of MPE, the Multiprogramming Executive operating system.

Within the classic division, there are two additional subdivisions, based on the method used for peripheral connections: the original SIO machines, and the later HP-IB machines.  The I/O interfacing hardware differs between the two types of machines, as do the privileged I/O machine instructions.  The user instruction sets are identical, as are the register sets visible to the programmer.  The I/O drivers are different to account for the hardware differences, and therefore they run slightly different versions of MPE.

This implementation simulates two classic machines: the Series III and the Series 58.  The Series III may be configured with the 30341A HP-IB Interface Module that connects HP-IB peripheral devices to the system.

The Series III represents the high end of the SIO machines, which consist of the 3000 CX, the Series I, the Series II, and the Series III.  These machines use I/O interfaces based on the I/O Processor (IOP) bus.

The CX and the Series I, which is a repackaged CX, are essentially subsets of the Series II/III — a smaller instruction set, limited memory size, and lower-precision floating-point instructions.  Simulation of these machines may be added in the future.

The CX and Series I support 64K 16-bit words of memory.  The Series II supports up to 256K, divided into four banks of 64K words each, and the Series III extends this to 1024K words using 16 banks (although MPE V/R can accommodate bank numbers up to six bits in length).

The Series 58 is in the upper-mid range of the HP-IB machines, which consist of some 19 models from the low-end Series 30 to the high-end Series 70.  The I/O interfaces for these machines connect to an Intermodule Bus (IMB).  The various models differ mainly in memory, I/O capacity, and processor speed, with maximum memory sizes varying from 512K for the Series 30 to 8192K for the Series 70.  The Series 58 supports 4096K.

Memory is divided into variable-length code and data segments, with the latter containing a program’s global data area and stack.  Memory protection is accomplished by checking program, data, and stack accesses against segment base and limit registers, which can be set only by MPE.  Bounds violations cause automatic hardware traps to handler routines within MPE.  Some violations may be permitted; for example, a Stack Overflow trap may cause MPE to allocate a larger stack and then restart the interrupted instruction.  Memory references are position-independent, so moving segments to accommodate expansion requires only resetting of the segment registers to point at the new locations.  Code segments are fully reentrant and shareable, and both code and data are virtual, as the hardware supports absent code and data segment traps.

The classic 3000s are stack machines.  Most of the instructions operate on the value on the top of the stack (TOS) or on the TOS and the next-to-the-top of the stack (NOS).  To improve execution speed, the 3000 has a set of hardware registers that are accessed as the first four locations at the top of the stack, while the remainder of the stack locations reside in main memory.  A hardware register renamer provides fast stack pushes and pops without physically copying values between registers.

## Simulator Files

The simulator sources are divided into a set of files for the Simulator Control Program and its support libraries, and a set of files for the HP 3000 device simulations; the latter reside in a subdirectory of the directory that contains the SCP files.  The former set is common to all SIMH simulators, whereas the latter set is specific to the virtual machine being simulated.  The files that make up this simulator are:

| Subdirectory | File | Contains |
| --- | --- | --- |
| HP3000 | hp3000_cpu.h | CPU architectural declarations |
|  | hp3000_cpu_fp.h | Floating-point interface declarations |
|  | hp3000_cpu_io.h | CPU-to-I/O controllers interface declarations |
|  | hp3000_defs.h | System architectural declarations |
|  | hp3000_gic.h | General I/O Channel architectural declarations |
|  | hp3000_imb.h | Intermodule Bus architectural declarations |
|  | hp3000_io.h | Device-to-IOP/channel interface declarations |
|  | hp3000_mem.h | Main memory subsystem interface declarations |
|  | hp_disclib.h | MAC/ICD disc controller simulator library declarations |
|  | hp_tapelib.h | 797x tape controller simulator library declarations |
|  | hp3000_adcc.c | Asynchronous Data Communications Channel simulator |
|  | hp3000_atc.c | Asynchronous Terminal Controller simulator |
|  | hp3000_clk.c | System Clock simulator |
|  | hp3000_cpp.c | Channel Program Processor simulator |
|  | hp3000_cpu.c | CPU simulator |
|  | hp3000_cpu_base.c | CPU base instruction set simulator |
|  | hp3000_cpu_cis.c | CPU COBOL II Extended Instruction Set simulator |
|  | hp3000_cpu_eis.c | CPU Extended Instruction Set simulator |
|  | hp3000_cpu_fp.c | CPU floating point instruction set simulator |
|  | hp3000_ds.c | Cartridge Disc Interface simulator |
|  | hp3000_gic.c | General I/O Channel simulator |
|  | hp3000_gic_dc.c | CS/80 disc drive simulator |
|  | hp3000_gic_lpa.c | Amigo line printer interface simulator |
|  | hp3000_gic_ma.c | Amigo magnetic tape subsystem simulator |
|  | hp3000_imb.c | Intermodule Bus simulator |
|  | hp3000_imba.c | Intermodule Bus Adapter simulator |
|  | hp3000_iop.c | I/O processor simulator |
|  | hp3000_lp.c | Line Printer Interface simulator |
|  | hp3000_mem.c | Main memory subsystem simulator |
|  | hp3000_mpx.c | Multiplexer Channel simulator |
|  | hp3000_ms.c | Magnetic Tape Controller Interface simulator |
|  | hp3000_scmb.c | Selector Channel Maintenance Board simulator |
|  | hp3000_sel.c | Selector Channel simulator |
|  | hp3000_sys.c | SCP interface |
|  | hp_disclib.c | MAC/ICD disc controller simulator library |
|  | hp_tapelib.c | 797x tape controller simulator library |
| doc | hp3000_guide.pdf | HP 3000 Simulator User’s Guide |
|  | hp3000_diag.txt | SIMH/HP 3000 Diagnostics Performance Report |
|  | hp3000_release.txt | HP 3000 Simulator Release Notes |

PDF files of the original HP 3000 hardware and software manuals are available from these repositories:

- Bitsavers

- The HP Computer Museum

The reference materials used to develop the various simulated devices are listed in the comments at the start of the respective simulator source modules.

See the Available Software section of the release notes file for information on the availability of operating system software that runs on the simulator.

# Simulator Features

The HP 3000 simulator contains the following device simulations:

| Device Name | Simulates |
| --- | --- |
| CPU | 30003B Series III Computer with up to 1024 KW of memory 32558A Series 58 Computer with up to 4096KW of memory |
| CPP | Channel Program Processor |
| IOP | 30003B I/O Processor |
| MCL | 30007A Memory Control and Logging interface 30172A Memory Control and Logging interface |
| SEL | 30030C Selector Channel |
| ATCD | 30032B Asynchronous Terminal Controller data interface |
| ATCC | 30032B Asynchronous Terminal Controller control interface |
| SCMB1, SCMB2 | 30033A Selector Channel Maintenance Boards |
| MPX | 30036B Multiplexer Channel |
| CLK | 30135A System Clock |
| LP | 30209A Line Printer Controller with one 2607/13/17/18 line printer |
| MS | 30215A Magnetic Tape Controller with four 7970B/E drives |
| DS | 30229B Disc Interface with eight 7905/7906/7920/7925 drives |
| IMBA | 30340A Intermodule Bus Adapter |
| GIC | 31262A General I/O Channel |
| ADCC | 31264A Asynchronous Data Communications Channel |
| DC | HP-IB connector with eight CS/80 disc and tape drives |
| LPA | 26069A HP-IB to differential translator with one 2611/13/17/18/19 line printer |
| MA | HP-IB Magnetic Tape Controller with four 7970E drives |

Typically, one instance of each listed device may be installed in the simulated computer chassis.  Some devices support multiple connected units.  As an example, the DS device simulates a single 30229B Disc Interface that connects up to eight drives.  However, a second 30229B connecting an additional eight drives is not supported.

Certain devices support the creation of more than one instance; these are noted in their respective individual device descriptions below.  For example, the GIC device supports the creation of up to 14 separate instances, representing the installation of 14 GIC Printed Circuit Assemblies in the computer chassis.  Each GIC instance is a separate device, and all GICs may be used concurrently.

The simulator has been tested with and supports the following operating systems:

- MPE V/P version E.00.01.

- MPE V/R version E.01.00.

- MPE V/E version G.3P.00.

In addition, the simulator generally passes the HP 32230 Stand-Alone Diagnostics suite, the GIC, ADCC, and 7970E diagnostics of the HP 32231 Diagnostic/Utility System, and the GIC and CS/80 diagnostics of the HP 32341 Diagnostic/Utility System III; see hp3000_diag.txt for details.

The simulator may be configured to stop for any of these conditions:

- Attempted execution of an instruction that enters an infinite loop (e.g., BR P+0).

- Attempted execution of a PAUS instruction.

- Attempted execution of an undefined instruction.

- Attempted execution of an unimplemented instruction.

- Completion of the cold-load process.

MPE normally handles these conditions internally.  However, when automating the cold-load process or running diagnostics, it may be helpful to set one or more of these stop conditions.  In particular, stopping a simulator command file after cold loading allows additional system configuration to be performed before execution is begun.  Also, some diagnostics execute a PAUS instruction to wait for operator action, rather than external interrupts, or branch in an infinite loop to indicate an error condition.  Unless the corresponding simulator stops are active, no indications of these conditions will be observed.

The simulator also provides extensive facilities for tracing CPU and I/O device operations.

## Hardware-Equivalent Actions

The current implementation does not provide simulations of the CPU or peripheral device front panels.  Instead, commands entered through the simulation console are used to perform hardware actions.  The simulation commands that substitute for CPU front-panel actions are:

| Hardware Front-Panel Action | Equivalent Simulation Command |
| --- | --- |
| Pressing the RUN/HALT button while halted | RUN |
| Pressing the RUN/HALT button while running | CTRL+E CTRL+E |
| Pressing the START button | START |
| Pressing the LOAD or LOAD and ENABLE buttons | LOAD |
| Pressing the DUMP or DUMP and ENABLE buttons | DUMP |
| Toggling the CPU RESET switch | RESET |
| Setting the PF/ARS switch to ENABLE | SET CPU ARS |
| Setting the PF/ARS switch to DISABLE | SET CPU NOARS |
| Displaying the Current Instruction Register | EXAMINE CIR |
| Displaying the System Switch Register | EXAMINE SWCH |
| Setting the System Switch Register | DEPOSIT SWCH <value> |

Mounting media on a peripheral device is simulated by the ATTACH command.  For example, entering the ATTACH LP <printer-image-filename> command is equivalent to loading paper into an HP 2617A Line Printer.  Inserting a disc pack into an HP 7905A disc drive set for unit 2 is simulated by the ATTACH DS2 <disc-image-filename> command.

Each of these commands is explained in more detail below.

In hardware, loading programs into memory from a device is accomplished by setting the System Switch Register on the Series III to the desired control and device number and pressing the LOAD and ENABLE buttons together to initiate the cold load sequence.  On the Series 58, pressing the START button initiates a cold load from the system disc.  In simulation, a cold load may be performed either explicitly or implicitly, as described in the START, LOAD, DUMP, and BOOT section below.  Explicit operation is initiated by a START, LOAD, or DUMP command.  As a convenience, BOOT <device> commands may be used to implicitly cold load their respective devices and are described in the individual device descriptions below.

## Simulator-Specific Commands

In general, all of the commands documented in the SIMH Users' Guide manual are available for use with the HP 3000 simulator.  Commands whose execution or parameters are implementation-defined are specified below.

### Numeric Display and Entry

When examining or depositing into memory, the radix for addresses is octal, and the default radix for numeric data is octal.  The data default may be changed for a specific device with the SET <device> <radix> command, or the radix may be overridden temporarily with a command line switch, as follows:

| SET <radix> Option | Command Switch | Data Interpretation |
| --- | --- | --- |
| BINARY | –B | Binary values |
| OCTAL | –O | Octal values |
| DECIMAL | –D | Decimal values |
| HEX | –H | Hexadecimal values |

When examining or depositing into device registers, the default radix for the specified register is used unless overridden with one of the above command line switches.  Defaults are listed in the register table associated with each device.

When examining or depositing into attached device files, the radix for addresses is decimal, and the default radix for numeric data is octal.  The data default may be changed with the SET <device> <radix> command, or the radix may be overridden temporarily with one of the above command line switches.

### Symbolic Display and Entry

When examining or depositing into memory, certain registers, or attached device files, command line switches specifying the symbolic mode and format may be used to override the default numeric mode, as follows:

| Switch | Mode Interpretation |
| --- | --- |
| –A | A single character in the right-hand byte |
| –C | A two-character packed string |
| –E | An EDIT subprogram operation mnemonic |
| –ER | An EDIT subprogram operation mnemonic starting with the right-hand byte |
| –I | An I/O program instruction mnemonic |
| –L | A channel program instruction mnemonic |
| –M | A CPU instruction mnemonic |
| –T | A CPU status mnemonic |

In addition, certain 8-bit devices allow pairs of successive bytes in their attached device files to be interpreted as 16-bit words.  These devices normally display or enter values as bytes but can be overridden to work with words by including one the following mode switches:

| Switch | Mode Interpretation |
| --- | --- |
| –C | A two-character packed string |
| –M | A CPU instruction mnemonic |
| –W | A 16-bit numeric value |

In the absence of a mode switch, entering values with a leading ' (apostrophe) implies –A, a leading " (quotation mark) implies –C, and a leading alphabetic character implies –M.  The specific registers supporting symbolic mode are indicated in their respective device sections below.

If the –C switch is specified, the value is displayed as two characters separated by a comma.  Alphanumeric, punctuation, and symbol characters are displayed within apostrophes, control characters are displayed as ASCII name abbreviations, and characters above 128 decimal are displayed in escaped numeric form with a leading backslash followed by an octal number.  Depositing with –C accepts two displayable characters.  If a single character is supplied, the low byte of the resulting value will be zero; follow the character with a space to pad the low byte with a blank.

If the –M switch is specified, the value is displayed or accepted as a CPU machine instruction mnemonic if it is defined and implemented in the currently enabled firmware set.  If it is not, it is displayed as a numeric value in the CPU's data radix or is rejected if depositing.  Any numeric operands present are displayed or entered in a default radix unless overridden by the addition of one of these mutually exclusive format switches:

| Switch | Format Interpretation |
| --- | --- |
| –A | A single character in the right-hand byte |
| –B | A binary value |
| –O | An octal value |
| –D | A decimal value |
| –H | A hexadecimal value |

Numeric operands are displayed or entered in a default radix suitable to the type of the value.  For CPU instruction operands:

- Register-relative displacements, S-register decrements, and K fields are displayed or entered in the CPU’s address radix, which is octal.

- Shift counts, bit positions, and starting bits and counts are displayed or entered in decimal unless overridden by a switch on the command line.

- CIR values for the PAUS and HALT instructions are displayed or entered in octal unless overridden by a switch on the command line.

- Immediate values are displayed or entered in the CPU's data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.

For I/O and channel program instruction operands:

- Address values are displayed in the CPU’s address radix, which is octal.

- Counts are displayed in decimal unless overridden by a switch on the command line.

- Control and status values are displayed in the CPU's data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.

For CPU status values:

- The current code segment number is displayed in the CPU’s data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.

For EDIT subprogram operands:

- Branch displacement fields are displayed in the CPU’s address radix, which is octal.

- Character counts, loop counts, and source and target adjustments are displayed in decimal unless overridden by a switch on the command line.

- Invalid operation codes are displayed in the CPU's data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.

Each EDIT subprogram operation occupies from 1 to 257 bytes in memory.  If a multi-byte operation begins within the examined memory range, the additional bytes required to complete the operation are displayed.  Adding the –R switch will begin decoding with the right-hand (low-order) byte of the first memory word in the range rather than with the left-hand byte.  Invalid extended operation codes are displayed as a comma-separated pair of half-byte values (e.g., “17,12”).

If the indicated display radix is not the same as the current CPU data radix, a leading character @, %, #, or !, is printed for binary, octal, decimal, or hexadecimal numbers, respectively.  When entering machine instruction mnemonics, these characters may also prefix numeric operands to override the radix defaults.  However, note that the percent character must be doubled to avoid interpreting it as a standard substitution variable (i.e., LDI %%12 will be interpreted as LDI 12 octal, but LDI %12 will be interpreted as LDI 2, assuming that substitution variable %1 is null).  A safer approach when an octal numeric override is needed would be to include the –O switch on the command line.

When the simulator examines the bit patterns of instructions in memory or registers to display in mnemonic form, each will fall into one of four categories:

- Defined (canonical) instruction encodings, where all bits are defined or all reserved bits are zero (e.g., LOAD).

- Undefined (non-canonical) instruction encodings, where reserved fields are “don't care” bits (e.g., MOVE).

- Undefined (non-canonical) instruction encodings, where reserved fields are decoded (e.g., IXIT).

- Unimplemented instruction encodings (e.g., stack opcode 072, or EADD without the EIS firmware option installed).

The names of the canonical instructions in category 1 are displayed in uppercase, as are the names of the non-canonical instructions in category 2.  The non-canonical instruction names in category 3 are displayed in lowercase.  This is to indicate to the user that the instructions that will be executed may not be the ones expected from the decoding.  Instruction names in category 4 that correspond to supported firmware options are displayed in uppercase, regardless of whether or not the option is enabled.  Category 4 encodings that do not correspond to instructions are displayed in octal.

### Memory Addressing

The CPU simulator supports two forms of memory addresses:

- An absolute address consisting of a 4- or 6-bit bank number and a 16-bit offset within the bank, separated by a period (e.g., 17.177777).

- A relative address consisting of a 16-bit offset within a bank specified by a bank register (e.g., 177777).

Command line switches modify the interpretation of relative addresses as follows:

| Switch | Interpretation |
| --- | --- |
| –P | The implied bank number is obtained from PBANK |
| –S | The implied bank number is obtained from SBANK |

If no switch is specified, the implied bank number is obtained from DBANK.

#### EXAMINE, DEPOSIT, IEXAMINE, and IDEPOSIT

The following CPU memory address forms are valid:

EXAMINE <bank>.<offset>
EXAMINE <dbank-offset>
EXAMINE -P <pbank-offset>
EXAMINE -S <sbank-offset>

Addresses are always displayed in <bank>.<offset> form.

#### BREAK and NOBREAK

The following address forms are valid:

BREAK
BREAK <bank>.<offset>
BREAK <pbank-offset>

The address defaults to the current values of PBANK and/or P.  All breakpoint addresses are stored and displayed in <bank>.<offset> form.

#### RUN and GO

The following address form is valid:

RUN <pbank-offset>

The offset is stored in P.  If P does not lie within the PB-to-PL range, the command will be rejected.

The SIMH RUN command typically resets all devices before beginning execution.  For the HP 3000, CPU and I/O resets are performed at the start of the cold load sequence; a reset after the cold load sequence completes will destroy the Interrupt Control Stack register setup performed by the microcode.  Therefore, in this simulator, the RUN and GO commands are equivalent; neither resets the devices.  If an explicit CPU reset is desired, the RESET CPU command may be used.

### START, LOAD, DUMP, and BOOT

The START, LOAD, and BOOT commands are used to initiate the HP 3000 cold-load sequence that bootstraps an operating system or diagnostic program from a storage device.  The DUMP command initiates the 3000 cold-dump sequence that writes the contents of memory to a storage device for later analysis.  These commands behave differently, depending on the system type (IOP-based or IMB-based).  The START command exists only in IMB-based systems.

#### Cold Load and Dump Commands for the Series III

The LOAD command implements the cold load facility.  It is equivalent to pressing the LOAD and ENABLE buttons on the hardware front panel.  The syntax is:

LOAD [<control/devno>]

The SWCH register controls the cold load sequence; the upper byte contains a device-specific control value, and the lower byte contains the device number.  If the optional control/device number value is supplied, it is deposited into the SWCH register before initiating the cold load sequence.  Otherwise, the SWCH register value must be set before entering the command.  In either case, the device number must be between 3 and 63 for an SIO device, or 125 for an HP-IB device, or the command will not complete.

The DUMP command implements the cold dump facility.  It is equivalent to pressing the DUMP and ENABLE buttons on the hardware front panel.  The syntax is:

DUMP [<control/devno>]

The CPU DUMPCTL and DUMPDEV settings control the cold dump sequence and correspond to the settings of the jumper blocks on the rear of the CPU front panel.  If the optional control/device number value is supplied, it is deposited into the SWCH register before initiating the cold dump sequence.  Otherwise, the DUMPCTL and DUMPDEV values are deposited into the SWCH register.  A successful cold dump sets the CIR register to the number of 64K memory banks dumped.  For example, dumping a 512K-word system will display Cold dump complete, CIR: 000010 if the operation succeeds.

The control/device number values are entered in the current CPU data radix, which defaults to octal.  Before entering either command, unit 0 of the associated device must be attached.  A failed cold load will cause a system halt.  A failed cold dump will set the CIR register to the memory bank number at which the operation failed; if the dump device is not ready or not write-enabled, the CIR register will be set to zero.

#### Cold Load and Dump Commands for the Series 58

The system control panel of the Series 58 contains START, LOAD, and DUMP buttons.  Each button is associated with a pair of thumbwheel switches that are used to set the default channel and device numbers for the action.  In simulation, these buttons are implemented as commands.  All three commands initiate the cold-load process — the

START and LOAD commands from a disc or tape device, respectively, containing an operating system, and the DUMP command from a disc or tape device containing the Software Dump Facility program.  The syntax of the commands is:

START [<channel/device>]
LOAD  [<channel/device>]
DUMP  [<channel/device>]

If the optional channel/device value is supplied, it is deposited into the SWCH register before initiating the cold load sequence.  Otherwise, the channel and device values from the corresponding thumbwheel switches on the system control panel are deposited into the SWCH register.  The combined channel and device numeric values are entered in the current CPU data radix, which defaults to octal, with the value equal to the channel number times eight plus the device number.  A successful cold load causes an immediate entry into the cold-load trap handler unless the COLDLOAD simulation stop is enabled.  Setting thumbwheel switches and simulation stops are detailed in the Central Processing Unit section below.

#### Cold Loading with the BOOT Command

Bootable peripherals, such as the tape and disc drives, also implement the BOOT command.  Entering a BOOT command for one of these devices will preset the SWCH register to the appropriate value before executing a cold load.  Cold load supports booting only from unit 0 of the target device.

### POWER FAIL and POWER RESTORE

The POWER FAIL command (and its alternate forms POWER OFF and POWER DOWN) simulates removing AC power from the system.  If the CPU was executing when the command is entered, a power-fail interrupt is generated, and MPE initiates its power-fail processing that ends with a programmed HALT.  Otherwise, power is removed, and the CPU remains halted.  POWER FAIL is accepted only when power is on.

The POWER RESTORE command (and its alternate forms POWER ON and POWER UP) simulates restoring AC power to the system; it is accepted only when power is off.  If the CPU was executing when power failed, a power-on trap is generated, and, if auto-restart is enabled, MPE initiates its power-recovery processing.  If auto-restart is disabled, the trap is set up but the simulator must be resumed with a CONTINUE command to execute the trap.  If the CPU was halted when power failed, power is restored, and the CPU remains halted.  Once power is off, the CONTINUE, STEP, GO, and RUN commands will not be accepted until power is restored.

### TRACE

The TRACE command configures and operates the internal device tracing facility.  Tracing is described in detail in the Tracing Simulator Operations section below.

### Device Configuration

Most devices support user configuration.  The general forms of the configuration commands are:

SET [<switch> ...] <device> <option> [,<option> ...]
SET [<switch> ...] <unit> <option> [,<option> ...]

The options available and applicable switches are described in the individual device descriptions below.

### Enabling and Disabling Devices

All devices other than the processor devices may be disabled or enabled.  Disabling a device simulates removing the associated interface from the system cabinet.  To disable or enable a device, use:

| Command | Action |
| --- | --- |
| SET <device> DISABLED | Disable the device |
| SET <device> ENABLED | Enable the device |

Devices that consist of multiple addressable units connected to a controller allow the units to be individually disabled or enabled.  Disabling simulates disconnecting the associated unit from the controller.  The commands to disable or enable a unit are:

| Command | Action |
| --- | --- |
| SET <unit> DISABLED | Disable the unit |
| SET <unit> ENABLED | Enable the unit |

Each of the above command options is replicated in the option tables of the devices to which they apply.

### I/O Interface Assignments

HP 3000 Series III hardware I/O interfaces have configuration jumpers that set device numbers, interrupt masks, and Multiplexer Channel service request priority numbers.  In addition, the interrupt poll order, and therefore the interrupt priority, is established by the position of each interface along the daisy-chained interrupt cable.

Device options that may be specified to change or display interface configurations are:

| Option | Valid Values | Action |
| --- | --- | --- |
| DEVNO | 0-127 | Set the device number |
| INTMASK | 0-15, D, E | Set the interrupt mask |
| INTPRI | 0-31 | Set the interrupt priority |
| SRNO | 0-15 | Set the service request number |

For example:

SET MS DEVNO=6
SET MS INTMASK=E
SHOW MS DEVNO,INTMASK

The interrupt mask may be set to a numeric value, to D to disable the mask always, or to E to enable the mask always.  Numeric mask values may be shared among devices.  However, if two device, interrupt priority, or service request priority numbers conflict, the simulator will report the error when execution is attempted.

The default settings are:

| Device Name | Device Number | Interrupt Priority | Interrupt Mask | Service Request Number |
| --- | --- | --- | --- | --- |
| CPU | – | – | – | – |
| IOP | – | – | – | – |
| SEL | – | – | – | – |
| MPX | 127 | – | – | – |
| ATCD | 7 | 0 | E | – |
| ATCC | 8 | 8 | E | – |
| CLK | 3 | 1 | – | – |
| DS | 4 | 4 | E | – |
| LP | 14 | 18 | E | 11 |
| MS | 6 | 14 | E | 3 |
| SCMB1 | 65 | 10 | – | 0 |
| SCMB2 | 66 | 11 | – | 1 |
| IMBA | 125 | 12 | – | – |

The SCMB device numbers and interrupt priorities may be set to any unused values.  The device numbers of the MPX and IMBA devices cannot be changed.

HP 3000 Series 58 hardware I/O interfaces and the 30340A Intermodule Bus Adapter for the Series III have configuration switches that set the IMB channel numbers to which they respond.  IMB I/O devices include a CHANNEL option to set the channel address switch.  The default settings are:

| Device Name | Channel Number |
| --- | --- |
| GIC | 11 |
| DC | 11 |
| LPA | 11 |
| MA | 9 |
| ADCC | 1 |
| IMBA | 1 |

The IMBA channel number cannot be changed.

### SAVE and RESTORE

SAVE and RESTORE are supported only when the simulator executable used to restore the simulator state file is the same simulator executable used to save the file.  Correctly restoring the state of the simulator depends on the layout of internal structure variables being identical to the layout of the structure variables that were saved.  This is guaranteed only when using the same executable, as the layout used is implementation-defined.

## Realistic, Calibrated, and Optimized Timing

Operating  systems and diagnostics that interact with I/O devices often have embedded timing dependencies.  In practice, such software may contain assumptions regarding the time taken by certain operations.  For instance, an I/O driver may “know” that a line printer takes 50 milliseconds to print a line, and therefore it can ignore interrupts safely for several milliseconds after initiating the print cycle.  In hardware, these latencies are functions of physical movements — disc seeks, magnetic tape reads, printer paper advances, etc.  A simulated device must model these physical latencies as time delays between the initiation and completion of various activities.  For best performance, these delays want to be as short as possible.  However, they cannot be arbitrarily short without violating the timing requirements of system software.

Moreover, diagnostics often measure the time taken by physical operations to determine if a device is operating properly.  In such cases, delays optimized to be “as short as possible” will cause the diagnostic to fail.  For these applications to succeed, the simulated delays must approximate the realistic hardware delays, in terms of the number of CPU instructions executed between initiation and completion.

A third consideration is the delay to use for a time-of-day clock.  Typically, such a device generates interrupts at some preset rate.  Were a simulated clock to schedule these interrupts “as fast as possible,” the observed time of day would run far ahead of a wall clock.  Even scheduling the interrupts to occur at a realistic rate, as measured in the number of CPU instructions executed between ticks, would still increment the simulated clock at a very fast rate, given that the simulator is two orders of magnitude faster than a real HP 3000.  For this device, the simulated clock must tick at the same rate as a physical wall clock, so that the time of day advances as expected.

To accommodate these conflicting requirements, user control of simulated device timing is provided.

Devices simulate their I/O operation delays by counting specified numbers of “event ticks.”  For example, a disc seek completion might be scheduled to occur 500 event ticks after command initiation.  Generally, one event tick is counted for each machine instruction executed, although some complex instructions may count an event tick for each internal step of the instruction (e.g., for each word moved within a block-move instruction).  Device simulations provide commands that determine how the appropriate tick counts are selected for I/O operations timing:

| Command | Action |
| --- | --- |
| SET <device> REALTIME | Configure the device to use realistic timing |
| SET <device> CALTIME | Configure the device to use calibrated timing |
| SET <device> FASTTIME | Configure the device to use optimized timing |

A device configured to use realistic timing selects its tick counts to encompass the same number of machine instructions as would be executed in hardware.  In realistic-time mode, an operation taking ten milliseconds in hardware will complete after 4000 machine instructions are executed, based on an average instruction execution time of 2.5 microseconds.  In this mode, a software program will execute approximately the same number of instructions during a device operation that it would do on a real machine.  Host machine speed and execution of concurrent host programs will not affect the number of simulated instructions executed for a given operation.

A device configured to use calibrated timing selects its tick counts to align the simulated operation periods with the corresponding time periods on the host system.  In calibrated-time mode, an operation taking ten milliseconds in hardware will complete after ten milliseconds has elapsed on the host system.  Because the simulator is generally two orders of magnitude faster than the original hardware, a software program will execute far more code during a device operation than it would do on a real machine.  In this mode, the amount of code that executes will vary with the speed of the host machine and the load placed on that machine by other concurrent processes, as the simulator continually adjusts the tick counts up and down to maintain synchronization with the host time.

A device configured to use optimized timing selects its tick counts to minimize the operation delays.  In this “fast-time” mode, an operation taking ten milliseconds in hardware will complete after the minimum amount of time acceptable to the executing software has elapsed.  Consequently, device operations will complete far more quickly than they would do in hardware.  As noted above, though, there are limits to how fast I/O operations may occur without causing software malfunctions.  The default optimized-timing settings have been empirically determined to work with the supported operating systems, and each device simulator allows the user to modify those settings via registers if needed.

To illustrate how the modes affect timing, consider a simulation of a Teletype terminal that operates at 10 characters per second.  If the simulator runs 15 times faster than a real machine, then a user would observe that printing 100 characters takes:

- 10 seconds in CALTIME mode
(100 characters × n event ticks per character adjusted to take exactly 100 mS each on the host system)

- 667 milliseconds in REALTIME mode
(100 characters × 40,000 event ticks per character × 2.5 µS per tick ÷ 15 times hardware speed)

- 8.33 milliseconds in FASTTIME mode
(100 characters × 500 event ticks per character × 2.5 µS per tick ÷ 15 times hardware speed)

If the SCP SET THROTTLE command is used to reduce the speed of the simulator, CALTIME operations will not be affected, but REALTIME and FASTTIME operations will slow proportionally.  Reducing simulator speed to that of the original hardware will cause REALTIME operation times to equal CALTIME times.

Devices offer only those modes that are generally useful.  For example, the CPU process clock and system clock may be configured to use REALTIME or CALTIME modes.  The real-time mode will satisfy the expectations of hardware diagnostics that check the timing of operations via delay loops, whereas the calibrated mode will update the MPE time-of-day clock as expected by users of the simulated system.  FASTTIME mode is not offered, as it makes no sense to ignore the programmed time period settings and use a fixed arbitrary period instead.  If this were done, the MPE time-of-day clock would run abnormally quickly, and diagnostics would fail while reporting that the tested hardware is running abnormally slowly.

Devices performing input or output typically offer a REALTIME mode for use when running the diagnostics and a FASTTIME mode for use when running operating systems.  In general, software running under simulation will run faster when devices are configured for optimized-time mode, and this is the default for all peripheral devices.

## The Simulation Console and the System Console

When the simulator is started, the SCP command prompt appears at the simulation console.  For windowed host operating systems, this is typically the same window from which the simulator was started.  Once an execution command such as RUN is entered at the simulation console, the standalone diagnostics and MPE operating system begin their user interactions at the system console.  The HP 3000 reserves port 0 of the first terminal multiplexer for the system console.

For convenience and by default, the system console is initially connected to the simulation console, so that SCP and MPE operator commands may be entered from the same window.  Additional user terminals may be connected via Telnet or serial ports to the multiplexers as described later.

The system console may be separated from the simulation console by using the SET CONSOLE TELNET=<port> or SET CONSOLE SERIAL=<port> command.  This leaves the simulation console at the initiating window and moves the system console to a Telnet or serial port, allowing the use of an HP terminal or terminal emulator.  Entering the SET CONSOLE NOTELNET or SET CONSOLE NOSERIAL command will rejoin the consoles

## Tracing Simulator Operations

The simulator provides options for extensive tracing of the internal states of all CPU and peripheral devices.  This is useful as an aid to hardware and software debugging as well as to gain an understanding of the internal operations of the simulated devices.  Each device offers multiple trace reporting levels, from command overviews to detailed backplane signal assertions.  Tracing for each device and its separate reporting states may be enabled independently and

are documented in the following sections that describe the simulated devices.

The state options table given for each device is arranged in order of increasing detail.  The first option listed provides the broadest overview with the least specific detail and generates the smallest number of trace lines.  Subsequent options provide increasing detail at the expense of larger trace listings.  Enabling all trace options for a given device provides the fullest picture of device operation but may generate a very large number of trace lines.

Some options enable tracing of periodic events, e.g., a clock tick or a device poll.  Use caution when specifying these options, as the trace list may fill rapidly.  Options that trace periodic behavior are noted in the option descriptions.

Trace Overview

Once trace execution begins, the internal device states selected by the user are written continuously to trace memory, which is implemented as a circular buffer whose size is established by the user.  State acquisition continues, with the buffer wrapping around to overwrite earlier states as needed automatically, until it is manually halted or until a specified trigger condition is met.  After trace completion, the stored state list may be written to an external text file for review.

The formats of the trace output are specific to the devices being traced.  Examples are provided in each device description section below.

Normally, all specified device states are included in the state list.  However, some devices provide user-specified trace windowing, which restricts acquisition to just those states of interest, such as terminal I/O to a specific multiplexer port or channel programs operating on a specific bus address.  For devices that support multiple peripheral units, windowing ensures that only relevant states are included in the trace listing.

Because decoding and formatting of the state list is deferred until the trace listing is generated, active acquisition places a very small load on the host system, allowing simulation to proceed at an almost normal rate.  Before trace execution begins, and after the trigger condition has occurred and trace memory has filled, tracing places no load on the simulator, so that it operates at full speed.

### Trace Commands

The trace facility is modeled after the HP 64620A Logic State Analyzer for the HP 64000 Logic Development System.  Commands are provided to configure the memory, trigger, storage, window, and output list specifications.  The following commands are available:

| Command | Action |
| --- | --- |
| TRACE MEMORY | Allocate a trace memory buffer of a specified size |
| TRACE TRIGGER | Specify the trigger condition(s) that stop trace acquisition |
| TRACE STORE | Specify the device states to store to trace memory |
| TRACE WINDOW | Specify the device condition(s) that enable states to be stored |
| TRACE EXECUTE | Begin state acquisition and the search for the trigger conditions(s) |
| TRACE HALT | End trace acquisition immediately without waiting for the trigger condition |
| TRACE LIST | Specify the trace list output file and/or write the listing to the file |
| SHOW TRACE | Display the current trace configuration |

Each of these commands is described in detail below.

#### TRACE MEMORY

Configuring the memory specification with the TRACE MEMORY command establishes the size of the memory buffer used to store acquired states.  The syntax of the command is:

TRACE MEMORY=<count>[<multiplier>]

...where count is the number of device states to save, and the optional multiplier is K or M to interpret the specified count as multiples of one thousand or one million.  Each state requires approximately 50 bytes of memory.  The TRACE MEMORY command discards any previous trace memory buffer, including any acquired states, and allocates a new, empty buffer of the specified size.  If the specified count is zero, any existing buffer is discarded but no new buffer is allocated.  This action may be desirable to reduce the simulator's memory requirement for continued operation after tracing to a large buffer has been completed and listed.

Memory size is limited to the memory available to the simulator program.  The default memory size is zero.

#### TRACE TRIGGER

Configuring the trigger specification with the TRACE TRIGGER command establishes one or more conditions that must be satisfied to enable termination of state acquisition.  When trace execution begins, states are acquired continuously until the specified trigger condition is met.  Once triggered, state acquisition continues until the trace memory is full or a manual halt is requested.  The syntax of the command is:

TRACE TRIGGER [MODIFY] <dev> {NOTHING | <src>=<value>[;<src>...]}[ OR <dev>...]
TRACE TRIGGER NOTHING
TRACE TRIGGER ANYTHING
TRACE TRIGGER POSITION={START | CENTER | END}

For the first command form, dev is the name of a simulated device, src is the name of the device-specific trigger source that controls acquisition, and value is the source value that satisfies the trigger condition.  Multiple trigger conditions may be specified for a given device or for multiple devices.  Triggering occurs if any of the conditions are satisfied.

Some trigger sources may have optional qualifying sources that restrict the triggering conditions.  For example, triggering may be specified on any disc read command, on a read command issued to a specific bus device, or a read command to a specific bus device for a specific block number.  For these types of triggers, the condition and all of the specified qualifiers must be satisfied.

Each new trigger specification replaces any earlier specification, unless the MODIFY option is included.  With the MODIFY option, the specified trigger condition(s) either add to, delete from, or modify any existing conditions.  Specifying NOTHING for the device trigger condition will remove all trigger conditions for that device.

For the second command form, triggering never occurs.  In this case, states are acquired until tracing is manually halted.  This mode may be useful if the desired states precede a simulation stop, such as an MPE System Failure halt.

For the third command form, triggering occurs immediately after trace execution begins.  In this case, the trace memory will be filled with the states that follow execution, and acquisition ceases when trace memory is full.  This mode may be useful if the desired states immediately follow a command, such as an MPE cold load.

The fourth command form specifies the location of the triggering state in the trace memory.  Desired placement of the trigger state depends on whether the device states of interest follow, surround, or precede the trigger state.

Specifying START places the trigger state at the start of the trace list, and acquisition stops when the remainder of the trace memory is filled.  Specifying END places the trigger state at the end of the trace list, and acquisition stops immediately.  Specifying CENTER places the trigger state in the middle of the trace list, and acquisition stops after the end of the list is reached.  Trigger position is automatically set to START when triggering on ANYTHING and to END when triggering on NOTHING.

In the resulting trace list, states preceding the trigger are marked with – –, while states that follow the trigger are marked with ++.  The trigger itself state is marked with >>.  Note that the trigger state always appears in the list, even if the state is not part of the storage specification.

The trigger specification may be changed only when tracing is inactive.  If trace acquisition is active, Command not allowed is displayed.

The default trigger condition is ANYTHING, and the default trigger position is START.

#### TRACE STORE

Configuring the storage specification with the TRACE STORE command determines the states for a given device that will be stored in the trace memory during acquisition.  The syntax of the command is:

TRACE STORE [MODIFY] <dev> {NOTHING | ANYTHING | [<sta>[;<sta>...]]}[ OR <dev>...]
TRACE STORE NOTHING

For the first command form, dev is the name of a simulated device and sta is the device-specific state that is to be stored in trace memory.  Multiple states may be specified for a given device or for multiple devices, and all states specified will be stored during acquisition as they occur.

Certain device states are commonly specified for tracing.  If an explicit list of states is not given for a device, an implicit set of states is used.  Consult the individual device descriptions for the lists of traceable states, including those that are assigned to the implicit set.

Each new store specification replaces any earlier specification, unless the MODIFY option is included.  With the MODIFY option, the specified states either add to, delete from, or modify any existing storage set.  Specifying NOTHING for the state will remove all states specified for that device.  Specifying ANYTHING for the state will add the full set of states available for that device.

For the second command form, no states for any device will be stored.  This inhibits trace execution, as at least one state must be stored before tracing may be initiated.

The storage specification may be changed only when tracing is inactive.  If trace acquisition is active, Command not allowed is displayed.

The default storage condition is NOTHING.

#### TRACE WINDOW

Configuring the window specification with the TRACE WINDOW command restricts the storage of states to those that match certain criteria.  During trace acquisition, a configured device window is enabled (“open”) only when the matching criteria is satisfied.  Unspecified device windows are enabled always, thus permitting all configured states to be stored.  The syntax of the command is:

TRACE WINDOW [MODIFY] <dev> {ENABLE | <src>=<value>[;<src>...]}[ OR <dev>...]
TRACE WINDOW ENABLE

For the first command form, dev is the name of a simulated device, src is the name of the device-specific window source that controls acquisition of the device states, and value is the source value that satisfies the enabling condition.  Multiple window conditions may be specified for a given device or for multiple devices.  States specified in a TRACE STORE command for a given device are placed in trace memory only when the associated window is enabled.

Some window sources may have optional qualifying sources that restrict the enabling conditions.  For example, window enabling may be specified on execution of a specific machine instruction or on all instructions that fall within a specified range.  For these types of windows, the condition and all of the specified qualifiers must be satisfied.

Each new window specification replaces any earlier specification, unless the MODIFY option is included.  With the MODIFY option, the specified window condition(s) either add to, delete from, or modify any existing conditions.  Specifying ENABLE for the condition will remove all window conditions for that device and enable the window unconditionally.

For the second command form, all device windows are enabled, thus removing any restrictions on state storage.

The window specification may be changed only when tracing is inactive.  If trace acquisition is active, Command not allowed is displayed.

The default window condition is ENABLE.

#### TRACE EXECUTE

The TRACE EXECUTE command initiates state acquisition.  If the current trigger condition is ANYTHING, Trace in process is displayed to indicate that the trigger condition has been satisfied.  Otherwise, Waiting for trigger is displayed to indicate that the trigger condition is being sought, and Trace in process is displayed when the trigger condition is satisfied.  When trace memory is full, execution stops, the current time is saved as the acquisition time for the trace list, and Trace complete is displayed.

Attempting execution without defining a trace memory buffer, while state acquisition is currently active, or without defining at least one storage state will result in a Command not allowed error.

#### TRACE HALT

The TRACE HALT command ends the current trace operation.  If trace acquisition is currently active, tracing is stopped, the current time is saved as the acquisition time for the trace list, and Trace halted is displayed.  If the trigger condition had not been satisfied, the last state captured is marked as the trigger state; otherwise the state that satisfied the trigger condition is marked.

If tracing is inactive, the TRACE HALT command is ignored.  Attempting to halt a trace without defining a trace memory buffer will result in a Command not allowed error.

#### TRACE LIST

Configuring the list specification with the TRACE LIST command generates the formatted list of traced states and/or establishes the text file that holds the trace list.  The syntax of the command is:

TRACE [-N] LIST=<filename>
TRACE [-N] LIST
TRACE LIST NOTHING

The first command form specifies the list output filename, which will be used for all subsequent trace lists until it is closed, or another list file is specified  If the trace memory buffer has not been defined, the command completes.  If tracing is currently executing, it is halted.  Then if trace memory is defined but contains no trace states, Trace memory is empty is displayed, and the command completes.  If trace memory is occupied, then the list of states is decoded, formatted, and written to the list file, prefaced with the date and time of state acquisition.  If the –N (new file) switch is specified, the list file is cleared before writing the new trace list.  Otherwise, the trace list is appended to the file.

The second command form performs the same actions as the first, except that the previously defined list file is used.  If a list file has not been defined, then the Command not allowed error is displayed.

The third command form closes any existing list file without specifying a new file.

If trace memory contains acquired trace states that have not been listed, attempting to exit the simulator will display Exiting will lose trace memory contents; OK to exit [N]?  Answering NO or pressing ENTER will return to the SCP prompt to allow a TRACE LIST command to be issued.  Answering YES will exit without saving the list.

The default list configuration is NOTHING.

#### SHOW TRACE

The SHOW TRACE command displays the current trace configuration.  As an example, these commands:

sim> TRACE MEMORY=100000
sim> TRACE TRIGGER CPU INSTRUCTION=FDIV,ADD OR MCL ADDRESS=1230;STATUS=WRITE
sim> TRACE TRIGGER POSITION=CENTER
sim> TRACE STORE CPU OR ADCC CMD;CSIRQ;XFER
sim> TRACE WINDOW ADCC PORT=2;3;4;5
sim> TRACE –N LIST=trace.log
sim> SHOW TRACE

...produce this display:

Trace Specification

   MEMORY
      is 100K states

   TRIGGER
      on CPU instruction is FDIV,ADD
        or MCL address is 00.001230 and status is Write
      position is center of trace

   STORE
      on CPU state is INSTR;DATA;REG;OPND;IRQ
        or ADCC state is CMD;CSIRQ;XFER

   WINDOW
      enabled on ADCC port is 2-5

   LIST
      on trace.log
      is empty

### Obsolete Commands

SCP provides two commands to configure its debug facility.  These were used by earlier versions of the simulator but have now been replaced by the trace commands described above.

The SET CONSOLE DEBUG=<filename> command was used to establish a debug log file.  The command still exists in SCP, but the simulator does not write to this file.  It has been replaced with the TRACE LIST=<filename> command.

The SET <device> DEBUG=<option> commands were used to enable the desired debug states.  These SCP commands now fail with Command not allowed errors.  They have been replaced by the TRACE STORE command.  The new command is equivalent but more convenient, as it allows multiple devices to be configured with a single command.

# Processor Device Simulations

The HP 3000 Series III computer consists of the following subsystems:

- 30003B Central Processing Unit

- 30003B I/O Processor

- 30007A Memory Control and Logging interface

- 30030C Selector Channel

- 30036B Multiplexer Channel

The HP 3000 Series 58 computer consists of the following subsystems:

- 32558A Central Processing Unit

- Channel Program Processor

- 30172A Memory Control and Logging interface

## Central Processing Unit

The HP 3000 Central Processing Unit contains the microprogrammed machine instruction execution unit, main memory, and process clock.  CPU options specify the memory size, installed firmware, and simulation configuration.  The CPU is configured with commands of the form:

SET [-F] CPU <option>

Device options that may be specified are:

| Option | Action |
| --- | --- |
| III | Configure the CPU as a Series III; default |
| S58 | Configure the CPU as a Series 58 |
| 128K | Set the memory size to 128K words |
| 256K | Set the memory size to 256K words |
| 384K | Set the memory size to 384K words |
| 512K | Set the memory size to 512K words |
| 768K | Set the memory size to 768K words |
| 1024K | Set the memory size to 1024K words; Series III default |
| 2048K | Set the memory size to 2048K words; Series 58 default |
| 4096K | Set the memory size to 4096K words |
| EIS | Enable the Extended Instruction Set firmware; default |
| NOEIS | Disable the Extended Instruction Set firmware |
| CIS | Enable the COBOL II Extended Instruction Set firmware; Series 58 default |
| NOCIS | Disable the COBOL II Extended Instruction Set firmware; Series III default |
| CSTX | Enable the MPE V/E CST Expansion firmware; Series 58 default |
| NOCSTX | Disable the MPE V/E CST Expansion firmware |
| ARS | Enable auto-restart after a power failure; default |
| NOARS | Disable auto-restart after a power failure |
| CALTIME | Use calibrated process clock timing; default |
| REALTIME | Use realistic process clock timing |
| IDLE | Enable idle detection |
| NOIDLE | Disable idle detection; default |
| START=<chan>[;<dev>] | Set the channel and device numbers of the start device; default is 11, 0 |
| LOAD=<chan>[;<dev>] | Set the channel and device numbers of the load device; default is 9, 0 |
| DUMP=<chan>[;<dev>] | Set the channel and device numbers of the dump device; default is 11, 0 |
| DUMPDEV=<n> | Set the dump device number; default is 6 |
| DUMPCTL=<n> | Set the dump device control value; default is 004 |
| STOP=<option> | Enable simulation stops |
| NOSTOP | Disable simulation stops; default |

Selecting the CPU model enables the set of I/O devices that are compatible with the model and disables all incompatible devices.  For example, the ATC device is enabled when the CPU is a Series III and disabled when the CPU is a Series 58.  The current CPU model also controls the applicability of certain configuration commands, as noted below.

The initial configuration is a Series III system with 1024K words of memory.  Available memory sizes are specific to the CPU model selected.  If the memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation before proceeding.  The confirmation request may be suppressed by using the –F (force) switch.  Data in the truncated portion of memory is lost.

The SET CPU EIS command simulates the installation of the HP 30012A Extended Instruction Set firmware on the Series III ROM PCA and removal of the W1 and W8 jumpers from the CIR PCA to enable decoding of the instructions.  The set includes four-word extended-precision floating-point instructions and decimal arithmetic instructions.  The EIS is standard on HP 3000 Series II and III machines, so this option is enabled by default.  If the set is disabled with the SET CPU NOEIS command, MPE will emulate the instructions in software.

The SET CPU CIS command simulates the installation of the HP 32234A COBOL II Extended Instruction Set firmware on the Series III ROM PCA and removal of the W4 jumper from the CIR PCA to enable decoding of the instructions.  The HP 32233A COBOLII/3000 compiler and programs produced by this compiler require the firmware to execute.  If it is not installed, these programs will abort at run time with ILLEGAL INSTRUCTION errors.

The EIS and CIS firmware is standard on the Series 58.  Commands that attempt to change these firmware settings on this model are rejected with Command not allowed errors.

The SET CPU CSTX command simulates the installation of the MPE V/E Code Segment Table Expansion firmware.  This permits the MPE V/E operating system to be configured with larger code segment and code segment extension tables.  However, earlier operating systems, including MPE IV and MPE V/P, will not run with the expansion firmware enabled.  For these systems, the SET CPU NOCSTX command can be used to disable the firmware.  CST Expansion firmware is standard on the Series 58.  It is not available for the Series III.

The position of the PF/ARS switch on the back of the Series III system control panel determines how the CPU responds to AC power returning after a power failure.  In the enable position, simulated by the SET CPU ARS command, power restoration will initiate an automatic restart of the operating system.  In the disable position, simulated by the SET CPU NOARS command, power restoration will set up the system for a restart, which must be performed manually by entering a CONTINUE, STEP, GO, or RUN command.

Automatic power fail recovery is standard on the Series 58, so entering the SET CPU ARS and SET CPU NOARS commands will be rejected with Command not allowed errors.

Calibrated timing adjusts the process clock to match actual elapsed time.  Realistic timing bases the process-clock interval on machine instructions executed.  For the Series 58, the process clock and the system clock that maintains the time of day for MPE are equivalent.

When enabled by a SET CPU IDLE command, execution of a PAUS instruction will idle the simulator.  While idle, the simulator does not use any host system processor time.  Idle detection requires that the process clock and system clock be set to calibrated timing.  Idling is disabled by default.

The START, LOAD, and DUMP options configure the default channel and device numbers on the corresponding thumbwheel switches of the Series 58 system control panel.  The START and LOAD settings reference the system disc and tape devices, respectively, that contain an operating system, and the DUMP setting references a disc or tape device that contains the Software Dump Facility program.  For each setting, if the device number is omitted, it defaults to the device at bus address 0.

The DUMPDEV and DUMPCTL options configure the set of jumpers on the rear of the Series III front panel that preset the device number and control value for the cold dump process.  These values are used when the DUMP command is entered without the optional control/device number.  The device number range is 0–127 decimal, and the control value range is 0–377 octal.  The default control value corresponds to the Write Record tape command.

Entering a START, LOAD, or DUMP option while the CPU is configured as a Series III or entering a DUMPDEV or DUMPCTL option while the CPU is configured as a Series 58 will be rejected.

The SET CPU STOP command enables one or more simulation stop conditions.  These are described in the Simulation Stops section below.

The CPU configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW CPU | Display the device configuration |
| SHOW CPU STOPS | Display the enabled simulation stops |
| SHOW CPU DUMP | Display the cold dump device number and control value (Series III only) |
| SHOW CPU PANEL | Display the system control panel settings (Series 58 only) |
| SHOW CPU SPEED | Display the current simulation speed |

When the process clock is calibrated, the current simulation speed, expressed as a multiple of the speed of a real HP 3000, may be obtained with the SHOW CPU SPEED command.  The multiple is referenced to the configured CPU model.  The speed reported will not be representative if the simulator was executing a PAUS instruction when it was stopped.

### System Halt

The MPE operating system is supported by special microcode features that perform code and data segment mapping, segment access bounds checking, privilege checking, etc.  The layout of certain in-memory tables is known to both the OS and the microcode and is used to validate execution of instructions.  For instance, every stack instruction is checked for a valid access within the stack segment boundaries, which are set up by the OS before program dispatch.  For this reason, the 3000 cannot be operated as a “bare” machine, because these tables will not have been initialized.  Similarly, the cold load process by which the OS is loaded from storage media into memory is entirely in microcode, as machine instructions cannot be executed until the required tables are loaded into memory.

This OS/microcode integration means that the microcode may detect conditions that make continued execution impossible.  An example would be an absent segment fault for the segment containing the disc I/O driver.  If such a condition is detected, the CPU does a system halt.  This fatal microcode error, distinct from a regular programmed halt, causes operation to cease until the CPU is reset.

A system halt stops the simulator and reports the system halt code to the simulation console.  The code indicates the reason for the system halt, as follows:

| Code | Reason for System Halt |
| --- | --- |
| 1 | STT violation while executing in segment 1 |
| 2 | Absent code segment while executing on the ICS |
| 3 | Code segment 1 is absent |
| 4 | Stack overflow while executing on the ICS |
| 5 | CST length violation |
| 6 | I/O device timeout while executing IXIT or cold load |
| 7 | Cold load channel program checksum error |
| 8 | Cold load channel program abort |
| 9 | Dispatcher already enabled while executing PSEB |
| 12 | Cold load channel is not the system controller |
| 13 | CST violation while executing in segment 1 |
| 17 | Cold load channel cannot become the controller |
| 23 | External interrupts disabled while executing LOCK |
| 33 | Attempt to trace segment 1 |

If a system halt has occurred, simulator execution may not be resumed until a RESET, RESET CPU, or one of the cold-load commands has been entered.

### Idling

The PAUS instruction suspends instruction execution until an interrupt occurs.  It is intended to defer instruction fetches from memory to allow full-bandwidth access by the I/O Channels.  If enabled, the simulator will idle only during execution of PAUS.

### Simulation Stops

The simulator can be configured to detect certain machine instruction conditions and stop execution when one or more of them occur.  Four stop options control the simulation stop conditions:

| Option | Stop Condition |
| --- | --- |
| LOOP | Stop when an infinite loop is executed |
| PAUSE | Stop when a PAUS instruction is executed |
| UNDEF | Stop when an undefined instruction is executed |
| UNIMPL | Stop when an unimplemented instruction is executed |
| COLDLOAD | Stop when a cold load completion trap occurs |

Setting the LOOP option stops the simulator if it attempts to execute an instruction that enters an infinite loop (e.g., BR P+0).  The branch instructions TBA, TBX, BCC, BR, BCY, BNCY, BOV, and BNOV result in an infinite loop if the branch displacement is zero and the branch condition is true.  The remaining branch instructions cannot result in an infinite loop, as they all modify the CPU state and so eventually will reach a point where they drop out of their loops.

Setting the PAUSE option stops the simulator if execution of a PAUS instruction is attempted.

Setting the UNDEF option stops the simulator if execution of a non-canonical instruction (i.e., an instruction containing a decoded reserved bit pattern other than that defined in the Machine Instruction Set manual) is attempted.  Setting the UNIMPL option stops the simulator if execution of an unimplemented instruction is attempted.

When the simulator examines the bit patterns of instructions to execute, each will fall into one of four categories:

- Defined (canonical) instruction encodings, where all bits are defined or all reserved bits are zero (e.g., LOAD).

- Undefined (non-canonical) instruction encodings, where reserved fields are “don't care” bits (e.g., MOVE).

- Undefined (non-canonical) instruction encodings, where reserved fields are decoded (e.g., IXIT).

- Unimplemented instruction encodings (e.g., stack opcode 072, or EADD without the EIS firmware option installed).

Instructions in categories 1 and 2 are always executed.  The UNDEF option stops the simulator for instructions in category 3.  The intent is to catch instructions containing reserved fields with values that may change the meaning of those instructions.  The UNIMPL option stops the simulator for instructions in category 4.

Undefined instruction mnemonics are printed in lowercase when displaying memory or tracing execution.  For example, tracing execution of the canonical SED 1 instruction encoding (030041 octal) prints SED 1 in the trace.  Tracing a non-canonical encoding (030043 octal) prints sed 1.

Setting the COLDLOAD option stops the simulator when the cold load trap is taken at the end of the loading process.  The option is only needed with IMB systems, as the IOP systems automatically stop when the cold load is complete.  In a simulator command file, stopping after cold loading allows additional system configuration to be performed before execution begins.

After a simulation stop, execution may be resumed in one of two ways.  If the cause of the stop has not been remedied and the stop option has not been disabled, resuming execution with CONTINUE, STEP, GO, or RUN will cause the stop to occur again.  Alternately, specifying the –B switch with any of the preceding commands will resume execution while bypassing the stop for the current instruction.

Bypassing a stop has the following effect:

| Option | Bypass Action |
| --- | --- |
| LOOP | Resume execution of the infinite loop |
| PAUSE | Resume execution with the instruction following PAUS |
| UNDEF | Resume execution with decoding as in the hardware |
| UNIMPL | Resume execution with an Unimplemented Instruction trap |

Explicitly bypassing the COLDLOAD stop is not needed.  When the stop occurs, the cold load trap has already been taken, and execution is stopped at the entry to the cold load handler.  Resuming execution, with or without the –B switch, proceeds to execute the handler.

Note that the PAUSE bypass action corresponds in hardware to pressing the HALT button and then the RUN button.  If the stop conditions are disabled, the simulator performs the actions above, except that PAUS suspends instruction execution until a device interrupt occurs.

### Tracing

The CPU device supports configuration of the trigger, store, and window specifications.  The trigger sources for the TRACE TRIGGER CPU command are:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| INSTRUCTION | opcode | IGNORE | mask | Machine instruction executions |
| TRAP | trap | PARAMETER | value | Internal trap executions |
| INTERRUPT | devno | — | — | External interrupt executions |

The INSTRUCTION source triggers when the machine instruction designated by the opcode executes.  The opcode may be either an instruction mnemonic and associated operand or a numeric value.

If the IGNORE qualifier is added, the mask value is a set of "don't care" bits that is applied to the opcode before comparison.  Qualification is performed by ANDing the current instruction with the complement of the specified mask value and then comparing the result with the specified opcode value.

Numeric values entered for the opcode and mask parameters may be one or two words, corresponding to the one- or two-word CPU instructions.  Two-word values are entered as a 16-bit first word and a 16-bit second word, separated by a period (e.g., 020302.000006).  The values are entered in the CPU’s data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Adding the IGNORE qualifier allows triggering on a range of instructions or an instruction with a range of operand values.  As examples, specifying INSTRUCTION=020460;IGNORE=000017 will trigger on execution of any of the COBOL-II firmware instructions, and specifying INSTRUCTION=PCAL 0;IGNORE=000377 will trigger on execution of any PCAL instruction.

Stack instructions may be configured to trigger on execution of the left opcode, the right opcode, or either opcode.  For example, specifying INSTRUCTION=ADD,NOP;IGNORE=000077 will trigger only when ADD is executed as the left operand, while INSTRUCTION=NOP,ADD;IGNORE=007700 will trigger only when ADD is executed as the right operand.  To trigger on execution in either position, specify INSTRUCTION=ADD without a qualifier.

The TRAP source triggers when the internal trap specified by the trap value occurs.  Adding the PARAMETER qualifier triggers only when the trap occurs with the specified parameter value.  The valid trap values are:

| Trap | Trap Description | Parameter Value Description |
| --- | --- | --- |
| 1 | Bounds Violation | Trap handler program label |
| 20 | Unimplemented Instruction | Trap handler program label |
| 21 | STT Violation | Segment number |
| 22 | CST Violation | Segment number |
| 23 | DST Violation | Trap handler program label |
| 24 | Stack Underflow | Trap handler program label |
| 25 | Privileged Mode Violation | Trap handler program label |
| 30 | Stack Overflow | Trap handler program label |
| 31 | User Trap | Condition identifier |
| 37 | Absent Code Segment | Program label/N/0 |
| 40 | Trace | Program label/N/0 |
| 41 | STT Entry Uncallable | Called program label |
| 42 | Absent Data Segment | Data Segment Table index |
| 43 | Power On | Trap handler program label |
| 44 | Cold Load | Trap handler program label |
| 45 | System Halt | System halt code |

For example, specifying TRAP=1 will trigger when a bounds violation occurs.

The PARAMETER qualifier may be useful in restricting trap trigger conditions to specific instances.  For example, the parameter value for the Absent Code Segment and Trace traps depends on when the trap occurs.  It is the program label (if on procedure entry), the number N of procedure parameters to delete (if on procedure exit), or zero (if on interrupt exit).

User Traps will not occur unless the User Traps bit (T bit) is set in the Status register.  If enabled, the parameter will indicate the specific type of user trap taken, as follows:

| Value | Description |
| --- | --- |
| 1 | Integer Overflow |
| 2 | Float Overflow |
| 3 | Float Underflow |
| 4 | Integer Divide by Zero |
| 5 | Float Divide by Zero |
| 10 | Extended Float Overflow |
| 11 | Extended Float Underflow |
| 12 | Extended Float Divide by Zero |
| 13 | Decimal Overflow |
| 14 | Invalid ASCII Digit |
| 15 | Invalid Decimal Digit |
| 16 | Invalid Word Count |
| 17 | Invalid Decimal Length |
| 20 | Decimal Divide by Zero |

For example, specifying TRAP=31;PARAMETER=4 will trigger only when an Integer Divide by Zero trap occurs.  Specifying TRAP=31 without a qualifier will trigger when any user trap occurs.

The INTERRUPT qualifier triggers when the external interrupt occurs on the specified device number devno.

Trap, parameter, and interrupting device number values are entered in the CPU’s data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

The storage states that may be configured for the TRACE STORE CPU command are:

| State | Implicit | State Description |
| --- | --- | --- |
| INSTR |  | Machine instruction executions |
| DATA |  | Memory data accesses |
| FETCH |  | Memory instruction fetches |
| REG |  | Register values |
| OPND |  | Memory operand values |
| IRQ |  | Interrupt request and trap executions |
| PSERV |  | Process clock service events (periodic) |

The INSTR state traces instruction executions.  Each instruction is printed before it is executed.  The two opcodes of a stack instruction are printed together before the left-hand opcode is executed.  If the right-hand opcode is not NOP, it is reprinted before execution, with dashes replacing the just-executed left-hand opcode.

The DATA state traces reads from and writes to memory.  Each access is classified by the memory bank register that is paired with the specified offset — DMA, absolute, program, data, or stack.  DMA (channel) accesses derive their bank addresses from the banks specified by Set Bank I/O program orders.  Absolute accesses always use bank 0.  Program, data, and stack accesses use the bank addresses in the PBANK, DBANK, and SBANK registers, respectively.

The FETCH state traces instruction fetches from memory.  These accesses are separated from those traced by the DATA option because fetches usually are of little interest except when debugging the fetch/execute sequence.  Because the HP 3000 has a two-stage pipeline, fetches load the NIR (Next Instruction Register) with the instruction after the instruction about to be executed from the CIR (Current Instruction Register).

The REG state traces register values.  Two sets of registers may be printed.  After executing each instruction, the currently active TOS registers, the index register, and the status register are printed.  After executing an instruction that may alter the base registers, the program, data, and stack segment base registers are printed.

The OPND state traces memory byte operand values.  Some instructions take memory and register operands that are difficult to decode from DATA or REG traces.  This option presents these operands in a higher-level format.  The memory bank and address values are always those of the operands.  The operand data and values printed are specific to the instruction.  For example, the ALGN instruction prints its source and target operands, digit counts, and fraction counts, and the EDIT instruction displays its subprogram operations.

The IRQ state traces interrupt and trap requests, including dispatcher interrupts and interrupt returns.  These items are also traced with the INSTR option, but the IRQ option is useful if full instruction tracing is unnecessary, for example when tracing I/O device operation.

The PSERV state traces process clock event service entries.  Each trace reports whether the CPU was executing on the Interrupt Control Stack or the user stack when the process clock ticked.  Execution on the ICS implies that the operating system is executing.  As the process clock ticks every millisecond, enabling PSERV tracing can quickly produce a large number of trace lines.

If one or more of the CPU trace states are in effect at the time that a simulation stop occurs, an instruction trace showing the stop reason will be included.

Omitting the list of storage states in a TRACE STORE CPU command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW CPU command is:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| INSTRUCTION | opcode | IGNORE | mask | Machine instructions executed |

The INSTRUCTION source restricts the storage of all states to those associated with the machine instruction designated by the opcode, which may be either an instruction mnemonic and associated operand or a numeric value.  The storage window is enabled when the specified opcode is executed and is disabled when a different instruction is executed.  This source allows detailed tracing of specified instructions while minimizing the trace list size compared to a full instruction trace.  Adding the IGNORE qualifier allows matching a range of instructions or an instruction with a range of operand values.

The syntax of the opcode and mask values and their interactions are the same as those described for the trigger command at the beginning of this Tracing section above.

A section of an example trace list is:

**CPU fetch: 00.010342  020320    instruction fetch
**CPU instr: 00.010341  000300  ZROX,NOP
**CPU   reg: 00.006500  000000    X 000000, M i t r o c CCG
**CPU fetch: 00.010343  041100    instruction fetch
**CPU instr: 00.010342  020320  PLDA
**CPU  data: 00.000000  001340    absolute read
**CPU   reg: 00.006500  000001    A 001340, X 000000, M i t r o c CCG
**CPU fetch: 00.010344  037777    instruction fetch
**CPU instr: 00.010343  041100  LOAD DB+100
**CPU  data: 00.002100  123003    data read
**CPU   reg: 00.006500  000002    A 123003, B 001340, X 000000, M i t r o c CCL
**CPU fetch: 00.010345  023404    instruction fetch
**CPU instr: 00.010344  037777  ANDI 377
**CPU   reg: 00.006500  000002    A 000003, B 001340, X 000000, M i t r o c CCG
**CPU fetch: 00.010346  002043    instruction fetch
**CPU instr: 00.010345  023404  MPYI 4
**CPU   reg: 00.006500  000002    A 000014, B 001340, X 000000, M i t r o c CCG
**CPU fetch: 00.010347  020320    instruction fetch

The trace formats are interpreted as follows:

**CPU instr: 00.010341  000300  ZROX,NOP

Instruction mnemonic(s)

Octal data (instruction opcode)

Octal address (P)

Octal bank (PBANK)

**CPU instr: 00.001240  000006  external interrupt
**CPU instr: 00.023736  000000  unimplemented instruction trap

Interrupt classification

Parameter

Octal address (P) at interrupt

Octal bank (PBANK) at interrupt

**CPU instr: 00.056235  101005  simulation stop: Unimplemented instruction

Stop reason

Octal status register value (STA)

Octal address (P)

Octal bank (PBANK)

**CPU  data: 00.002100  123003    data read
**CPU  data: 00.000000  001340    absolute read

Memory access classification

Octal data (memory contents)

Octal address (effective address)

Octal bank (PBANK, DBANK, or SBANK)

**CPU fetch: 00.010342  020320    instruction fetch

Memory access classification

Octal data (instruction opcode)

Octal address (P + 1)

Octal bank (PBANK)

**CPU   reg: 00.006500  000002    A 123003, B 001340, X 000000, M i t r o c CCL

Register values (0-4 TOS registers, X, STA)

Octal stack register count (SR)

Octal stack memory address (SM)

Octal bank (SBANK)

**CPU   reg: 00.000000  000001    PB 010000, PL 025227, DL 001770,
                                  DB 002000, Q  006510, Z  007000

Base register values

Current code segment number (from STA)

Zero

Octal bank (DBANK)

**CPU  opnd: 00.135771  000000    DFLC '+','!'
**CPU  opnd: 00.045071  000252    target fraction 3 length 6,"002222"

Instruction-specific operand value

Instruction-specific data value

Octal address (effective address)

Octal bank (PBANK, DBANK, or SBANK)

**CPU pserv: Process clock delay 3970 service entered on the ICS

Service entry and stack status

For OPND traces of byte-array operands, the data values printed are the relative byte addresses.  For EDIT subprogram operations, the value of the loop counter is printed.

Enabling CPU tracing can produce a very large number of trace lines very quickly, so care should be used to enable tracing only around the area of interest.  Careful configuration of the trigger, storage, and window specifications will ensure that the states of interest are captured in the list.

### Registers

The CPU state contains the registers visible to the programmer and the interrupt system control registers:

| Name | Size | Radix | Symbolic | Read-Only | Description |
| --- | --- | --- | --- | --- | --- |
| CIR | 16 | – |  |  | Current Instruction Register |
| NIR | 16 | – |  |  | Next Instruction Register |
| PB | 16 | 8 |  |  | Program Base Register |
| P | 16 | 8 |  |  | Program Counter |
| PL | 16 | 8 |  |  | Program Limit Register |
| PBANK | 4 | 8 |  |  | Program Segment Bank Register |
| DL | 16 | 8 |  |  | Data Limit Register |
| DB | 16 | 8 |  |  | Data Base Register |
| DBANK | 4 | 8 |  |  | Data Segment Bank Register |
| Q | 16 | 8 |  |  | Stack Marker Register |
| SM | 16 | 8 |  |  | Stack Memory Register |
| SR | 3 | 8 |  |  | Stack Register Counter |
| Z | 16 | 8 |  |  | Stack Limit Register |
| SBANK | 4 | 8 |  |  | Stack Segment Bank Register |
| RA | 16 | 8 |  |  | Top of Stack Register |
| RB | 16 | 8 |  |  | Top of Stack – 1 Register |
| RC | 16 | 8 |  |  | Top of Stack – 2 Register |
| RD | 16 | 8 |  |  | Top of Stack – 3 Register |
| X | 16 | 8 |  |  | Index Register |
| STA | 16 | – |  |  | Status Register |
| SWCH | 16 | 8 |  |  | Switch Register |
| CPX1 | 16 | 8 |  |  | Run-Mode Interrupts Register |
| CPX2 | 16 | 8 |  |  | Halt-Mode Interrupts Register |
| PCLK | 16 | 8 |  |  | Process Clock Register |
| CR | 16 | 8 |  |  | Timer Count Register |
| SIR | 16 | 8 |  |  | Status and Interrupt Register |

The CIR and NIR registers default to CPU instruction mnemonic format, and the STA register defaults to CPU status mnemonic format for display and entry but may be overridden with a numeric-format switch, if desired.  The RA, RB, RC, RD, X, and SWCH registers may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.  The CR and SIR registers are present in the Series 58 and are significant only when that CPU model is selected.

Three additional, hidden, read-only registers may be displayed if mentioned explicitly.  CNTR represents the hardware counter used by the microcode.  When a PAUS or HALT instruction is executed, the microcode stores the value of the SR register in the CNTR register before flushing the TOS registers to memory (so SR is always zero after executing either instruction).  The MOD register holds the Series III module number during an unsolicited module interrupt.  SP0 is a scratch-pad register that contains the numeric halt code when a system halt occurs.  CNTR and MOD are provided for the CPU diagnostic and have no other use under simulation.  SP0 is provided for the cold dump process to record the reason for a system halt.

## Memory Control and Logging interface

The 30007A Memory Control and Logging interface for the Series III and the 30172A Memory Control and Logging interface for the Series 58 control access to the main memory for the CPU.

Replacing the core memory used in the earlier 3000 CX machines, the Series II introduced an all-semiconductor memory using 4K NMOS RAMs that provided error detection and correction.  Single-bit errors are corrected automatically, and double-bit errors are detected.  All errors are logged in hardware, and the logs are downloaded periodically by MPE to allow preventative maintenance and replacement of failing parts.

The Series III supports a main memory size of 128K to 1024K words in 128K increments using 16K RAMs.  It uses three types of memory PCAs:

- 30007-60005 MCL (Memory Control and Logging, up to 512K words)

- 30008-60003 SMA (Semiconductor Memory Array, 128K words, 22 bits)

- 30009-60002 FLI (Fault Logging Interface, up to 1024K words)

A 128K system uses one of each PCA.  A 1024K system uses 2 MCLs, 8 SMAs, and 1 FLI.  Six check bits (all on the SMA) are used.  The standalone FLI PCA may be replaced with a 30135-60063 System Clock/Fault Logging Interface that combines both devices on a single PCA.

The Series 58 supports a main memory size of 2048K to 4096K words in 512K increments using 256K RAMs.  It uses four types of memory PCAs:

- 30172-60001 MCL (Memory Control and Logging, up to 4096K words)

- 30161-60001 SMA (Semiconductor Memory Array, 512K words, 39 bits)

- 30478-60001 SMA (Semiconductor Memory Array, 1024K words, 39 bits)

- 30479-60001 SMA (Semiconductor Memory Array, 2048K words, 39 bits)

Main memory uses 39-bit words, arranged into 32 data bits and 7 error check bits.  Communication with the CPU and channel controllers via DMA is over the Intermodule Bus.  The CPU or channel issues the Memory Read, Memory Write, Memory Read/Write Ones, and Memory Write Control/Read Status IMB commands, which are decoded and executed by the MCL.  The MCL is also responsible for determining which half of the 32-bit data word is accessed for a given address.  The IMB has 24 address lines and so supports a maximum of 16 megawords of memory.

The MCL device does not currently support fault logging.

### Tracing

The MCL device supports configuration of the trigger and store specifications.  The trigger source for the TRACE TRIGGER MCL command is:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| ADDRESS | address | DATA | data | Memory accesses |
|  |  | STATUS | status |  |

The ADDRESS source triggers when the memory word at the address is accessed.  The address is an absolute address consisting of a 4- or 6-bit bank number and a 16-bit offset within the bank, separated by a period (e.g., 17.177777).  If the bank number is omitted, bank 0 is assumed.

If the DATA qualifier is added, the accessed word must contain the data value specified.  If the STATUS qualifier is added, the access must match the status value, which is either READ or WRITE.  Triggering will occur only when the condition and all of the specified qualifiers are satisfied.

The address value is entered in the CPU's address radix, which is octal.  The data value is entered in the CPU’s data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

The storage states that may be configured for the TRACE STORE MCL command are:

| State | Implicit | State Description |
| --- | --- | --- |
| DATA |  | Memory data accesses |
| IMBUS |  | Intermodule Bus signals and data words read and written |

The DATA state traces reads from and writes to memory by the Series 58 memory controller.  The IMBUS state traces the Intermodule Bus signals and data exchanged with the Series 58 memory controller.  These are always DMA accesses on behalf of devices connected to the General I/O Channel.  These states do not occur if the CPU is configured as a Series III.

Omitting the list of storage states in a TRACE STORE MCL command uses the indicated implicit state.

The trace formats are interpreted as follows:

**MCL  data: 00.002100  123003    dma read
**MCL  data: 00.000000  001340    dma write

Memory access classification

Octal data (memory contents)

Octal address

Octal bank (DBANK)

**MCL imbus: Memory controller received opcode Write address 00015470 data 041407
**MCL imbus: Memory controller returned data 000000 with signals ADN | DDN | PRO

Bus opcode, address, data, and signals

## I/O Processor

The HP 30003B I/O Processor works in conjunction with the Series III CPU and Multiplexer Channel to service the device interfaces.  All I/O interfaces are connected to the IOP bus, which transfers direct I/O orders to the interfaces and handles memory reads and writes between the interfaces and the CPU stack.  In addition, it provides the memory interface for Multiplexer Channel transfers, as well as fetching I/O program orders from main memory for the channel.

The IOP device has no configuration options.  It does provide tracing, though, with selectable filtering by device number, using these device options:

| Option | Action |
| --- | --- |
| FILTER=<list> | Suppress tracing for device numbers in the list |
| NOFILTER | Enable tracing for all device numbers; default |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |

Enabling IOP tracing can produce a very large number of trace lines very quickly, so care should be used to enable tracing only around the area of interest.  If only certain devices are of interest, others may be filtered out of the results by specifying their device numbers in a SET IOP FILTER command.  This command takes a list of individual device numbers separated by semicolons and ranges of the form low-high.  For example, SET IOP FILTER=3;7-9 would exclude lines pertaining to device numbers 3, 7, 8, and 9 from the trace report.

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| DIO | Direct I/O orders issued |
| IRQ | Interrupt requests received |
| DATA | Multiplexer Channel memory data accesses |

The DIO option traces direct I/O orders that are sent to devices.  The IRQ option traces interrupt requests received and granted.  The DATA option traces memory accesses performed on behalf of the Multiplexer Channel.  Each access is classified as either an absolute access in memory bank 0 or a DMA access in the memory bank specified by a Set Bank I/O program order.

The trace formats are interpreted as follows:

**IOP   dio: Test I/O order sent to device number 3

I/O order and device number

**IOP   irq: Device number 6 acknowledged interrupt request at priority 14

Device number and interrupt priority number

**IOP  data: 00.001400  040000    dma write
**IOP  data: 00.000030  001434    absolute read

Memory access classification

Octal data (memory contents)

Octal address

Octal bank number

The I/O Processor state contains these registers:

| Name | Size | Radix | Read-Only | Description |
| --- | --- | --- | --- | --- |
| IOA | 8 | 8 |  | I/O Address Register |

## Selector Channel

The HP 30030C Selector Channel provides high-speed data transfer between a device and the Series III main memory.  While several interfaces may be connected to the selector channel bus, only one transfer is active at a time, and the channel remains dedicated to that interface until the transfer is complete.  The channel contains its own memory port controller, so transfers to and from memory bypass the I/O Processor.

Once started by an SIO instruction, the channel executes I/O programs independently of the CPU.  Program words are read, and device status is written back, directly via the port controller.

The SEL device has no configuration options.  It does provide tracing, though, using these device options:

| Option | Action |
| --- | --- |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Channel command initiations and completions |
| PIO | Programmed I/O orders executed |
| STATE | Channel state changes executed |
| SR | Service requests received |
| DATA | Channel memory data accesses |

The CSRW option traces the beginning and ending of channel programs.  The PIO option traces programmed I/O orders that are sent to devices.  The STATE option traces the entries into each of the internal channel execution states.  The SR option traces service requests received from the device.  The DATA option traces memory accesses performed by the port controller.  Each access is classified as either an absolute access in memory bank 0 or a DMA access in the memory bank specified by a Set Bank I/O program order.

The trace formats are interpreted as follows:

**SEL  csrw: Device number 4 asserted REQ for channel initialization
**SEL   pio: Channel loaded IOCW 040000 (Control) from address 102757
**SEL state: Channel entered the Fetch sequence with 14 clock cycles remaining
**SEL    sr: Device number 4 asserted CHANSR

Operational message

**SEL  data: 00.041746  000000    dma write
**SEL  data: 00.000020  102757    absolute read

Memory access classification

Octal data (memory contents)

Octal address

Octal bank number

The Selector Channel state contains these registers:

| Name | Size | Radix | Symbolic | Description |
| --- | --- | --- | --- | --- |
| IDLE | 1 | 2 |  | Channel is inactive |
| SREQ | 1 | 2 |  | Channel is requesting service |
| DEVNO | 8 | 10 |  | Device number of the active interface |
| EXCESS | 32 | 10 |  | Channel cycles used in excess of allocation |
| SEQ | 3 | 10 |  | Current sequencer state |
| ORDER | 4 | 8 |  | Current SIO order |
| ROLOVR | 1 | 2 |  | Word count has rolled over |
| PFCNTL | 1 | 2 |  | Control word should be prefetched |
| PFADDR | 1 | 2 |  | Address word should be prefetched |
| BANK | 4 | 8 |  | Memory bank |
| WCOUNT | 12 | 10 |  | Word count |
| PCNTR | 16 | 8 |  | I/O Program Counter |
| CNTL | 16 | 8 |  | I/O Control Word |
| CNBUF | 16 | 8 |  | I/O Control Word buffer |
| ADDR | 16 | 8 |  | I/O Address Word |
| ADBUF | 16 | 8 |  | I/O Address Word buffer |
| INBUF | 16 | 8 |  | Input buffer |
| OUTBUF | 16 | 8 |  | Output buffer |

The INBUF and OUTBUF registers may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

## Multiplexer Channel

The HP 30036B Multiplexer Channel provides high-speed data transfer between from one to sixteen devices and the Series III main memory.  Concurrent transfers for multiple devices are multiplexed on a per-word basis, dependent on the service request priorities assigned to the participating interfaces.  Interfaces must have additional hardware to be channel-capable, as the channel uses separate control and data signals from those used for direct I/O.  In addition, the multiplexer and selector channels differ somewhat in their use of the signals, so interfaces are generally designed for use with one or the other (the Selector Channel Maintenance Board is a notable exception that uses jumpers to indicate which channel to use).

Once started by an SIO instruction, the channel executes I/O programs independently of the CPU.  Program words are read, and device status is written back, by calls to the I/O Processor.

The MPX device supports configuration of the device number used for the diagnostic interface and of tracing using these device options:

| Option | Action |
| --- | --- |
| DEVNO=<n> | Set the device number; default is 127 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Channel control, status, read, and write actions |
| PIO | Programmed I/O commands issued |
| STATE | Channel state changes executed |
| SR | Service requests received |
| IOBUS | I/O bus signals and data words received and returned |

The CSRW option traces the beginning and ending of channel programs, as well as control, status, read, and write commands sent to the diagnostic interface.  The PIO option traces programmed I/O orders that are sent to devices.  The STATE option traces the entries into each of the internal channel execution states.  The SR option traces service requests received from the devices.  The IOBUS option traces the I/O backplane signals and data received and returned via the diagnostic interface.  As the Multiplexer Channel uses the I/O Processor as its interface to main memory, channel memory access tracing is enabled by the IOP DATA trace option.

Examples of the trace formats follow:

**MPX  csrw: Device number 65 asserted REQ for channel initialization
**MPX  csrw: Control is address RAM | load registers | RAM address 15
**MPX   pio: Channel SR 0 loaded IOCW 050000 (Sense) from address 021207
**MPX state: Channel SR 3 entered State C
**MPX    sr: Device number 65 asserted SR0
**MPX iobus: Received data 000200 with signals DCONTSTB

The control, status, read, and write values, and the I/O bus signals are decoded and presented in symbolic format for easier interpretation.

The Multiplexer Channel state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| IDLE | 1 | 2 | Channel is inactive |
| COUNT | 32 | 10 | Count of active transfers |
| EXCESS | 32 | 10 | Channel cycles used in excess of allocation |
| CNTL | 16 | 8 | Control word |
| STAT | 16 | 8 | Status word |
| ROLOVR | 1 | 2 | Word Count Rollover flip-flop |
| DEVEND | 1 | 2 | Device End flip-flop |
| STATR [0:15] | 4 | 2 | State RAM, SR 0-15 |
| AUX [0:15] | 6 | 8 | Auxiliary RAM, SR 0-15 |
| ORDER [0:15] | 4 | 8 | I/O Order RAM, SR 0-15 |
| CNTR [0:15] | 16 | 8 | Counter RAM, SR 0-15 |
| ADDR [0:15] | 16 | 8 | Address RAM, SR 0-15 |

## Channel Program Processor

The Channel Program Processor is part of the standard microcode of the IMB-based systems, and a modified CPP is also used with the HP 30341A HP-IB Interface Module, where it resides on the processor card.  It assumes the function of the multiplexer and selector channels of the IOP-based systems, except that instead of having dedicated hardware to execute channel programs, dedicated firmware is used instead.  The CPP gains control between machine instructions when the IMB CSRQ or IRQ signal is asserted.  The CPP also executes a subset of the I/O instructions of the HP-IB machines on behalf of the HP 30340A Intermodule Bus Adapter.  When the CPP exits, either at the end of a channel program or when the program is waiting for a device event, such as a parallel poll response, CPU instruction execution resumes.

The CPP device has no configuration options.

### Tracing

The CPP device supports configuration of the store and window specifications.

The storage states that may be configured for the TRACE STORE CPP command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Channel program command and instruction executions |
| OPND |  | SIOP channel program instructions |
| SERV |  | Channel processor service events |
| DATA |  | Data read from and written to memory |

The CMD state traces the commands and channel instructions executed by the channel processor, as well as those I/O instructions executed at the request of the IMBA.  The OPND state traces the channel program initiated by an SIOP instruction.  This state attempts to list the entire program through the final Interrupt/Halt instruction reachable from the initial location.  It accounts for intermediate conditional and unconditional jumps but may misinterpret memory contents if the locations skipped contain data rather than instructions.  The SERV state traces channel processor event service scheduling and entries.  The DATA state traces memory accesses performed by the channel processor.

Omitting the list of storage states in a TRACE STORE CPP command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW CPP command is:

| Source | Value | Source Description |
| --- | --- | --- |
| CHANNEL | range | Channel addressing |

The CHANNEL source restricts the storage of all states to those associated with the range of channel numbers designated by the range.  The value is a list of individual channel numbers separated by commas and ranges of the form low-high.  For example, CHANNEL=3,7-9 would include in the trace list only those states pertaining to channel numbers 3, 7, 8, and 9.

The storage window is enabled when a channel in the specified range is addressed and is disabled when a different channel is addressed.  This source allows detailed tracing of specified channels while eliminating states for channels that are not of interest.

The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details

Examples of the trace formats follow:

**CPP  cmd: Channel processor servicing channel 1
**CPP  cmd: Channel processor executing IOCL for the IMBA
**CPP  cmd: Executing channel 11 Identify | response 0212H

**CPP opnd: 00.140576  001416    Read secondary 0EH count 20 address 00124621
**CPP opnd: 00.140603  001000    Wait | response 0036
**CPP opnd: 00.140605  002400    Device Specified Jump 140610
**CPP opnd: 00.140610  000601    Interrupt/Halt 0001 | CPVA 1

Channel program instruction

Octal data (memory contents)

Octal address

Octal bank number

**CPP serv: Channel processor running
**CPP serv: Completion delay 2 service scheduled
**CPP serv: Channel processor idled

**CPP data: 00.001423  003000    bank 0 read
**CPP data: 00.001424  000000    bank 0 read
**CPP data: 00.000770  000007    absolute read
**CPP data: 00.000774  100000    absolute write

Memory access classification

Octal data (memory contents)

Octal address

Octal bank number

Channel program instruction opcodes and operand values are decoded and presented in symbolic format for easier interpretation.  Memory operands are printed in the CPU's address radix, which is octal.  General data operands are printed in the CPU's data radix, which defaults to octal but may be set to a different radix.  Command secondaries and Amigo IDs are printed in hexadecimal and suffixed with the letter H.  Data accesses for values that must reside in bank 0, such as channel program instructions, are classified separately from values that may reside in any bank, such as DRT entries.

### Registers

The Channel Program Processor state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| ENTDLY | 24 | 10 | Entry delay until CSRQ recognition |
| IRQEN | 1 | 2 | Interrupt requests are enabled |
| IMBIRQ | 8 | 8 | Pending interrupt device number |
| LIMIT | 16 | 2 | Set of channels that have reached their execution limit |
| RESTART [0:15] | 8 | 2 | Set of channel devices that are to be restarted |
| FILTER | 16 | 2 | Bitmap of channels to include in traces |

# Programmed I/O Device Simulations

The Series III CPU controls these I/O device interfaces with direct I/O instructions:

- 30032B Asynchronous Terminal Controller

- 30033A Selector Channel Maintenance Board

- 30135A System Clock

- 30340A Intermodule Bus Adapter

## 30032B Asynchronous Terminal Controller

The HP 30032B Asynchronous Terminal Controller is a 16-channel terminal multiplexer used with the HP 3000 CX through Series III systems.  The ATC connects from 1 to 16 serial terminals or modems to the HP 3000 at programmable baud rates from 75 to 2400 bits per second.  Character sizes are also programmable from 5 to 12 bits in length, including the start and stop bits.  Each channel can be independently configured, including for separate send and receive rates.  The ATC is not buffered, so the CPU has to retrieve each character from a given channel before the next character arrives.  To avoid saturating the CPU with interrupt requests, the ATC maintains an internal "mini-interrupt" system that queues requests and holds additional interrupts off until the CPU acknowledges the current request.

The HP 3000CX and Series I use a dedicated serial interface for the system console, while user terminals are connected to the ATC.  For the Series II and III, the separate card is eliminated, and channel 0 of the ATC is reserved for the console.

The ATC consists of two devices:

- ATCD — the Terminal Data Interface (TDI)

- ATCC — the Terminal Control Interface (TCI)

The Terminal Data Interface provides the serial data line connections for terminals and data sets.  Five additional receive-only auxiliary channels may be connected as a group under software control to one of the sixteen main lines to detect the incoming baud rate.  The Terminal Control Interface provides serial control and status lines for Bell 103 data sets.

### Terminal Data Interface

The TDI provides the Transmitted Data (BA) and Received Data (BB) lines for up to sixteen terminals.  It performs input and output through Telnet sessions connected to a user-specified listening port or through individually specified host serial ports.  The TDI supports concurrent Telnet and serial connections.  The ATTACH command specifies the local port to be used for Telnet connections:

ATTACH ATCD <port>

...where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  When the TDI is attached and the simulator is running, the multiplexer listens for connections on the specified port and assigns them to channels in ascending numeric order.

The ATTACH command is also used to specify the host serial port for an individual TDI channel:

ATTACH ATCD<chan> <port-name>[;<rate>-<size><parity><stopbits>]

...where chan is the TDI channel number from 1-15, and port-name is the host name of the serial port to use (e.g., /dev/ttyS0 or COM1).

An optional serial port configuration string may be supplied after the host name.  The required values are:

- rate is the baud rate in bits per second.

- size is the character size in bits including the parity bit, if designated.

- parity designates the parity to use: N (no), E (even), O (odd), M (mark), or S (space).

- stopbits is the number of stop bits (1, 1.5, or 2).

If the port configuration string is omitted, the default configuration specified by the host system for that port is used.

TDI configuration options are available for the device and for the individual units.  The command forms are:

SET ATCD <device-option>
SET ATCDn <unit-option>

Device options that may be specified are:

| Option | Action |
| --- | --- |
| CONNECT | Wait for and establish a channel connection |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| TERMINAL | Connect using Telnet or serial ports; default |
| DIAGNOSTIC | Connect using diagnostic test cables |
| LINEORDER=<c1>[;<c2>...] | Set the channel connection order |
| DEVNO=<n> | Set the device number; default is 7 |
| INTMASK=<n> | Set the interrupt mask; default is E |
| INTPRI=<n> | Set the interrupt priority; default is 0 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

Connections to the listening port or a serial port are normally established automatically while the simulation is executing.  Connections attempted while the simulation is stopped are deferred until simulated execution is resumed.  In cases where connections must be established before simulation resumes, such as when the executing program immediately writes to or reads from the channel, the SET ATCD CONNECT command may be used.  If a connection is pending, it will be established immediately.  Otherwise, the simulator will wait for a connection.  To abort the wait manually and return to the SCP prompt, enter the SCP interrupt character, which defaults to CTRL+E.

The TDI supports programmable data transfer rates from 75 to 2400 baud.  When realistic timing is enabled, the simulation accurately models the transmission and reception baud rates (in machine instructions).  For example, a port set for 1200 baud will take twice as long to output a given listing as a port set for 2400 baud.  Optimized timing reduces the timing to the minimum necessary to operate correctly; this is much faster than the real terminal multiplexer would operate.

The delay time used by the simulation in FASTTIME mode may be set via the register interface.  This value may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the abnormally rapid TDI operation.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized time.

Enabling the diagnostic mode simulates the installation of eight HP 30062-60003 diagnostic test (loopback) cables between channels 0-1, 2-3, etc., as required by the multiplexer diagnostic.  Each cable connects the Send Data line of one channel to the Receive Data line of the other channel, and vice versa.  In addition, all sessions are disconnected, and the multiplexer is detached from the Telnet listening port.  While in diagnostic mode, the ATTACH ATCD command is not allowed.  Enabling terminal mode allows the multiplexer to be attached to accept incoming connections again.

The LINEORDER option specifies the order in which new connections are assigned to multiplexer channels.  The arguments may be single channel numbers or ranges of channel numbers of the form low-high, with multiple arguments separated by semicolons, except that channel 0 may not be included, as it is reserved for the system console.  Telnet connections to the listening port will be assigned to multiplexer channels in the sequence specified.  Omitted channels will not receive connections, unless the ALL keyword is supplied as the last argument.  In the absence of a SET ATCD LINEORDER command, connections will be assigned by default in ascending channel order.  The default order may be reestablished by specifying the command SET ATCD LINEORDER=ALL.

Unit options that may be specified for individual TDI channels are:

| Option | Action |
| --- | --- |
| LOCALACK | Discard ENQ and reply with ACK internally; default |
| REMOTEACK | Transmit ENQ and receive ACK from the remote device |
| CAPSLOCK | Upshift lowercase input characters to uppercase; default for channel 0 |
| NOCAPSLOCK | Input characters are unchanged; default for channels 1-15 |
| UC | Upshift lowercase output characters to uppercase |
| 7B | Output with high-order bit cleared; default for channels 1-15 |
| 7P | Output with high-order bit cleared, non-printing suppressed; default for channel 0 |
| 8B | Output characters without changing |
| LOG=<filename> | Enable output logging |
| NOLOG | Disable output logging; default |
| DISCONNECT | Disconnect the channel |

Channels that are configured for MPE terminal type 10 expect HP terminals or terminal emulators to be connected and will handshake transfers by sending an ENQ and expecting an ACK in reply.  A device that does not provide ENQ/ACK handshaking will hang during output.  However, if LOCALACK is specified, the handshake will take place locally within the simulator.  The ENQ will be discarded, and a locally generated ACK will be returned to the caller.  In addition to enabling the use of non-HP terminals, this option significantly improves the performance of common HP terminal emulators.  Specifying REMOTEACK will pass ENQ to the terminal for handling; this is necessary to avoid output overruns if a real HP terminal is connected to a serial port.

Some HP 3000 programs, e.g., SLEUTH, require command input in upper case, although mixed-case output is supported.  As an aid to avoid toggling the host keyboard in and out of CAPS LOCK mode, the multiplexer provides this function locally.  The default modes are CAPSLOCK for channel 0 (the system console) and NOCAPSLOCK for the other channels.

Each channel may be set to one of four output modes (UC, 7B, 7P, or 8B).  The default mode is 7P for channel 0 and 7B for the other channels.  If the system console is set to MPE terminal type 10, the mode must be changed to 7B or 8B to allow ESC, DC1, and ENQ characters to pass through to the terminal.  Alternatively, the SET CONSOLE PCHAR command may be used to redefine the set of printable characters.  Modes UC and 7P are compatible with terminal types 0-9.

File transfer using the Reflection terminal emulator requires an 8-bit data path.  To achieve this, the session must use MPE terminal type 12 (this may be configured either during a system reload or by specifying the TERM=12 parameter when logging on with :HELLO), and the channel must be set to 8B, REMOTEACK, and NOCAPSLOCK modes.

Each channel supports independent I/O logging to a file.  Adding the –N (new file) switch clears the contents of the log file if it is present.  Without the –N switch, channel output will be appended to any preexisting log file content.  Disabling logging also closes the log file.

A channel may be manually disconnected from its associated Telnet session with the SET ATCDn DISCONNECT command.  Otherwise, the connection will remain open until disconnected either by the Telnet client or a DETACH ATCD command, unless the channel is controlled by the TCI.  For a serial connection, the SET ATCDn DISCONNECT command will drop and then raise the Data Terminal Ready line; to disconnect the serial port, issue a DETACH ATCDn command.

TDI device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW ATCD | Display the device and unit configuration |
| SHOW ATCD MODES | Display the connection modes |
| SHOW ATCD CONNECTIONS | Display the channel connections |
| SHOW ATCD STATISTICS | Display the channel I/O statistics |
| SHOW ATCD LINEORDER | Display the channel connection order |
| SHOW ATCD DEVNO | Display the device number assignment |
| SHOW ATCD INTMASK | Display the interrupt mask setting |
| SHOW ATCD INTPRI | Display the interrupt priority assignment |

TDI unit configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW ATCD<n> | Display the selected channel configuration |
| SHOW ATCD<n> LOG | Display the selected channel logging status |

In addition to the current configuration settings, a channel controlled by the TCI will be marked data set in the listing; a channel not controlled will be marked direct.

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Interface control, status, read, and write actions |
| SERV | Line unit service events |
| PSERV | Poll unit service events (periodic) |
| XFER | Data receptions and transmissions |
| IOBUS | I/O bus signals and data words received and returned |

The CSRW option traces control, status, read, and write commands sent to the interface.  The SERV option traces line unit event service scheduling and entries.  The line service is entered to transmit or receive each character at the configured baud rate.  The PSERV option traces poll unit event service entries.  The poll service is entered once every 10 milliseconds to poll the Telnet port for connections and input characters to be received by the active channels.  The XFER option traces the characters received and transmitted by the active channels.  The IOBUS option traces the I/O backplane signals and data received and returned via the interface.

Examples of the trace formats follow:

**ATCD  csrw: Channel 2 connection established
**ATCD  csrw: Status is DIO OK | complete | send
**ATCD  serv: Channel 0 delay 1708 service scheduled
**ATCD pserv: Poll service entered
**ATCD  xfer: Channel 0 character 'G' sent
**ATCD iobus: Received data 000000 with signals DREADSTB

The Terminal Data Interface state contains these registers:

| Name | Size | Radix | Symbolic | Description |
| --- | --- | --- | --- | --- |
| CNTL | 16 | 8 |  | Control register |
| STAT | 16 | 8 |  | Status register |
| READ | 16 | 8 |  | Read register |
| WRITE | 16 | 8 |  | Write register |
| FLAG | 1 | 2 |  | Data flag flip-flop |
| MASK | 1 | 2 |  | Interrupt mask flip-flop |
| FTIME | 24 | 10 |  | Fast receive/send time |
| RSTAT [0:20] | 16 | 8 |  | Receive channel status, channels 0-20 |
| RPARM [0:20] | 16 | 8 |  | Receive channel parameters, channels 0-20 |
| RBUFR [0:20] | 16 | 8 |  | Receive channel buffers, channels 0-20 |
| SSTAT [0:15] | 16 | 8 |  | Send channel status, channels 0-15 |
| SPARM [0:15] | 16 | 8 |  | Send channel parameters, channels 0-15 |
| SBUFR [0:15] | 16 | 8 |  | Send channel buffers, channels 0-15 |

The READ, WRITE, RBUFR, and SBUFR registers may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

### Terminal Control Interface

The TCI provides the Request to Send (CA) and Data Terminal Ready (CD) control lines, and the Data Set Ready (CC) and Carrier Detect (CF) status lines for each of sixteen terminals or data sets.  The modem controls model the Bell 103A data set without ring detection.

Data Terminal Ready must be set to enable a channel to accept an incoming connection.  When a channel connects, Data Set Ready and Carrier Detect will be set.  Configuring the terminal subtype to 1 in MPE will enable the TCI to establish these settings.  In addition, aborting a session will drop Data Terminal Ready after the user is logged out, which will drop the Telnet connection.  If the user drops the Telnet connection manually, the session will be logged out.  These actions will not occur for channels configured for terminal subtype 0 in MPE, which does not use the TCI.

ATCC device options that may be specified are:

| Option | Action |
| --- | --- |
| TERMINAL | Connect using Telnet or serial ports; default |
| DIAGNOSTIC | Connect using diagnostic test cables |
| DEVNO=<n> | Set the device number; default is 8 |
| INTMASK=<n> | Set the interrupt mask; default is E |
| INTPRI=<n> | Set the interrupt priority; default is 8 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

Enabling the diagnostic mode simulates the installation of eight HP 30062-60003 diagnostic test (loopback) cables between channels 0-1, 2-3, etc., as required by the multiplexer diagnostic.  Each cable connects the Data Terminal Ready and Request to Send lines, respectively, of one channel to the Data Set Ready and Carrier Detect lines of the other channel, and vice versa.  Enabling terminal mode allows the multiplexer to respond to incoming connections again.

There are no TCI units or unit commands.

TCI device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW ATCC | Display the device configuration |
| SHOW ATCC MODE | Display the connection mode |
| SHOW ATCC DEVNO | Display the device number assignment |
| SHOW ATCC INTMASK | Display the interrupt mask setting |
| SHOW ATCC INTPRI | Display the interrupt priority assignment |

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Interface control, status, read, and write actions |
| PSERV | Poll unit service events (periodic) |
| XFER | Control and status line changes |
| IOBUS | I/O bus signals and data words received and returned |

The CSRW option traces control, status, read, and write commands sent to the interface.  The PSERV option traces poll unit event service entries.  The poll service is entered once every 10 milliseconds to poll the channels for changes in the control or status lines.  The XFER option traces the changes in control and status lines by the active channels.  The IOBUS option traces the I/O backplane signals and data received and returned via the interface.

Examples of the trace formats follow:

**ATCC  csrw: Channel 2 control is C2 | C1 | ~S2 | ~S1
**ATCC pserv: Poll service entered
**ATCC  xfer: Channel 2 line status is RTS | DTR
**ATCC iobus: Received data 031374 with signals DCONTSTB

The Terminal Control Interface state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| CNTL | 16 | 8 | Control register |
| STAT | 16 | 8 | Status register |
| CNTR | 16 | 10 | Channel counter |
| SCAN | 1 | 2 | Scan flip-flop |
| MASK | 1 | 2 | Interrupt mask flip-flop |
| C2 [0:15] | 1 | 2 | Control line 2, channels 0-15 |
| C1 [0:15] | 1 | 2 | Control line 1, channels 0-15 |
| S2 [0:15] | 1 | 2 | Status line 2, channels 0-15 |
| S1 [0:15] | 1 | 2 | Status line 1, channels 0-15 |
| ES2 [0:15] | 1 | 2 | Enable status line 2 interrupt, channels 0-15 |
| ES1 [0:15] | 1 | 2 | Enable status line 1 interrupt, channels 0-15 |
| MS2 [0:15] | 1 | 2 | Status line 2 match, channels 0-15 |
| MS1 [0:15] | 1 | 2 | Status line 1 match, channels 0-15 |

## 30033A Selector Channel Maintenance Board

The HP 30033A Selector Channel Maintenance Board provides the circuitry necessary to test the I/O bus signals driven and received by the Selector and Multiplexer Channels.  Used with the Stand-Alone Selector Channel and Multiplexer Channel diagnostics, the SCMB is used to verify that the correct bus signals are driven in response to each of the programmed I/O orders, and that the channel responds correctly to the signals returned to it.  The SCMB functions as a programmable interface that can log incoming signals and drive the outgoing signals, as well as simulate a number of interface hardware faults.  Two SCMBs are provided; they are named SCMB1 and SCMB2.

Both SCMBs are normally disabled, as they are used only with the standalone diagnostics.  The CPU and Selector Channel diagnostics both use one SCMB.  If SCMB1 is used, it may be referred to as SCMB if desired.  The Multiplexer Channel diagnostic uses both SCMBs.

Device options that may be specified for the two SCMBs are:

| Option | Action |
| --- | --- |
| SC | Connect the SCMB to the Selector Channel bus |
| MX | Connect the SCMB to the Multiplexer Channel bus; default |
| DEVNO=<n> | Set the device number; defaults are 65 and 66 |
| INTPRI=<n> | Set the interrupt priority; defaults are 10 and 11 |
| SRNO=<n> | Set the service request number; defaults are 0 and 1 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable device |
| DISABLED | Disable device; default |

The SC option installs the SCMB on the Selector Channel bus and configures operation for the Selector Channel diagnostic.  The MX option installs the SCMB on the Multiplexer Channel bus and configures operation for the Multiplexer Channel and CPU diagnostics.  The SRNO value may be changed only when the SCMB is connected to the Multiplexer Channel bus.

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Interface control, status, read, and write actions |
| XFER | Data read and write transfers |
| SERV | SR delay service events |
| IOBUS | I/O bus signals and data words received and returned |

The CSRW option traces control, status, read, and write commands sent to the interface.  The XFER option traces data words transferred to and from the interface by the channel.  The SERV option traces event service scheduling and entries used to schedule delays in the service requests to the channel.  The IOBUS option traces the I/O backplane signals and data received and returned via the interface.

Examples of the trace formats follow:

**SCMB1  csrw: Control is device end | load control IOAW | count nothing
**SCMB1  xfer: Counter/buffer value 022222 read
**SCMB1  serv: Delay 2 SR service scheduled
**SCMB1 iobus: Received data 000000 with signals ACKSR | PCONTSTB | CHANSO

The SCMB state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| CNTL | 16 | 8 | Control register |
| STAT | 16 | 8 | Status register |
| CNTR | 16 | 8 | Counter/buffer register |
| SIOBSY | 1 | 2 | SIO is active |
| CHANSR | 1 | 2 | Channel service request is active |
| DEVSR | 1 | 2 | Device service request is active |
| INXFR | 1 | 2 | Input transfer is active |
| OUTXFR | 1 | 2 | Output transfer is active |
| JMPMET | 1 | 2 | Jump condition is met |
| XFRERR | 1 | 2 | Transfer error condition is present |
| EOT | 1 | 2 | End of transfer condition is present |
| TRMCNT | 1 | 2 | Terminal count condition is present |
| MISCMP | 1 | 2 | Miscompare condition is present |
| DEVEND | 1 | 2 | Device end condition is present |
| STOP | 1 | 2 | Transfer has been stopped |

## 30135A System Clock

The HP 30135A System Clock is used with Series II and III systems and provides a programmable interval clock employed as the MPE time-of-day and process-switching clock.  The clock provides programmable periods of 10 microseconds to 10 seconds in decade increments.  Each “tick” of the clock increments a presettable counter that may be compared to a selected limit value.  The clock may request an interrupt when the values are equal, and a status indication is provided if the counter reaches the limit a second time without acknowledgement.

CLK device options that may be specified are:

| Option | Action |
| --- | --- |
| CALTIME | Use calibrated timing; default |
| REALTIME | Use realistic timing |
| DEVNO=<n> | Set the device number; default is 3 |
| INTPRI=<n> | Set the interrupt priority; default is 1 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

Calibrated timing aligns the simulated clock periods with the clock on the host system.  When calibrated, each of the programmable periods will elapse after the corresponding amount of host-system time.

When realistic timing is enabled, the simulation models the programmable periods in terms of machine instructions executed.  Calibrated timing is required to enable idling.  Realistic timing is necessary to pass the hardware diagnostics.

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Interface control, status, read, and write actions |
| PSERV | Clock unit service events (periodic) |
| IOBUS | I/O bus signals and data words received and returned |

The CSRW option traces control, status, read, and write commands sent to the interface.  The PSERV option traces event service scheduling and entries, which occur at a periodic rate dependent on the programmable configuration.  The IOBUS option traces the I/O backplane signals and data received and returned via the interface.

Examples of the trace formats follow:

**CLK  csrw: Control is load rate | select limit | 1 millisecond rate
**CLK  csrw: Status is DIO OK | LR = CR | limit selected | 1 millisecond rate
**CLK pserv: Service entered with counter 0 increment 1 limit 1
**CLK pserv: Rate 1 millisecond delay 389 service rescheduled
**CLK iobus: Received data 000000 with signals DSETINT
**CLK iobus: Returned data 000000 with signals INTREQ

The System Clock state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| CNTL | 16 | 8 | Control Register |
| STAT | 16 | 8 | Status Register |
| COUNT | 16 | 8 | Count Register |
| LIMIT | 16 | 8 | Limit Register |
| RATE | 3 | 8 | Clock Rate Register |
| SYSIRQ | 1 | 2 | System Interrupt Request flip-flop |
| LIMIRQ | 1 | 2 | Count = Limit Interrupt Request flip-flop |
| OVFIRQ | 1 | 2 | Count = Limit Overflow Interrupt Request flip-flop |

## 30340A Intermodule Bus Adapter

The HP 30340A Intermodule Bus Adapter is a component of the 30341A HP-IB Interface Module (nicknamed the "Starfish") that connects HP-IB peripheral devices to the Series III.  HP-supported devices are the HP 7933 CS/80 disc drive, the HP 7976 ½-inch magnetic tape drive, and the HP 2680 laser page printer.  A small, rack-mounted chassis houses the module and contains these Printed Circuit Assemblies:

- 30340-60002 Intermodule Bus Adapter (IMBA)

- 30070-60012 Processor

- 31000-60053 Bus Interface Controller (BIC)

- 31262-60001 General I/O Channel (GIC)

- 30340-60001 Backplane (6 position)

The IMBA connects via flat ribbon cables to the Series III IOP bus, the IOP/Power bus, and to an additional 30030-60020 Port Controller installed in the Series III chassis.  The Processor is a Series 33 processor card containing modified firmware that implements the Channel Program Processor and Series 33 I/O instruction set.  The BIC and GIC are standard Series 33 cards.

Intermodule Bus

Port Controller

IMB Adapter

GIC

GIC

Main Memory

IOP Bus

IOP/Power Bus

HP-IB Discs

HP-IB Tapes

and Printers

BIC

Processor

Graphically, the module appears as follows:

In simulation, the following I/O devices are used in a Series III system to implement the HP-IB Interface Module

- 30340A Intermodule Bus Adapter

- Channel Program Processor

- 31262A General I/O Channel

- CS/80 Disc Drives

- Amigo Tape Drives

The HP 30340A Intermodule Bus Adapter is installed in the Starfish chassis and serves as the gateway between the Series III and the Starfish.  It serves three functions.

First, it enables the Series III to communicate with the Starfish processor that runs HP-IB channel programs.  It is attached to the Series III's IOP Bus and IOP Power Bus, and to the Starfish's Intermodule Bus (IMB).  Issuing an SIO order to the IMBA asserts the Channel Service Request (CSRQ) line on the IMB, causing the Starfish processor to run its Channel Program Processor (CPP) microcode.  The CPP retrieves the memory address of the channel program and other parameters from a "mailbox" area in the Series III main memory and initiates program execution.  Channel program completion status is returned to the Series III via the mailbox.  The IMBA is permanently assigned to IMB channel 1.

Second, it permits the DMA controller on the Starfish's General I/O Channel (GIC) to read and write data in the Series III main memory.  The IMBA interprets IMB Memory Read and Memory Write cycles and passes them through to a dedicated HP 30030 Port Controller card in the Series III.  It reflects main memory parity errors back to the GIC via the IMB.

Third, it allows the Starfish processor to assert the Series III Interrupt Request (INTREQ) line to cause an external interrupt.  Because the Starfish supports multiple devices and therefore multiple interrupt sources, the IMBA has a DEVNO response register that permits it to "masquerade" as different devices during an interrupt poll cycle.  The Starfish processor can configure the IMBA to supply any desired device number, rather than its own permanently assigned device number of 125.

The IMBA device supports configuration of the interrupt priority and of tracing using these device options:

| Option | Action |
| --- | --- |
| INTPRI=<n> | Set the interrupt priority; default is 12 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the HP-IB Module devices |
| DISABLED | Disable the HP-IB Module devices; default |

Unlike most Series III interfaces, the IMBA is not controlled by an I/O driver. Instead, MPE automatically detects the presence of the IMBA at system startup, regardless of whether or not HP-IB peripherals are configured into the system.  Therefore, the IMBA is disabled by default and must be enabled explicitly when HP-IB devices are to be used.  The HP-IB Interface Module operates as a unit, so enabling or disabling the IMBA enables or disables the entire module, affecting the CPP, GIC, DC, and MA devices.

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CSRW | Interface control, status, read, and write actions |
| DATA | Port controller data accesses |
| IOBUS | I/O Processor bus signals and data words received and returned |
| IMBUS | Intermodule Bus signals and data words received and returned |

The CSRW option traces control, status, read, and write commands sent to the interface.  The DATA option traces memory accesses performed by the port controller on behalf of the GIC.  The IOBUS option traces the I/O Processor backplane signals and data received and returned via the interface.  The IMBUS option traces the Intermodule Bus signals and data received and returned via the interface.

Examples of the trace formats follow:

**IMBA  csrw: Status is SIO OK | DIO OK | Power OK
**IMBA  csrw: Channel program started
**IMBA  data: 00.131220  004400    dma read
**IMBA  data: 00.124621  000017    dma write
**IMBA iobus: Received data 000000 with signals DSTATSTB
**IMBA iobus: Returned data 160000 with signals (none)
**IMBA imbus: Channel 1 asserts CSRQ1
**IMBA imbus: Channel 1 received opcode I/O Write command SMSK data 000020
**IMBA imbus: Channel 1 returned data 040000 with signals ADN | DDN | PRO

The control, status, and I/O bus signals are decoded and presented in symbolic format for easier interpretation.

The IMB Adapter state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| CSRQ | 1 | 2 | Channel service request flip-flop |
| DEVNO | 8 | 8 | Device number for interrupt acknowledgement |

# Selector Channel I/O Device Simulations

The Selector Channel controls these Series III I/O device interfaces:

- 30229B Disc Interface

## 30229B Disc Interface with Eight 7905/7906/7920/7925 Drives

The HP 30129A Cartridge Disc Subsystem connects the 7905A, 7906A, 7920A, and 7925A disc drives to the HP 3000.  The subsystem consists of a 30229B Cartridge Disc Interface, a 13037D Multiple-Access Disc Controller ("MAC"), and from one to eight MAC drives.  The subsystem uses the Selector Channel to achieve a 937.5 KB/second transfer rate to the CPU.

The disc controller connects from one to eight HP 7905 (15 MB), 7906 (20 MB), 7920 (50 MB), or 7925 (120 MB) disc drives to interfaces installed in from one to eight CPUs.  The drives use a common command set and present data to the controller synchronously at a 468.75 kiloword per second (2.133 microseconds per word) data rate.

The disc interface is used to connect the HP 3000 CPU to the 13037’s device controller.  While the controller supports multiple-CPU systems, the HP 3000 does not use this capability.

The simulation provides up to eight disc drives.  Drive types may be intermixed; 7905s are selected by default.  Attaching a disc image file to a unit simulates inserting a disc pack into a drive:

ATTACH [{-R | -N}] DSn <image-filename>

Adding the –R (read-only) switch is equivalent to setting the drive’s Disc Protect or Read Only switch to the On position.  Adding the –N (new file) switch creates a full-size image file, equivalent to formatting the new disc before use.

If the host operating system returns an error when seeking, reading, or writing a disc image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator disc library I/O error: No space left on device

A simulated disc seek will fail with Status 2 Error (Drive Fault) status, and the drive’s heads will unload.  Reloading the heads will clear the drive fault.  A simulated disc read or write will fail with Uncorrectable Data Error status.  The target operating system will then react to this error as though a real drive had encountered a bad disc sector.

A drive’s unit number is not set explicitly.  Instead, the drive unit number is derived from the simulation unit number.  For example, unit DS0 responds to disc unit select number 0.  Changing the unit select switch on a mounted drive is equivalent to detaching and reattaching the disc image file to the corresponding simulation unit.

Device and unit options include configuring the timing, drive type, protection and format status, and the ability to set drives ready or not-ready.  The command forms are:

SET DS <device-option>
SET DSn <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| DIAGNOSTIC | Reset the diagnostic override table |
| DIAGNOSTIC=<params> | Add an entry to the diagnostic override table |
| NODIAGNOSTIC | Clear the diagnostic override table; default |
| DEVNO=<n> | Set the device number; default is 4 |
| INTMASK=<n> | Set the interrupt mask; default is E |
| INTPRI=<n> | Set the interrupt priority; default is 4 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW DS | Display the device and unit configuration |
| SHOW DS DIAGNOSTIC | Display the diagnostic override table |
| SHOW DS TIMING | Display the timing mode |

When realistic timing is enabled, the simulation accurately models the disc movement times (in machine instructions).  For example, seeking takes longer if the positioner movement distance is farther.  Optimized timing reduces the timing to the minimum necessary to operate correctly; this is much faster than a real disc drive would operate.

The delay times used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the abnormally rapid disc operation.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized times.

The SET DS DIAGNOSTIC and SET DS NODIAGNOSTIC commands provide diagnostic support.  See the Diagnostic Support section below for details.

### Unit Options

Unit options that may be specified for individual disc drives are:

| Option | Action |
| --- | --- |
| 7905 | Set the drive type to 7905; default |
| 7906 | Set the drive type to 7906 |
| 7920 | Set the drive type to 7920 |
| 7925 | Set the drive type to 7925 |
| UNLOAD | Set the drive’s Run/Stop switch to Stop; default when detached |
| LOAD | Set the drive’s Run/Stop switch to Run; default when attached |
| PROTECT | Set the 7920’s or 7925’s Read Only switch to On |
| PROTECT=UPPER | Set the 7905’s or 7906’s Protect Upper Disc switch to On |
| PROTECT=LOWER | Set the 7905’s or 7906’s Protect Lower Disc switch to On |
| UNPROTECT | Set the 7920’s or 7925’s Read Only switch to Off |
| UNPROTECT=UPPER | Set the 7905’s or 7906’s Protect Upper Disc switch to Off |
| UNPROTECT=LOWER | Set the 7905’s or 7906’s Protect Lower Disc switch to Off |
| FORMAT | Set the drive’s Format switch to Enabled |
| NOFORMAT | Set the drive’s Format switch to Disabled; default |
| ENABLED | Enable the unit; default |
| DISABLED | Disable the unit |

The UNOAD and LOAD options unload and load the drive's heads from the disc pack, setting the drive not-ready and ready, respectively.  The former provides a convenient method of setting a drive "down" without detaching the associated disc image file.

PROTECT=UPPER and PROTECT=LOWER protect the upper and lower platters, respectively, of 7905 and 7906 drives from writing; PROTECT without a value protects both platters.  For the 7920 and 7925 drives, PROTECT protects the entire drive.  The UNPROTECT options remove drive protection and enable writing.

The FORMAT option enables certain controller commands, such as Initialize, that alter the sector address fields.  Typically, this option is needed when reloading the operating system from tape to disc.  The NOFORMAT option inhibits these commands and permits only the standard Write command, subject to the appropriate drive protection status.

Drive configuration may be displayed with the following command:

| Command | Action |
| --- | --- |
| SHOW DS<n> | Display the selected drive’s configuration |

### Diagnostic Support

The offline disc diagnostic (program D419A) tests features of the disc that are not supported in simulation, e.g., the detection and correction of sector data errors.  However, the simulation does support recovery from host file system errors, which are mapped to uncorrectable data errors.  Because file system errors cannot be generated intentionally, a table of diagnostic overrides is provided to return specified error status values instead of otherwise normal completion status.  This allows the error recovery simulation code to be tested.

The initially empty diagnostic override table may be populated with one or more entries using this command:

SET DS DIAGNOSTIC=<cylinder>;<head>;<sector>;<opcode>;<SPD>;<status>

...where:

- cylinder is a decimal value from 0 to 822.

- head is a decimal value from 0 to 8.

- sector is a decimal value from 0 to 63.

- opcode is an octal value from 0 to 26.

- SPD is any combination of the letters S, P, and D.

- status is an octal value from 0 to 37.

If a command specifies the opcode value as 15 (Request Syndrome) and the status value as 17 (Correctable Data Error), then four additional values must be appended to the command line above:

;<displacement>;<syndrome 1>;<syndrome 2>;<syndrome 3>

...where:

- displacement is a decimal value from –5 to 135.

- syndrome 1-3 are octal values from 0 to 177777.

When the table is populated, the first entry becomes the initial “current” entry.  As the diagnostic program issues each disc controller command, the cylinder, head, sector, and command opcode values from the current override entry are checked against the corresponding current controller values.  If the values match, then the controller status and Spare/Protected/Defective values are set from the entry rather than being generated by the command, and the next entry becomes the current entry.  These values will then be returned to the program as the completion status of the current command.  This process continues until the table is exhausted or the program ends.  The current entry may be reset to the first entry by specifying the SET DS DIAGNOSTIC command with no parameters.  Entering the SET DS NODIAGNOSTIC command will clear the table.

If the disc controller command performs address verification, then any SPD/status value(s) provided will be used as the result of the verification.  In particular, setting the P bit for a Write command will cause a Protected Track error if the drive’s Format switch is set to Disabled, and any status value other than Normal Completion, Correctable Data Error, or Uncorrectable Data Error will cause a verification abort.

In hardware, the errors that may occur during verification are Cylinder Miscompare, Head-Sector Miscompare, Sync Timeout, Illegal Spare Access, and Defective Track; any of these errors may be simulated.  In addition, an Uncorrectable Data Error will occur in hardware if the controller is unable to verify any of the 16 sectors starting at the sector preceding the target sector, but this error cannot be simulated.  Instead, specifying either a Correctable Data Error or an Uncorrectable Data Error will cause an abort at the end of the first sector of a Read, Write, or Verify command.

Correctable Data Error and Uncorrectable Data Error may also be specified for the Request Syndrome command.  The former requires the values to be returned for the displacement and three syndrome words; the latter does not, as the values are meaningless if the error cannot be corrected.

### BOOT Command

The interface supports the BOOT command as an alternative to LOAD.  The BOOT DS0 command is equivalent in hardware to setting the SWCH register to configure the control byte and device number, and pressing the LOAD and ENABLE buttons to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | DS device number |  |  |  |  |  |  |  |

The control byte defaults to the Cold Load Read disc command.

### Tracing and Registers

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CMD | Controller commands executed |
| INCO | Controller command initiations and completions |
| CSRW | Interface control, status, read, and write actions |
| STATE | Controller command state changes executed |
| SERV | Disc unit service events |
| XFER | Data reads and writes |
| IOBUS | I/O bus signals and data words received and returned |

The CMD option traces the commands executed by the disc controller.  The INCO option traces the beginning and ending of commands, including command termination status.  The CSRW option traces control, status, read, and write commands sent to the interface.  The STATE option traces the entries into each of the internal controller execution states.  The SERV option traces disc unit event service scheduling and entries.  The XFER option traces the data words read from or written to the disc.  The IOBUS option traces the signals and data received and returned via the I/O backplane and interface and via the data, flag, and function buses between the interface and controller.

Examples of the trace formats follow:

**DS    cmd: Unit 0 Seek to cylinder 410 head 2 sector 47
**DS   inco: Unit 0 Seek command completed with Normal Completion status
**DS   csrw: Control is 004000 (Write)
**DS  state: Unit 0 Write data phase event entry
**DS   serv: Unit 0 Write data phase delay 1 service scheduled
**DS   xfer: Unit 0 Write word 3400 is 177777
**DS  iobus: Received data 005000 with signals PCONTSTB
**DS  iobus: Controller (idle) received data 005000 with flags CMRDY | EOD

The Disc Interface state contains these registers:

| Name | Size | Radix | Read- Only | Description |
| --- | --- | --- | --- | --- |
| SIOBSY | 1 | 2 |  | SIO is active |
| DEVSR | 1 | 2 |  | Device service request is active |
| INXFR | 1 | 2 |  | Input transfer is active |
| OUTXFR | 1 | 2 |  | Output transfer is active |
| INTMSK | 1 | 2 |  | Interrupt mask is enabled |
| JMPMET | 1 | 2 |  | Jump condition is met |
| DEVEND | 1 | 2 |  | Device end is active |
| DATOVR | 1 | 2 |  | Data overrun has occurred |
| ENDDAT | 1 | 2 |  | End of data has occurred |
| TEST | 1 | 2 |  | Test mode is active |
| WAIT | 1 | 2 |  | Data wait is active |
| CLEAR | 1 | 2 |  | A hardware clear is active |
| CMRDY | 1 | 2 |  | A command word is ready |
| DTRDY | 1 | 2 |  | A data word is ready |
| EOD | 1 | 2 |  | The last data word has been transferred |
| INTOK | 1 | 2 |  | An interrupt is allowed |
| OVRUN | 1 | 2 |  | The current transfer has overrun |
| XFRNG | 1 | 2 |  | The current transfer is no good |
| BUFFER | 16 | 8 |  | Data buffer register |
| STATUS | 16 | 8 |  | Status register |
| RETRY | 4 | 10 |  | Retry counter |
| OPCODE | 5 | 8 |  | Controller operation code |
| CSTATS | 5 | 8 |  | Controller status |
| CSTATE | 2 | 10 |  | Controller state |
| EOC | 1 | 2 |  | End of cylinder has been seen |
| VERIFY | 1 | 2 |  | Address verification is active |
| SPDU | 16 | 8 |  | Spare/Protected/Defective flags and unit number |
| FLMASK | 4 | 8 |  | File mask |
| CYL | 16 | 10 |  | Controller cylinder |
| HEAD | 6 | 10 |  | Controller head |
| SECTOR | 8 | 10 |  | Controller sector |
| COUNT | 16 | 10 |  | Word or sector counter |
| SECBUF [0:137] | 16 | 8 |  | Sector data buffer |
| INDEX | 8 | 10 |  | Sector buffer current index |
| LENGTH | 8 | 10 |  | Sector buffer valid length |
| TTIME | 24 | 10 |  | Fast one-track seek time |
| FTIME | 24 | 10 |  | Fast full-stroke seek time |
| STIME | 24 | 10 |  | Fast one-sector rotation time |
| XTIME | 24 | 10 |  | Fast one-word transfer time |
| GTIME | 24 | 10 |  | Fast intersector gap rotation time |
| OTIME | 24 | 10 |  | Fast controller overhead time |
| UCYL [0:8] | 10 | 10 |  | Current cylinder, drives 0-7 and controller |
| UOPCODE [0:8] | 6 | 8 |  | Current operation code, drives 0-7 and controller |
| USTATUS [0:8] | 32 | 2 |  | Unit status, drives 0-7 and controller |
| USTATE [0:8] | 4 | 10 |  | Current command state, drives 0-7 and controller |
| UPOS [0:8] | 32 | 10 |  | Current byte position, drives 0-7 and controller |
| UWAIT [0:8] | 32 | 8 |  | Scheduled wait delay, drives 0-7 and controller |

The BUFFER and SECBUF registers may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

# Multiplexer Channel I/O Device Simulations

The Multiplexer Channel controls these Series III I/O device interfaces:

- 30209A Line Printer Controller

- 30215A Tape Controller

## 30209A Line Printer Controller with One 2607/13/17/18 Line Printer

The HP 30118A, 30127A, 30128A, and 30133A Line Printer Subsystems connect the 2607A, 2613A, 2618A, and 2617A printers, respectively, to the HP 3000.  Each subsystem consists of a 30209A Line Printer Controller, employing a 30051A Universal Interface (Differential) and an interconnecting cable, and an HP 2607A (200 lines per minute), HP 2613 (300 lpm), HP 2617 (600 lpm), or HP 2618 (1250 lpm) line printer.  These subsystems employ the Multiplexer Channel to achieve a 360 KB/second transfer rate from the CPU.

The simulation provides one printer unit.  The 2617 is selected by default.  Attaching a text file to the unit simulates loading paper into the printer:

ATTACH [-N] LP <image-filename>

Adding the –N (new file) switch clears the contents of the image file if present.  Without the –N switch, printer output will be appended to any preexisting image file content.

The printer unit may also be attached to a FIFO (pipe) file to direct printer output to a receiving program.  The receiving end of the file must be connected before the ATTACH is given to avoid erroneous console operation.  The –N switch is not relevant when a FIFO file is attached.

Printer output written to the image file is typically buffered by the host operating system’s underlying stream I/O routines.  While it is running, the simulator flushes the file after each printer top-of-form request to permit convenient inspection of the image file.  Stopping the simulator also flushes the file.

If the host operating system returns an error when writing to the printer image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator printer I/O error: No space left on device

The printer goes offline with an alarm condition, and the simulator stops.  Simulation may then be resumed, either with the printer set back online if the problem is fixed, or with the printer remaining offline if the problem is uncorrectable.

Detaching the text file from the unit with the DETACH LP command simulates running out of paper.  If the command is entered while there are characters in the print buffer or, for the 2607 only, the print location is not at the top of the form, Command not completed is displayed on the simulation console, and the file remains attached until the required conditions are true.  Once simulation is resumed and the print operations complete, the printer is set offline and detached automatically.

Detaching may also be forced with the DETACH –F LP command.  This simulates physically removing the paper and takes effect immediately, regardless of any printing operations in progress.  A forced detach is performed automatically if the DETACH LP command is entered while the CPU is halted.

Device and unit options include configuring the printer type and timing, output format, vertical format unit (VFU), and the ability to set the printer offline or online.  The command forms are:

SET LP <device-option>
SET LP <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| PRINTER | Connect using the printer interface cable; default |
| DIAGNOSTIC | Connect using the Diagnostic Hardware Assembly |
| DEVNO=<n> | Set the device number; default is 14 |
| INTMASK=<n> | Set the interrupt mask; default is E |
| INTPRI=<n> | Set the interrupt priority; default is 18 |
| SRNO=<n> | Set the service request number; default is 11 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

The printer supports realistic and optimized timing modes.  Realistic timing attempts to model the print buffer load and print-and-space operation delays inherent in the physical hardware.  For example, output of lines with more characters takes longer than output of lines with fewer characters, and spacing six lines takes approximately six times longer than spacing one line.

Optimized timing reduces operation delays to the minimums necessary to operate correctly; this is much faster than a real line printer would operate.

The delays used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the unusually rapid print operations.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized times.

The DIAGNOSTIC option simulates the installation of the HP 30049C Diagnostic Hardware Assembly in place of the printer interface cable.  This is needed to run the offline universal interface diagnostic (program D435A).  The CNLED register provides the state of the CONT 6 through CONT 10 LEDs, and the J2WX, DATOUT, DCOUT, DFIN, and DENDIN registers provide the logic levels of the corresponding test pins on the DHA.

Setting the PRINTER option reconnects the cable between the line printer and the interface.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW LP | Display the device and unit configuration |
| SHOW LP MODES | Display the timing and connection modes |

### Unit Options

Unit options that may be specified are:

| Option | Action |
| --- | --- |
| 2607 | Set the printer model to 2607 |
| 2613 | Set the printer model to 2613 |
| 2617 | Set the printer model to 2617; default |
| 2618 | Set the printer model to 2618 |
| VFU | Install the HP standard 66-line VFU tape; default |
| VFU=<filename> | Install a custom VFU tape image |
| OFFLINE | Set the printer offline; default when detached |
| ONLINE | Set the printer online; default when attached |
| EXPAND | Write expanded output to the printer image file; default |
| COMPACT | Write compact output to the printer image file |

The 2607, 2613, 2617, and 2618 options select the printer model.  Each printer is configured with Option 001, which provides a 128-character set for the HP 2607 and 96-character sets for the HP 2613, 2617, and 2618.  The 2607 and 2618 support 132-character print line lengths, while the 2613 and 2617 support 136-character lines.  Exceeding the line length on the 2607 causes an automatic print-and-space operation.  Exceeding the line length on the 2613, 2617, or 2618 discards the excess characters.

The VFU option configures the printer’s vertical format unit, as described in the Vertical Format Unit section below.

The OFFLINE and ONLINE options place the printer offline and online, respectively.  The former provides a convenient method of setting the printer "down" without detaching the associated output file.

The printer will not go offline if there are characters in the print buffer.  Instead, the offline condition is held off until the line is printed and paper movement is complete.  The SET LP OFFLINE command checks for data in the print buffer or a print operation in progress.  If either condition is true, the action is deferred, and Command not completed is displayed on the simulation console.  A SHOW LP command will show that the device is still online.  Once simulation is resumed and the print operation completes, the printer is set offline.  No console message reports this, although a subsequent SHOW LP command will indicate the new status.

Entering a SET LP ONLINE command while an offline or detach action is deferred will cancel the action without triggering any programmed online-to-offline or offline-to-online status transition interrupt.  A RESET LP command also cancels any deferred offline or detach action.  Additionally, it clears the print buffer and terminates any print action in progress, so a SET LP OFFLINE or DETACH LP will succeed if issued afterward.

The EXPAND and COMPACT options control the format of lines written to the printer image file.  In compact mode, a carriage-return/line-feed character pair terminates a printed line, but subsequent line spacing is performed by line-feed characters alone.  A top-of-form request will emit a form-feed character instead of the number of line-feeds required to reach the top of the next form.  This mode is suitable for sending the printer output to a physical printer connected to the host.

In expanded mode, paper advance is handled by emitting the correct number of carriage-return/line-feed pairs.  This mode is suitable for retaining printer output as a text file.

The HP 2613, 2617, and 2618 printers are capable of overprinting.  If the printer is sent a format code to suppress spacing, the characters in the buffer are printed and the buffer is emptied, but the paper is not advanced.  The next print operation will print its characters over those already present on the paper.  In simulation, overprinting is performed in one of two ways, depending on the print mode.

In compact mode, overprinting is simulated by emitting a carriage-return character at the end of the line.  In expanded mode, overprinting is simulated by merging characters in the buffer before writing them as a single line in the printer image file.  As the second and subsequent lines are output, each new character is compared with its corresponding character in the buffer.  If the character in the buffer is a space, the new character replaces it.  If the new character is a space or is the same as the character in the buffer, the character in the buffer is retained.  Otherwise, the character in the buffer is replaced with a special character representing an overprinted combination.  This character defaults to DEL (octal 177) but may be changed by altering the OVPCHR register value.

The HP 2607 printer cannot overprint.  A request to suppress spacing will result in a single-line paper advance after printing.

### Vertical Format Unit

The HP 2607 supports an 8-channel vertical format unit (VFU), and the HP 2613, 2617, and 2618 printers support 12-channel VFUs.  A continuous punched paper tape that controls paper spacing is installed in the VFU reader.  The printer may be commanded to advance the paper until a punched hole is detected in a specified VFU channel.  The length of the tape establishes the length of the forms loaded into the printer.

Initially, the standard VFU tape (part number 1535-2655 for the HP 2607 or 02613-80001 for the HP 2613, 2617, and 2618) is installed.  This tape associates channels 1-8 with the following printer actions:

| Channel | Printer Advances to |
| --- | --- |
| 1 | Top of form |
| 2 | Bottom of form |
| 3 | Next single space |
| 4 | Next double space |
| 5 | Next triple space |
| 6 | Next half-page |
| 7 | Next quarter-page |
| 8 | Next sixth-page |

For the 02613-80001 tape, channel 9 is punched the same as channel 2, and channels 10-12 are uncommitted.

The simulator supports the use of custom VFU tape images.  A custom tape may be installed in place of the standard tape by issuing the SET LP VFU=<filename> command.  Custom tapes must reserve channel 1 for the top-of-form location, but the other channels may be defined as desired.

A custom tape image is a plain-text file that starts with a VFU definition line and then contains one channel-definition line for each line of the form.  The number of channel-definition lines establishes the form length.

A semicolon appearing anywhere on a line begins a comment, and the semicolon and all following characters are ignored.  Zero-length lines, including lines beginning with a semicolon, are ignored.  Note that a line containing one or more blanks is not a zero-length line, so, for example, the line “ ; a comment starting in column 2” is not ignored.

The first (non-ignored) line in the file must be a VFU definition line of this exact form:

VFU=<punch characters>,<no-punch character>[,<title>]

...where:

- punch characters is a set of one or more characters used interchangeably to represent a punched location.

- no-punch character is a single character representing a non-punched location.

- title is an optional description that is printed by the SHOW LP VFU command; the description “Custom VFU” is used if the title is omitted.

If the VFU line is missing or not of the correct form, then Format error is displayed on the simulation console, and the VFU tape is not changed.

The remaining lines define the channels punched for each line of the printed form.  The line format consists of a sequence of punch, no-punch, and “other” characters, in channel order.  Each punch or no-punch character defines a channel state, starting with channel 1 and proceeding left-to-right until all channels for the VFU are defined; any the extra channel states on the line are ignored.  If the line terminates before all channels are defined, the remaining channels are set to the no-punch state.  Any "other" characters, i.e., neither punch characters nor no-punch characters, are ignored and may be used freely to delineate the tape channels.

For a standard 66-line form, the first printable line (form line 1) is paper line 4, and the last printable line (form line 60) is paper line 63.  The first channel-definition line in the file specifies form line 1, i.e., the top of the form, and must have a punch in channel 1.  Channel line 60 defines form line 60, i.e., the bottom of the form.  Six more channel lines follow: three for the bottom margin, and three for the top margin of the next form.

An example custom VFU file using a 66-line tape definition for an 8-channel VFU is:

; the VFU definition
;
; the set of punch characters is “1” and “X”
; the no-punch character is “0”
; all other characters are ignored

VFU=1X,0,A binary tape image

; the channel definitions

1 0 1 1 1 1 1 1   ; top of form (line 1; must have a punch in channel 1)
0-0-X-0-0-0-0-0   ; single space (line 2)
0011              ; channels 5-8 default to no-punch (line 3)
[...]
0 1 1 0 0 0 0 0   ; bottom of form (line 60)
0 0 0 0 0 0 0 0   ; bottom form margin (line 61)
0 0 0 0 0 0 0 0   ; bottom form margin (line 62)
0 0 0 0 0 0 0 0   ; bottom form margin (line 63)
0 0 0 0 0 0 0 0   ; top of form margin (line 64)
0 0 0 0 0 0 0 0   ; top of form margin (line 65)
0 0 0 0 0 0 0 0   ; top of form margin (line 66)

If a custom tape has been used, the standard tape may be reinstalled by issuing the SET LP VFU command.

Attempting to command an advance to a channel that is not punched will cause a tape fault, and the printer will go offline.  Setting the printer back online will clear the fault.

The current VFU definition may be displayed with the following command:

| Command | Action |
| --- | --- |
| SHOW LP VFU | Display the currently loaded VFU tape definition |

This command displays the current VFU tape title and then the channel definitions for each form line.  By default, a punched channel is indicated by an “O” character, and an unpunched channel is indicated by a period (“.”).  These characters may be changed by depositing new values into the PUNCHR and UNPCHR registers, respectively.

### Tracing and Registers

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CMD | Printer commands executed |
| CSRW | Interface control, status, read, and write actions |
| SERV | Printer, pulse, and transfer timer unit service events |
| XFER | Data transmissions |
| STATE | Device handshake state changes executed |
| IOBUS | I/O bus signals and data words received and returned |

The CMD option traces the commands executed by the printer.  The CSRW option traces control, status, read, and write commands sent to the interface.  The SERV option traces printer and interface unit event service entries.  The XFER option traces the data words and format commands written to the printer.  The STATE option traces the scheduling of and entries into each of the internal device handshake execution states.  The IOBUS option traces the signals and data received and returned via the I/O backplane.

Examples of the trace formats follow:

**LP   cmd: Printed 132 characters on line 8
**LP   cmd: Printer commanded to slew to VFU channel 1 from line 9
**LP   cmd: Printer advanced 58 lines to line 1
**LP  csrw: Control is character | interrupt status | word xfer
**LP  csrw: Status is DIO OK | interrupt | interrupt status | system
**LP  serv: Transfer delay 1491 service scheduled
**LP  serv: Device Command 1 state printer service entered
**LP  xfer: Character 'M' sent to printer
**LP  xfer: Format code 001 sent to printer
**LP state: Sequencer transitioned from Idle state to Device Flag 1 state
**LP iobus: Received data 046515 with signals ACKSR | PWRITESTB | CHANSO
**LP iobus: Returned data 060004 with signals INTREQ | JMPMET

The Line Printer Controller state contains these registers:

| Name | Size | Radix | Symbolic | Read-Only | Description |
| --- | --- | --- | --- | --- | --- |
| SIOBSY | 1 | 2 |  |  | SIO is active |
| CHANSR | 1 | 2 |  |  | Channel service request is active |
| DEVSR | 1 | 2 |  |  | Device service request is active |
| INXFR | 1 | 2 |  |  | Input transfer is active |
| OUTXFR | 1 | 2 |  |  | Output transfer is active |
| RDXFR | 1 | 2 |  |  | Read transfer is in progress |
| WRXFR | 1 | 2 |  |  | Write transfer is in progress |
| INTMSK | 1 | 2 |  |  | Interrupt mask is active |
| DEVCMD | 1 | 2 |  |  | Device command |
| DEVFLG | 1 | 2 |  |  | Device flag |
| DEVEND | 1 | 2 |  |  | Device end is active |
| SEQSTA | 8 | 10 |  |  | Sequencer state |
| CNTL | 16 | 8 |  |  | Control word |
| ISTAT | 16 | 8 |  |  | Interrupt status |
| DSTAT | 16 | 8 |  |  | Device status |
| READ | 16 | 8 |  |  | Read word |
| WRITE | 16 | 8 |  |  | Write word |
| J2WX | 10 | 2 |  |  | Jumper J2W10-J2W1 configuration |
| DATOUT | 16 | 8 |  |  | Data out word |
| DATIN | 16 | 8 |  |  | Data in word |
| DCOUT | 1 | 2 |  |  | Device command out |
| DFIN | 1 | 2 |  |  | Device flag in |
| DENDIN | 1 | 2 |  |  | Device end in |
| DIAGCN | 16 | 8 |  |  | DHA control word |
| CNLED | 5 | 2 |  |  | Control bit LEDs CONT 6-CONT 10 |
| PFWARN | 1 | 2 |  |  | Power-fail warning exists |
| PFAULT | 1 | 2 |  |  | Paper fault exists |
| TFAULT | 1 | 2 |  |  | Tape fault exists |
| OLPEND | 1 | 2 |  |  | Offline transition is pending |
| PRLINE | 8 | 10 |  |  | Current print line number |
| BUFIDX | 8 | 10 |  |  | Current print buffer index |
| PRTBUF [0:423] | 8 | 8 |  |  | Print buffer |
| OVPCHR | 8 | 8 |  |  | Overprint character |
| FORMLN | 8 | 10 |  |  | Form length in lines |
| VFU [0:144] | 12 | 2 |  |  | Vertical format unit channels |
| PUNCHR | 8 | 8 |  |  | Punched channel character |
| UNPCHR | 8 | 8 |  |  | Unpunched channel character |
| BTIME | 24 | 10 |  |  | Fast printer buffer load delay time |
| PTIME | 24 | 10 |  |  | Fast printing delay time |
| STIME | 24 | 10 |  |  | Fast paper slew per-line delay time |
| POS | 32 | 10 |  |  | Printer file current position |

The PRTBUF, OVPCHR, PUNCHR, and UNPCHR registers default to single-character format display and entry but may be overridden with numeric-format switches, if desired.  The READ, WRITE, DATOUT, and DATIN registers default to octal display but may be displayed in single-character format by specifying the –A switch.  Symbolic entry for these registers is also supported.

The PFAULT and TFAULT registers indicate a paper fault and a tape fault, respectively, when they contain the value 1.  Attaching the printer will clear a paper fault.  Setting the printer online will clear a tape fault.

The character used to represent an overprinted position may be changed by depositing a new value into the OVPCHR register.  For example, the DEPOSIT LP OVPCHR '@' command will change the character from the default DEL (octal 177) to the commercial-at sign (octal 100).  Similarly, the characters used to represent punched and unpunched VFU channels in the SHOW LP VFU display may be changed by depositing new values in the PUNCHR and UNPCHR registers, respectively.

The FORMLN register holds the current VFU form length, and the VFU register array holds the content of the current VFU tape.  The register defaults to binary display, with channel 1 in the most-significant bit and channel 8 or 12 in the least-significant bit, depending on the VFU width.  Elements 1-n correspond to VFU form lines 1-n, where a 1 value indicates a punch, and a 0 value indicates a no-punch.  Element 0 is the logical OR of elements 1-n, so a 1 value indicates that the channel is punched somewhere on the tape, and a 0 value indicates that the channel is not punched.

## 30215A Tape Controller with Four 7970B/E Drives

The HP 30115A Magnetic Tape Subsystem connects the 7970B/E ½-inch magnetic tape drives to the HP 3000.  The subsystem consists of a 30215A two-card tape controller processor and controller interface, and from one to four HP 7970B 800-bpi NRZI or HP 7970E 1600-bpi PE drives.  The two drive types may be mixed on a single controller.  The subsystem uses the Multiplexer Channel to achieve a 36 KB/second (NRZI) or 72 KB/second (PE) transfer rate to the CPU.

The simulation provides up to four tape drives.  7970Es are selected by default.  Attaching a tape image file to a unit simulates mounting a tape reel on a drive:

ATTACH [-R] [-F] MSn [<format>] <image-filename>

Adding the –R (read-only) switch is equivalent to mounting the tape without a write ring in place.  Adding the –F switch and a format identifier declares the tape image format to be used.  Supported formats are SIMH, E11, TPC, and P7B.  If the –F switch and a format identifier are not supplied, then SIMH tape image format is used.  Note that erase gaps embedded in the tape image file are supported only in SIMH image format mode.

If the host operating system returns an error when reading or writing a tape image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator tape library I/O error: No space left on device

If this or another tape library error occurs, e.g., due to an illegal or damaged tape format, the simulator stops with an appropriate error message.  Resuming the simulation will fail the tape operation with Tape Error status.  The target operating system will then react to this error as though a real drive had encountered a bad tape block (CRC or MTE).

A drive’s unit number is not set explicitly.  Instead, the drive unit number is derived from the simulation unit number.  For example, unit MS0 responds to tape unit select number 0.  Pressing a different unit select button on a mounted drive is equivalent to detaching and reattaching the tape image file to the corresponding simulation unit.  Pressing the OFF button is equivalent to setting the drive offline.

Device and unit options include configuring the drive type and timing, tape reel size, tape image format, and the ability to set drives offline or online.  The command forms are:

SET MS <device-option>
SET MSn <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| DEVNO=<n> | Set the device number; default is 6 |
| INTMASK=<n> | Set the interrupt mask; default is E |
| INTPRI=<n> | Set the interrupt priority; default is 14 |
| SRNO=<n> | Set the service request number; default is 3 |
| DEBUG=<option> | Enable tracing |
| NODEBUG | Disable tracing; default |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

When realistic timing is enabled, the simulation accurately models the tape movement times (in machine instructions).  For example, rewinding takes longer if the tape is positioned farther from the load point.  Realistic timing is necessary to pass the magnetic tape diagnostic timing tests.

Optimized timing reduces the timing to the minimum necessary to operate correctly; this is much faster than a real tape drive would operate.  In addition, enabling optimized timing also omits the initial erase gap normally written after the BOT marker.  This does not affect gaps written with the Write Gap command.

The delay times used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the unusually rapid tape operation.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized times.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW MS | Display the device and unit configuration |
| SHOW MS TIMING | Display the timing mode |

### Unit Options

Unit options that may be specified for individual tape drives are:

| Option | Action |
| --- | --- |
| 7970B | Set the drive type to 7970B |
| 7970E | Set the drive type to 7970E; default |
| REEL=<n> | Set the reel size in feet; default is unlimited |
| CAPACITY=<n> | Set the reel capacity in megabytes; default is unlimited |
| OFFLINE | Set the unit offline; default when detached |
| ONLINE | Set the unit online; default when attached |
| FORMAT=<fmt> | Set the tape image format; default is SIMH format |
| ENABLED | Enable the unit; default |
| DISABLED | Disable the unit |

The reel size may be set to 600-, 1200-, or 2400-foot capacity.  Setting the capacity or reel size to 0 specifies unlimited capacity; in this configuration, the controller never returns an end-of-tape indication.

The OFFLINE and ONLINE options place a drive offline and online, respectively.  The former provides a convenient method of setting a drive "down" without detaching the associated tape image file.

The tape image format for future ATTACH commands may be set to one of the format identifiers listed previously.  The unit must be detached when the format is set.

Drive configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW MS<n> | Display the selected drive’s configuration |
| SHOW MS<n> REEL | Display the selected drive’s reel size or capacity |
| SHOW MS<n> CAPACITY | Display the selected drive’s reel size or capacity |
| SHOW MS<n> FORMAT | Display the selected drive’s tape image format |

### BOOT Command

The interface supports the BOOT command as an alternative to LOAD.  The BOOT MS0 command is equivalent in hardware to setting the SWCH register to configure the control byte and device number, and pressing the LOAD and ENABLE buttons to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | MS device number |  |  |  |  |  |  |  |

The control byte defaults to the Read Record tape command.

### Tracing and Registers

When debug output logging is enabled, tracing may be configured by specifying one or more of the reporting level options:

| Option | Reporting Level |
| --- | --- |
| CMD | Controller commands executed |
| INCO | Controller command initiations and completions |
| CSRW | Interface control, status, read, and write actions |
| STATE | Controller command state changes executed |
| SERV | Tape unit service events |
| XFER | Data reads and writes |
| IOBUS | I/O bus signals and data words received and returned |

The CMD option traces the commands executed by the tape controller.  The INCO option traces the beginning and ending of commands, including the starting and ending tape position.  The CSRW option traces control, status, read, and write commands sent to the interface.  The STATE option traces the scheduling of and entries into each of the internal controller execution states.  The SERV option traces tape unit event service entries.  The XFER option traces the data words read from or written to the tape.  The IOBUS option traces the signals and data received and returned via the I/O backplane and interface and via the data, flag, and function buses between the interface and controller.

Examples of the trace formats follow:

**MS    cmd: Tape unit 0 loaded and set online
**MS    cmd: Tape unit 0 Write Record of 20-byte record succeeded
**MS   inco: Tape unit 0 Write Record started at position 6120
**MS   inco: Tape unit 0 Write Record completed at position 6148
**MS   csrw: Control is 000005 (Write Gap)
**MS   csrw: Status is ready | load point | 1600 bpi | no error | unit 0
**MS  state: Tape unit 0 Write Gap start phase delay 3794 service scheduled
**MS   serv: Tape unit 0 service entered
**MS   xfer: Tape unit 0 Write Record word 1 is 052525
**MS  iobus: Tape controller idle state returned data 000000 functions RQSRV
**MS  iobus: Received data 125252 with signals ACKSR | PWRITESTB | CHANSO

The Tape Controller state contains these registers:

| Name | Size | Radix | Read-Only | Description |
| --- | --- | --- | --- | --- |
| SIOBSY | 1 | 2 |  | SIO is active |
| CHANSR | 1 | 2 |  | Channel service request is active |
| DEVSR | 1 | 2 |  | Device service request is active |
| INXFR | 1 | 2 |  | Input transfer is active |
| OUTXFR | 1 | 2 |  | Output transfer is active |
| INTMSK | 1 | 2 |  | Interrupt mask is active |
| UINTRP | 1 | 2 |  | Unit interrupt is active |
| DEVEND | 1 | 2 |  | A device end has occurred |
| XFRERR | 1 | 2 |  | A transfer error has occurred |
| BUFWRD | 16 | 8 |  | Buffer word |
| ATUNIT | 16 | 10 |  | Unit number requesting attention |
| CLASS | 4 | 10 |  | Current command classification |
| FLAGS | 8 | 2 |  | Interface state flags |
| CSTATE | 4 | 10 |  | Controller execution state |
| STATUS | 16 | 8 |  | Controller status |
| USEL | 4 | 10 |  | Unit number currently selected |
| UATTN | 4 | 2 |  | Bitmap of units requesting attention |
| RECBUF [0:65537] | 8 | 8 |  | Record buffer |
| LIBSTA | 16 | 10 |  | Status from last tape support library call |
| LENGTH | 24 | 10 |  | Data buffer valid length |
| INDEX | 24 | 10 |  | Data buffer current index |
| GAPLEN | 32 | 10 |  | Length of erase gap preceding the current record |
| INPOS | 32 | 10 |  | Initial tape position |
| RSTART | 24 | 10 |  | Fast rewind start time |
| RRATE | 24 | 10 |  | Fast rewind rate |
| RSTOP | 24 | 10 |  | Fast rewind stop time |
| BTIME | 24 | 10 |  | Fast start time if positioned at the BOT |
| ITIME | 24 | 10 |  | Fast start time if positioned at an inter-record gap |
| DTIME | 24 | 10 |  | Fast data transfer time per byte |
| OTIME | 24 | 10 |  | Fast controller overhead time |
| UPROP [0:3] | 16 | 8 |  | Drive properties, drives 0-3 |
| USTATUS [0:3] | 16 | 2 |  | Unit status, drives 0-3 |
| UOPCODE [0:3] | 6 | 10 |  | Current operation code, drives 0-3 |
| USTATE [0:3] | 4 | 10 |  | Current command state, drives 0-3 |
| UPOS [0:3] | 32 | 10 |  | Current byte position, drives 0-3 |

The BUFWRD and RECBUF registers may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

# Intermodule Bus I/O Device Simulations

The Channel Program Processor controls these Intermodule Bus device interfaces:

- 31262A General I/O Channel (GIC)

- 31264A Asynchronous Data Communications Channel (ADCC)

The simulator provides the following HP-IB devices that may be attached to the GIC:

- CS/80 Disc and Tape Drives

- 7970E ½-inch Magnetic Tape Drives

## 31262A General I/O Channel

The HP 31262A General I/O Channel (GIC) connects HP-IB disc and tape units to the Intermodule Bus.  It serves as the gateway between the CPU and the Hewlett-Packard Interface Bus.

Each GIC provides a Processor to HP-IB (PHI) chip that acts as the bus controller to manage up to eight connected devices.  Each device may be from a different protocol class (disc vs. tape vs. printer, CS/80 vs. Amigo).  Although there are hardware restrictions on the type and number of devices that may be connected to a single GIC, those restrictions are not present in simulation.  Five hardware configuration switches are present on the GIC PCA.

In addition to the PHI, each GIC contains a DMA sequencer, a watchdog timer, and bus request and handshake circuitry.  It connects to the Intermodule Bus and has sixteen addressable registers — eight provided by the PHI, and eight provided by the DMA sequencer.  The sequencer is a state machine with 27 states; the current five-bit state number is presented as bits 3-7 of Register 8.  Once the target device is addressed to talk or listen and the DMA sequencer is configured with the starting address, byte count, and direction, the GIC is able to transfer the entire block to or from main memory without CPU intervention.  Transfers are performed with IMB memory read and write cycles.  When installed in a Starfish chassis, these are handled by the IMBA, which passes the requests through to a dedicated Series III port controller that interfaces to main memory.

### Device Options

Device options include configuring the channel number and various switch settings.  The command form is:

SET GIC <device-option>

The options that may be specified are:

| Option | Action |
| --- | --- |
| COUNT=<n> | Set the number of GIC instances; default is 1 |
| CHANNEL=<n> | Set switch S4 to the channel number; default is 11 |
| HPIB | Connect the HP-IB cable to the peripherals; default |
| DIAGNOSTIC | Connect the HP-IB cable to a second GIC |
| CPU | Set switch S1 to the CPU position; default |
| CPP | Set switch S1 to the CPP position |
| A | Set switch S2 to the A position; default |
| B | Set switch S2 to the B position |
| SYS | Set switch S3 to the ON (System Controller) position; default |
| NOSYS | Set switch S3 to the OFF (not System Controller) position |
| OPERATE | Set switch S5 to the OPER position; default |
| TEST | Set switch S5 to the TEST position |
| ENABLED | Enable the device; default for IMB systems |
| DISABLED | Disable the device; default for IOP systems |

The COUNT option determines the number of separate instances of the GIC interface.  While the various system configuration guides state that a small number of GICs are supported in hardware, this limitation does not exist in simulation.  If additional GIC instances are desired, the total number may be specified with the COUNT option.  For example, issuing the SET GIC COUNT=3 command will create two additional GIC instances, while issuing SET GIC COUNT=1 will remove all instances except the original one.  When more than one GIC instance exists, the original GIC is renamed GICA, while the additional GICs are named GICB, GICC, etc.  The original GIC will return to its original name if the count is reduced to 1.

The CHANNEL option configures the GIC to use the specified IMB channel address from 0-15.  Peripheral devices that connect to the GIC must be set to the same channel number.

Setting the DIAGNOSTIC option simulates the installation of an HP-IB cable between two GIC cards as required by the General I/O Channel Diagnostic; the DIAGNOSTIC option also must be set on the other GIC to complete the connection.  Setting the HPIB option simulates the installation of an HP-IB cable between the GIC and the peripherals.

The CPU, CPP, A, B, SYS, NOSYS, OPERATE, and TEST options configure the positions of the S1, S2, S3, and S5 hardware switches on the GIC PCA.  The default settings are used for normal MPE operation.  The alternate settings are used only when running the GIC diagnostic.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW GIC | Display the complete device configuration |
| SHOW GIC CHANNEL | Display the channel address |
| SHOW GIC COUNT | Display the number of GIC instances |

The GIC device has two units that implement the watchdog timer and the NRFD holdoff timer.  These units are disabled and not user-configurable.

In response to a START, LOAD, DUMP, BOOT, RUN, GO, CONTINUE, or STEP command to begin instruction execution, a check is made to ensure that GIC channel address conflicts and bus address conflicts are not present.  If the check detects an inconsistent configuration, the simulator will report the error to the simulation console, and execution will be inhibited.  For example:

sim> SET DC CHANNEL=11
sim> SET MA CHANNEL=11
sim> SHOW DC
DC, fast timing, channel=11, 8 units
  DC0, 404MB, not attached, 7933A, bus=0, unit=0
sim> SHOW MA
MA, fast timing, channel=11, bus=0, 4 units
  MA0, not attached, unlimited capacity, SIMH format, offline, write ring
  MA1, not attached, unlimited capacity, SIMH format, offline, write ring
  MA2, not attached, unlimited capacity, SIMH format, offline, write ring
  MA3, not attached, unlimited capacity, SIMH format, offline, write ring
sim> RUN
Bus address 0 conflict (DC and MA)

Simulation stopped, P: 000000 (NOP,NOP)
sim> SET MA CHANNEL=9


sim> SET GIC COUNT=2
sim> SHOW GICA
GICA, channel=11, HP-IB cable, ...
sim> SHOW GICB
GICB, channel=11, HP-IB cable, ...
sim> RUN
Channel number 11 conflict (GICA and GICB)

Simulation stopped, P: 000000 (NOP,NOP)
sim>

The simulator will not begin program execution until all conflicts are resolved.

### Tracing

The GIC device supports configuration of the store and window specifications.  The storage states that may be configured for the TRACE STORE GIC command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Interface commands executed |
| CSRW |  | Interface control, status, read, and write actions |
| CSIRQ |  | Interface channel service and interrupt request actions |
| SERV |  | Interface unit service events |
| XFER |  | Data received and transmitted via HP-IB |
| BUF |  | Data read from and written to the FIFO buffer |
| STATE |  | DMA sequencer state changes executed |
| IMBRQ |  | Intermodule Bus CSRQ and IRQ signal assertions |
| IMBUS |  | Intermodule Bus signals and data words received and returned |

The CMD state traces the commands executed by the PHI and DMA controller.  The CSRW state traces the control and status commands sent to the GIC.  The CSIRQ state traces channel service and interrupt requests and responses.  The SERV state traces the watchdog timer event scheduling and entries.  The XFER state traces the command and data bytes passed across the HP-IB cable.  The BUF state traces the data bytes loaded into and unloaded from the PHI FIFO buffers.  The STATE state traces DMA state machine changes.  The IMBRQ state traces the Intermodule Bus channel service request and interrupt request signal assertions.  The IMBUS state traces the Intermodule Bus signals and data received and returned via the channel.

Omitting the list of storage states in a TRACE STORE GIC command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW GIC command is:

| Source | Value | Source Description |
| --- | --- | --- |
| BUS | range | Bus device addressing |

The BUS source restricts the storage of states associated with device addresses other than those included in the range of bus addresses designated by the range.  The value is a list of individual addresses separated by commas and ranges of the form low-high.  For example, BUS=0-2,5 would exclude from the trace list any states pertaining to bus device addresses 3, 4, 6, and 7.

The storage window is initially enabled.  It is disabled when a device not in the specified range is addressed to talk or listen and is enabled when the device is unaddressed.  This source allows detailed tracing of specified bus device addresses while excluding states for devices that are not of interest.

Some states, such as the IMBUS states and certain CMD states, are not associated with any talker or listener.  Storage of these states is not inhibited by the window setting.

The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Examples of the trace formats follow:

**GIC   cmd: DMA completed with status 0
**GIC   cmd: PHI becomes system controller and goes offline with reset
**GIC   cmd: PHI becomes controller-in-charge with IFC
**GIC   cmd: PHI relinquishes controller-in-charge by going online
**GIC   cmd: DMA aborted

**GIC  csrw: Register 0 (FIFO) Data 171 sent to bus
**GIC  csrw: Register 0 (FIFO) Data Tagged | 001 received from bus
**GIC  csrw: Register 7 (Bus address) control is online | bus address 0
**GIC  csrw: Register 1 (Status) status is controller | system controller





**GIC csirq: Service requested for PHI interrupt

**GIC  serv: Timeout clock enabled
**GIC  serv: Channel 11 timeout delay 400778 service scheduled
**GIC  serv: Timeout service entered

**GIC  xfer: REN | IFC | SRQ | ATN | EOI | DAV | NRFD | NDAC | DIO | Mnemonic
**GIC  xfer: REN |     |     | ATN | EOI | DAV |      |      | 01H | Poll
**GIC  xfer: REN |     |     | ATN |     | DAV |      |      | DFH | Untalk
**GIC  xfer: REN |     |     |     |     | DAV |      |      | 40H | Data

**GIC   buf: Command byte 076 loaded into outbound FIFO count 1
**GIC   buf: Enable reception count 2 unloaded from outbound FIFO count 0
**GIC   buf: Tagged data byte 022 loaded into inbound FIFO count 2
**GIC   buf: Attempted load into full outbound FIFO data 000000


**GIC state: DMA executing state 8 count 20
**GIC state: DMA exiting in state 8 count 19

**GIC imbrq: Channel 11 asserts IRQ
**GIC imbrq: Channel 11 device 0 interrupted as DEVNO 88
**GIC imbrq: Channel 11 asserts CSRQ1

**GIC imbus: Channel 11 received opcode I/O Write command IOCL data 000000
**GIC imbus: Channel 14 asserts CSRQ1
**GIC imbus: Channel 14 returned data 000000 with signals ADN | DDN | PRO

### Registers

The GIC device state contains these registers:

| Name | Size | Radix | Description |
| --- | --- | --- | --- |
| REN | 1 | 2 | Bus REN signal |
| IFC | 1 | 2 | Bus IFC signal |
| SRQ | 1 | 2 | Bus SRQ signal |
| ATN | 1 | 2 | Bus ATN signal |
| EOI | 1 | 2 | Bus EOI signal |
| DAV | 1 | 2 | Bus DAV signal |
| NRFD | 1 | 2 | Bus NRFD signal |
| NDAC | 1 | 2 | Bus NDAC signal |
| DIO | 8 | 16 | Bus data |
| TALK | 32 | 2 | Bus talker bitmap |
| LSTN | 32 | 2 | Bus listeners bitmap |
| INFIFO [0:7] | 16 | 8 | PHI inbound FIFO data |
| OUTFIFO [0:7] | 16 | 8 | PHI outbound FIFO data |
| PHIREG[0:7] | 16 | 8 | PHI configuration registers 0-7 |
| GICREG[0:7] | 16 | 8 | GIC configuration registers 8-F |
| DMBUSY | 1 | 2 | DMA busy |
| DMSTATUS | 2 | 10 | DMA termination status |
| DMSTATE | 5 | 10 | DMA state number |
| DMADDRS | 16 | 8 | DMA address |
| DMCOUNT | 16 | 10 | DMA remaining byte count |
| IRQMASK | 16 | 8 | Interrupt request mask |

The INFIFO and OUTFIFO registers are arranged in first-in, first-out order.  Element 0 is always the first word to be removed from the FIFO memory, element 1 is the second word to be removed, etc.  The IRQMASK value shows the word supplied in the last SMSK IMB command.  The GIC interrupt mask flip-flop is set if the bit corresponding to the channel number is set.

## 31264A Asynchronous Data Communications Channel

The HP 31264A Asynchronous Data Communications Channel is an eight-port terminal multiplexer used with the

HP 3000 Series 30 through 70 systems.  The ADCC consists of a main channel that provides the first four ports and an extender channel that provides the second four.  Together, they connect from one to eight serial terminals or modems to the HP 3000 at 14 programmable baud rates from 50 to 9600 bits per second.  Character sizes are also programmable from 5 to 8 bits in length, excluding the start bit and one or two stop bits.  Parity is optional and may be configured for even or odd parity generation and checking.  Each port can be independently configured, including for separate send and receive rates.

MPE reserves port 0 of the ADCC on IMB channel 1 for the system console.  The other ports of the first ADCC, and all ports of additional ADCCs, may be used for user terminals.

The ADCC provides the Transmitted Data (BA) and Received Data (BB) serial lines, the Request to Send (CA), Data Terminal Ready (CD), Data Rate Selector (CH), and Secondary Request to Send (SCA) control lines, and the Clear to Send (CB), Data Set Ready (CC) Ring Indicator (CE), Data Carrier Detect (CF), and Secondary Data Carrier Detect (SCF) status lines for each of eight terminals or data sets.  The CE and CH signals are available only when the extender card is present.

The ADCC performs input and output through Telnet sessions connected to a user-specified listening port or through individually specified host serial ports.  Concurrent Telnet and serial connections are allowed.  The ADCC also supports serial printers by specifying the log files to capture the printed output.  No additional connections are needed to use this feature.

When configured by MPE as a modem port, Data Terminal Ready must be set to enable the port to accept an incoming connection.  When the port connects, Data Set Ready and Data Carrier Detect will be set.  In addition, aborting a session will drop Data Terminal Ready after the user is logged out, which will drop the Telnet connection.  If the user drops the Telnet connection manually, the session will be logged out.

The ATTACH command specifies the local network port to be used for Telnet connections:

ATTACH ADCC <netport>

...where netport is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  When the ADCC is attached and the simulator is running, the multiplexer listens for connections on the specified network port and assigns them by default to ADCC ports in ascending numeric order.

When multiple ADCC instances exist, each may be attached to its own unique network listening port.  With this configuration, each Telnet connection would specify the ADCC instance to use by its network port choice.  Alternatively, ADCC instances may be attached to port groups that use the same network listening port.  Attaching a second instance to the same port number as the first will make the two instances part of the same port group.  Telnet connections to the common listening port will assign the connections to the available terminal ports provided by the first instance and, when those are all occupied, to the ports provided by the second instance.  Only when all terminal ports of all ADCC instances belonging to the same port group are in use will a connection attempt be refused.  Multiple concurrent port groups are also possible by assigning different network ports to each group of instances that belong to a given port group.  A mix of individual and grouped ADCC instances is also possible.

The ATTACH command is also used to specify the host serial port for an individual ADCC port:

ATTACH ADCC<n> <serport-name>[;<rate>-<size><parity><stopbits>]

...where n is the ADCC port number from 0-3 or 0-7, depending on whether the extender is present, and serport-name is the host name of the serial port to use (e.g., /dev/ttyS0 or COM1).

An optional serial port configuration string may be supplied after the host name.  The required values are:

- rate is the baud rate in bits per second.

- size is the character size in bits including the parity bit, if designated.

- parity designates the parity to use: N (no), E (even), O (odd), M (mark), or S (space).

- stopbits is the number of stop bits (1, 1.5, or 2).

If the port configuration string is omitted, the default configuration specified by the host system for that port is used.

Configuration options are available for the device and for the individual units.  The command forms are:

SET ADCC <device-option>
SET ADCCn <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| COUNT=<n> | Set the number of ADCC instances; default is 1 |
| CHANNEL=<n> | Set switch S1 to the channel number; default is 1 |
| TERMINAL | Connect using Telnet or serial ports; default |
| DIAGNOSTIC | Connect using the diagnostic test connector |
| CPU | Set switch S2 to the CPU position; default |
| CPP | Set switch S2 to the CPP position |
| EXTENDER | Install the main and extender ADCC cards; default |
| NOEXTENDER | Install the main ADCC card only |
| CONNECT | Wait for and establish a port connection |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| LINEORDER=<c1>[;<c2>...] | Set the port connection order |
| ENABLED | Enable the device; default for IMB systems |
| DISABLED | Disable the device; default for IOP systems |

The COUNT option determines the number of separate instances of the ADCC interface.  If additional ADCC instances are desired, the total number may be specified with the COUNT option.  For example, issuing the SET ADCC COUNT=3 command will create two additional ADCC instances, while issuing SET ADCC COUNT=1 will remove all instances except the original one.  When more than one ADCC instance exists, the original ADCC is renamed ADCCA, while the additional ADCCs are named ADCCB, ADCCC, etc.  The original ADCC will return to its original name if the count is reduced to 1.

The CHANNEL option configures the ADCC to use the specified IMB channel address from 1-15.  One ADCC must be configured for channel 1 to support the MPE system console.  Additional ADCCs may specify any unused address.

Setting the DIAGNOSTIC option simulates the installation of the HP 5060-5563 ADCC Test Connector as required by the Asynchronous Data Communications Channel Diagnostic.  For each port, this connects the Send Data line to the Receive Data line, as well as the following sets of control and status lines: CA to CB and CF, CD to CC, SCA to SCF, and CH to CE.  In addition, all sessions are disconnected, and the multiplexer is detached from the Telnet listening port.  While in diagnostic mode, the ATTACH ADCC command is not allowed.  Setting the TERMINAL option simulates the installation of the ADCC Cable Hood between the ADCC and the terminal junction panel., allowing the multiplexer to be attached to accept incoming connections again.

The CPU and CPP options configure the position of the S2 hardware switches on the ADCC PCAs.  The default setting is used for normal MPE operation.  The alternate setting is used only when running the ADCC diagnostic.

The EXTENDER option installs the ADCC extender and the crossover connector between the main and extender PCAs to provide eight terminal ports and support for the CE and CH modem signals.  The NOEXTENDER option removes the extender, providing only the four ports of the main PCA.

Connections to the listening port or a serial port are normally established automatically while the simulation is executing.  Connections attempted while the simulation is stopped are deferred until simulated execution is resumed.  In cases where connections must be established before simulation resumes, such as when an executing program immediately writes to or reads from the terminal port, the SET ADCC CONNECT command may be used.  If a connection is pending, it will be established immediately.  Otherwise, the simulator will wait for a connection.  To abort the wait manually and return to the SCP prompt, enter the SCP interrupt character, which defaults to CTRL+E.

The ADCC supports programmable data transfer rates from 50 to 9600 baud.  When the REALTIME option is enabled, the simulation accurately models the transmission and reception baud rates (in machine instructions).  For example, a port set for 1200 baud will take twice as long to output a given listing as a port set for 2400 baud.  Selecting the FASTTIME option reduces the timing to the minimum necessary to operate correctly; this is much faster than the real terminal multiplexer would operate.

The delay time used by the simulation in FASTTIME mode may be set via the register interface.  This value may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the abnormally rapid operation.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized time.

The LINEORDER option specifies the order in which new connections are assigned to multiplexer ports.  The arguments may be single port numbers or ranges of port numbers of the form low-high, with multiple arguments separated by semicolons, except that port 0 may not be included for the ADCC configured as IMB channel 1, as it is reserved for the system console.  Telnet connections to the listening port will be assigned to multiplexer ports in the sequence specified.  Omitted ports will not receive connections, unless the ALL keyword is supplied as the last argument.  In the absence of a SET ADCC LINEORDER command, connections will be assigned by default in ascending port order.  The default order may be reestablished by specifying the command SET ADCC LINEORDER=ALL.

### Unit Options

Unit options that may be specified for individual ADCC ports are:

| Option | Action |
| --- | --- |
| LOCALACK | Discard ENQ and reply with ACK internally; default |
| REMOTEACK | Transmit ENQ and receive ACK from the remote device |
| CAPSLOCK | Upshift lowercase input characters to uppercase; default for port 0 of the first ADCC |
| NOCAPSLOCK | Input characters are unchanged; default |
| UC | Upshift lowercase output characters to uppercase |
| 7B | Output with high-order bit cleared; default |
| 7P | Output with high-order bit cleared, non-printing suppressed; default for port 0 |
| 8B | Output characters are unchanged |
| LOG=<filename> | Enable output logging |
| NOLOG | Disable output logging; default |
| DISCONNECT | Disconnect the port |

Ports that are configured for MPE terminal type 10 expect HP terminals or terminal emulators to be connected and will handshake transfers by sending an ENQ and expecting an ACK in reply.  A device that does not provide ENQ/ACK handshaking will hang during output.  However, if LOCALACK is specified, the handshake will take place locally within the simulator.  The ENQ will be discarded, and a locally generated ACK will be returned to the I/O driver.  In addition to enabling the use of non-HP terminals, this option significantly improves the performance of common HP terminal emulators.  Specifying REMOTEACK will pass ENQ to the terminal for handling; this is necessary to avoid output overruns if a real HP terminal is connected to a serial port.

Some HP 3000 programs, e.g., SLEUTH, require command input in upper case, although mixed-case output is supported.  As an aid to avoid toggling the host keyboard in and out of CAPS LOCK mode, the multiplexer provides this function locally.  The default modes are CAPSLOCK for port 0 of the ADCC on IMB channel 1 (the system console) and NOCAPSLOCK for all other ports.

Each port may be set to one of four output modes: UC, 7B, 7P, or 8B.  The default mode is 7P for channel 1 port 0 and 7B for all other ports.  If the system console is set to MPE terminal type 10, the mode must be changed to 7B or 8B to allow ESC, DC1, and ENQ characters to pass through to the terminal.  Alternatively, the SET CONSOLE PCHAR command may be used to redefine the set of printable characters.  Modes UC and 7P are compatible with MPE terminal types 4 and 6.

File transfer using the Reflection terminal emulator requires an 8-bit data path.  To achieve this, the session must use MPE terminal type 12 (this may be configured either during a system reload or by specifying the TERM=12 parameter when logging on with :HELLO), and the port must be set to 8B, REMOTEACK, and NOCAPSLOCK modes.

Each port supports independent I/O logging to a file.  Adding the –N (new file) switch clears the contents of the log file if it is present.  Without the –N switch, port output will be appended to any preexisting log file content.  Disabling logging also closes the log file.

A port that has been configured in MPE as a serial printer using driver HIOASLP2 must have its log file open before printing is attempted.  If the file is closed, the output will be silently discarded.  Placing the required SET ADCCn LOG=<filename> command in the simulator startup file is recommended.

A port may be manually disconnected from its associated Telnet session with the SET ADCCn DISCONNECT command.  Otherwise, the connection will remain open until disconnected either by the Telnet client or a DETACH ADCC command.  For a serial connection, the SET ADCCn DISCONNECT command will drop and then raise the Data Terminal Ready line; to disconnect the serial port, issue a DETACH ADCCn command.

ADCC device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW ADCC | Display the device and unit configuration |
| SHOW ADCC CHANNEL | Display the IMB channel assignment |
| SHOW ADCC CONNECTIONS | Display the port connections |
| SHOW ADCC STATISTICS | Display the port I/O statistics |
| SHOW ADCC LINEORDER | Display the port connection order |

ADCC unit configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW ADCC<n> | Display the selected port configuration |
| SHOW ADCC<n> LOG | Display the selected port logging status |

### Tracing

The ADCC device supports configuration of the trigger, store, and window specifications.  The trigger sources for the TRACE TRIGGER ADCC command are:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| INPUT | quoted-string | PORT | range | Characters read |
| BREAK | — | PORT | range | Break detected during reception |
| OVERRUN | — | PORT | range | Overrun detected during reception |
| OUTPUT | quoted-string | PORT | range | Characters written |

The INPUT source triggers when the quoted-string characters are received.  The BREAK source triggers when a break condition occurs during reception.  The OVERRUN source triggers when a data overrun condition occurs during reception.  If the PORT qualifier is added, these input trigger conditions must occur on a multiplexer port in the specified range value.  The value is a list of individual port numbers and ranges of port numbers of the form low-high, separated by commas.  For example, PORT=1,4-7 would trigger only if the specified input trigger condition(s) appeared on port numbers 1, 4, 5, 6, or 7.

The OUTPUT source triggers when the quoted-string characters are sent.  If the PORT qualifier is added, the output trigger condition must occur on a multiplexer port in the specified range value.

The syntax of the quoted-string values is described in the Quoted String section of the SIMH Users' Guide Supplement.  Separate input and output port qualifiers are provided, with the input qualifier applying to all three of the input sources.  The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

The storage states that may be configured for the TRACE STORE ADCC command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Interface commands executed |
| CSRW |  | Interface control, status, read, and write actions |
| CSIRQ |  | Interface channel service and interrupt actions |
| SERV |  | Port unit service events |
| PSERV |  | Poll unit service events (periodic) |
| XFER |  | Data receptions and transmissions |
| IMBRQ |  | Intermodule Bus CSRQ and IRQ signal assertions |
| IMBUS |  | Intermodule Bus signals and data words received and returned |

The CMD state traces the commands executed by the interface.  The CSRW state traces control, status, read, and write commands sent to the interface.  The CSIRQ state traces channel service and interrupt requests and responses.  The SERV state traces port unit event service scheduling and entries.  The port service is entered to transmit or receive each character at the configured baud rate.  The PSERV state traces poll unit event service entries.  The poll service is entered once every 10 milliseconds to poll the Telnet and serial ports for connections and input characters to be received by the active ADCC ports.  The XFER state traces the characters received and transmitted by the active ports.  The IMBRQ state traces the Intermodule Bus channel service request and interrupt request signal assertions.  The IMBUS state traces the Intermodule Bus signals and data received and returned via the interface.

Omitting the list of storage states in a TRACE STORE ADCC command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW ADCC command is:

| Source | Value | Source Description |
| --- | --- | --- |
| PORT | range | Port addressing |

The PORT source restricts the storage of all states to those associated with the range of multiplexer ports designated by the range.  The value is a list of individual port numbers and ranges of port numbers of the form low-high, separated by commas.  For example, PORT=1,3,5 would include in the trace list only those states pertaining to port numbers 1, 3, or 5.

The storage window is enabled when a port in the specified range is addressed and is disabled when a different port is addressed.  This source allows detailed tracing of specified port activity while eliminating states for ports that are not of interest.

The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Examples of the trace formats follow:

**ADCC   cmd: Port 7 connection established
**ADCC   cmd: Port 7 connection raised DSR
**ADCC   cmd: Port 7 modifier 6 transmitter is 1200 baud
**ADCC   cmd: Port 7 modifier 0 input character is 'H'
**ADCC   cmd: Port 1 modifier 0 output character is LF
**ADCC   cmd: Port 0 modifier 5 character BS is special
**ADCC   cmd: Port 1 DSJ 2 condition is modem change
**ADCC   cmd: Port 1 modifier 2 interface control is CA (RTS) | CD (DTR)
**ADCC   cmd: Port 3 disconnection dropped DSR

**ADCC  csrw: Register 0 (FIFO) command is Talk 7
**ADCC  csrw: Register 0 (FIFO) command is Listen 30
**ADCC  csrw: Register 0 (FIFO) command is Secondary 06H
**ADCC  csrw: Register 0 (FIFO) data out is 0BH
**ADCC  csrw: Register 0 (FIFO) special data in is 125
**ADCC  csrw: Register 2 (IRQ conditions) status is IRQ | available | empty
**ADCC  csrw: Register C (Interrupt) control is set | device 0

**ADCC csirq: Port 2 asserts CSRQ for DSJ 2 condition modem change

**ADCC csirq: OBII status is channel 2 | device 2

**ADCC csirq: Port 6 asserts CSRQ for DSJ 0 condition RR occupied
**ADCC csirq: OBSI status is device request | channel 2 | device 6

**ADCC  serv: Port 7 delay 1621 service scheduled
**ADCC  serv: Port 7 service entered

**ADCC pserv: Poll delay 3891 service entered
**ADCC pserv: Poll service stopped

**ADCC  xfer: Port 3 break detected
**ADCC  xfer: Port 3 UART received character CR
**ADCC  xfer: Port 3 UART sent character CR
**ADCC  xfer: Port 3 UART sent character LF
**ADCC  xfer: Port 3 character NUL discarded by output filter
**ADCC  xfer: Port 0 character ENQ absorbed internally
**ADCC  xfer: Port 0 UART received character ACK
**ADCC  xfer: Port 1 receiver data overrun

**ADCC imbrq: Channel 2 asserts IRQ
**ADCC imbrq: Channel 2 device 2 interrupted as DEVNO 18

**ADCC imbus: Channel 2 received opcode I/O Write command WIOC register C
              data 000012 with signals ADO | DDO | PRI
**ADCC imbus: Channel 2 returned data 000000 with signals ADN | DDN | IRQ | PRO
**ADCC imbus: Channel 2 asserts IRQ
**ADCC imbus: Channel 2 received opcode I/O Read command IPOLL register 0
              data 000000 with signals ADO | DDO | PRI
**ADCC imbus: Channel 2 returned data 020000 with signals ADN | DDN | IRQ | PRO

### Registers

The ADCC state contains these registers:

| Name | Size | Radix | Symbolic | Description |
| --- | --- | --- | --- | --- |
| ADDRGRP | 3 | 10 |  | HP-IB addressing group |
| PRIADDR | 5 | 10 |  | HP-IB primary address |
| SECADDR | 5 | 10 |  | HP-IB secondary address |
| IRQMASK | 16 | 8 |  | Interrupt mask channel bit |
| IRQ | 8 | 2 |  | Interrupt request port bits |
| CSRQ | 16 | 2 |  | Channel service request port bits |
| CPRUN | 8 | 2 |  | Channel program running bits |
| FTIME | 24 | 10 |  | Fast transmit/receive time |
| THR [0:7] | 8 | 8 |  | Transmitter holding register, ports 0-7 |
| TR [0:7] | 8 | 8 |  | Transmitter register, ports 0-7 |
| RHR [0:7] | 8 | 8 |  | Receiver holding register, ports 0-7 |
| RR [0:7] | 8 | 8 |  | Receiver register, ports 0-7 |
| DSJ [0:7] | 2 | 10 |  | Device Specified Jump response, ports 0-7 |
| MCONTROL [0:7] | 5 | 2 |  | Modem control lines, ports 0-7 |
| MSTATUS [0:7] | 5 | 2 |  | Modem status lines, ports 0-7 |
| SVMASK [0:7] | 3 | 2 |  | Service conditions mask, ports 0-7 |
| STMASK [0:7] | 5 | 2 |  | Modem status mask, ports 0-7 |
| STREF [0:7] | 5 | 2 |  | Modem status reference, ports 0-7 |

The ADDRGRP register value is 1 for a Listen Address Group command and 2 for a Talk Address Group command.  The PRIADDR value selects the port number to address, and the SECADDR value selects the operation to perform.

The THR, TR, RHR, and RR registers default to single-character display but may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

The MCONTROL register value displays five binary values corresponding to the input character echo state, plus the CA (RTS), CD (DTR), CH (DRS), and SCA (SRTS) modem control lines, respectively.

The MSTATUS, STMASK, and STREF register values each display five binary values corresponding to the CB (CTS), CC (DSR), CE (RI), CF (DCD), and SCF (SDCD) modem status lines, respectively.

## CS/80 Disc and Tape Drives

The HP 31262A General I/O Channel connects from one to eight Command Set 80 disc or tape units to the HP 3000.  Available models include disc drives ranging in capacity from 28MB to 574 MB and cartridge tape drives having 16 MB, 67 MB, or 134 MB capacity.  In hardware, up to four CS/80 drives can be connected to a single GIC; bus loading and the 1 MB/second data transfer rate impose the limitation.  This restriction does not exist under simulation.

Certain CS/80 disc models were offered with integrated cartridge tape drives.  These were typically configured as a unified controller occupying a single HP-IB address, with the disc and tape drives responding at that address as CS/80 unit numbers 0 and 1, respectively.  However, these combination models were supported by MPE only when the tape drive had its own dedicated controller.  The HP 7911, 7912, and 7914 models could be ordered with Option 001 to provide a second, independent tape controller that used a second HP-IB address, with the drive as unit 0.  The HP 7942 and 7946 models could not, meaning that they could not be used under MPE.

The CS/80 simulator does not support separate controllers for the combination drives.  Therefore, the HP 7911, 7912, and 7914 models are offered with Option 140, which deletes the integrated tape drive, and the HP 7942 and 7946 are replaced by the tape-less 7941 and 7945 models.  The combination models can be effectively simulated by including a standalone HP 9144 cartridge tape drive with each associated tape-less disc model.

Note that under MPE V/R, only the HP 7911, 7912, 7933, and 7935 disc models are supported.  These models are also supported under MPE V/P and V/E.  Later models, including the HP 9144 cartridge tape drive, are supported only by specific MPE V/E releases; see the HP Peripheral Configuration Guide (part number 5954-9306, February 1988), available from the HP Computer Museum, for details.

Attaching a disc or tape image file to a unit simulates installing a removable HP 7935 disc pack, loading the heads on a fixed disc drive, or inserting a tape cartridge into a tape drive:

ATTACH [{-E | -N}] [{-R | -X}] [-S] DCn <image-filename>

Specifying the –E (exist) switch ensures that the specified file exists; if it does not, File open error is printed.  Specifying the –N (new file) switch creates a new, blank, full-size image file and initializes the media.  If an existing file is specified with –N, it is reinitialized.  For tape images, initialization includes certification.  Omitting both switches will open the file if it exists or create the file if it does not; a created file will have zero size and must be initialized before use with the MPE VINIT subsystem FORMAT command.

For cartridge tape units, the –E and –N switches behave as above.  When creating a new file for a 9144A drive, including the –S (short-length) switch specifies a 16 MB, 150-foot cartridge tape image; omitting it specifies a long-length 67 MB, 600-foot cartridge.  For a 9145A drive, the respective sizes are 32 MB and 134 MB.  When attaching an existing file, the file size automatically determines the cartridge size; including –S verifies that the file being attached is a short cartridge image.

Specifying the –R (read-only) switch is equivalent to setting the cartridge's Write Protect switch to the Safe position.  –R is only valid with an existing file; if it is specified with a nonexistent file, File open error is printed.

Tape cartridges must be initialized and certified before they can be used.  Existing tape images are checked for initialization and certification when they are attached.  A certified image must contain only data records and file marks.  An initialized image must contain only erased blocks, data records, and file marks.  To be valid, each record of the image file must be properly formatted.  If the image fails the check, Volume is not initialized or Volume is not certified is printed; the file remains attached, but it must be initialized or certified before use.

For unrestricted use, a tape cartridge image file must contain the full number of formatted blocks defined by the cartridge size.  Some conversion programs produce properly formatted images containing less than the required number of tape blocks.  Such valid but incomplete files may be attached and will be marked as initialized and certified, but they are restricted to read-only access; Volume is write protected is printed on the console as a warning.  Incomplete files appear as short or long cartridges, depending on the number of blocks contained in the file.  Programs attempting to read blocks that are not physically present in the file will receive No Data Found errors.

The user may optionally extend such incomplete files to their full complement of tape blocks by specifying the –X (extend) switch.  This appends and certifies the remaining blocks in the image file.  Files are extended to the long cartridge size unless the –S switch is added and the number of blocks in the incomplete file is less than the defined short-cartridge count.  After extending, file use is unrestricted.  –X may also be used to extend a full-size short-length cartridge image to the long-length cartridge image size.  –X is ignored if the image is not initialized and certified; it only extends valid images.

To summarize, the switch combinations perform the following actions for disc and tape images, respectively:

| Switch | Disc Action |
| --- | --- |
| (none) | Open an existing disc file or create a new disc file |
| –E | Open an existing disc file |
| –N | Create and initialize a new disc file |

| Switch | Tape Action |
| --- | --- |
| (none) | Open an existing tape file or create a new long tape file |
| –S | Open an existing short tape file or create a new short tape file |
| –E | Open an existing tape file |
| –E –S | Open an existing short tape file |
| –N | Create and certify a new long tape file |
| –N –S | Create and certify a new short tape file |
| –R | Open and write protect an existing tape file |
| –R –S | Open and write protect an existing short tape file |
| –X | Open an existing tape file and extend it to long size if needed |
| –X –S | Open an existing tape file and extend it to short size if needed |

The –R, –S, and –X switches are used only with tape images and are rejected if specified for a disc image.  Specifying –S with an existing file that is larger than the short cartridge size is also rejected.  The –N and –R switches are mutually exclusive, as are the –X and –R switches.  All of these combinations result in Invalid switch errors.

At simulator startup, all units return Power Fail status.  If needed, this status may be cleared manually by issuing a RESET –C DC command to issue an HP-IB Device Clear to all enabled units.  An unattached unit also returns Not Ready status.

Attached media image files may be viewed or changed with an EXAMINE or DEPOSIT command that specifies a unit name and a decimal linear address.  These commands display or enter values as 8-bit bytes by default.  However, the –C, –M, and –W command-line switches may be included to interpret pairs of successive bytes as 16-bit words, as described the Symbolic Display and Entry section above.

Detaching the disc image file simulates unloading the heads and, for the HP 7935, removing the disc pack:

DETACH [-F] DCn

Detaching does not occur immediately if the CPU is running.  Instead, a Release Request is sent to the MPE operating system, and the simulator prints, e.g., Bus 0 unit 0 release requested on the simulation console.  If the operating system grants the request, the file is detached, and Bus 0 unit 0 released and detached is printed.  If the request is denied by the operating system, Bus 0 unit 0 release denied is printed, and the image file remains attached to the unit.

Specifying the –F (force) switch will detach the file immediately without notifying the operating system.  This switch should be used with care, as forcing a disc image change while MPE is running will result in disc corruption.  A forced detach is performed automatically if the DETACH DCn command is entered while the CPU is halted.

If the host operating system returns an error when seeking, reading, or writing a disc or tape image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator disc I/O error: No space left on device

A failed seek will report Unit Fault status.  A failed read or write will report Unrecoverable Data status.  The target operating system will then react to this error as though the drive had encountered a bad disc or tape block.

### Configuration

Device and unit options include configuring the timing, bus address, drive type, and the ability to add and remove drives from the bus.  The command forms are:

SET DC <device-option>
SET DCn <unit-option>

A note on terminology: both CS/80 and SIMH use the terms device and unit.  In hardware, a CS/80 device corresponds to a physical disc or tape drive identified by its bus address, and a CS/80 unit corresponds to the device controller or a separate storage implementation within a device identified by its unit number.  For example, the HP 7933A device contains a hard disc drive at unit 0 and the disc controller at unit 15.

A SIMH device refers to the set of connected drives, and a SIMH unit refers to one storage implementation connected to a media image file on the host platform.  For this DC device, CS/80 units and SIMH units correspond one-for-one, although the assignment of CS/80 units to SIMH units is arbitrary and determined by the unique combination of bus address and CS/80 unit number.  In the remainder of this section, the terms device and unit have their SIMH meanings unless the term CS/80 is specified explicitly.

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| COUNT=<n> | Set the number of DC instances; default is 1 |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| CHANNEL=<n> | Set the channel to which the drives are connected; default is 11 |
| ENABLED | Enable the device; default for IMB systems |
| DISABLED | Disable the device; default for IOP systems |

The COUNT option determines the number of separate instances of the DC drive set.  If additional GIC instances are created, then each GIC must have a corresponding set of connected drives.  The total number of drive sets may be specified with the COUNT option.  For example, issuing the SET DC COUNT=2 command will create an additional DC instance that may be connected to the second GIC by assigning the same channel number to both.  Issuing SET DC COUNT=1 will remove all instances except the original one.  When more than one DC instance exists, the original DC is renamed DCA, while the additional instances are named DCB, DCC, etc.  The original DC will return to its original name if the count is reduced to 1.

When realistic timing is enabled, the simulation accurately models the disc and tape movement times (in machine instructions).  For example, seeking takes longer if the positioner movement distance is farther.  Optimized timing reduces the timing to the minimum necessary to operate correctly; this is much faster than a real drive would operate.

The delay times used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the abnormally rapid operation.  Entering a power-on reset command, RESET –P DC, restores the original optimized times.

The CHANNEL option connects the DC drive set to the GIC with the specified IMB channel address from 2-15.  The address determines the specific GIC to which the drives will respond.  More than one DC instance can be connected to the same GIC, as long as the bus addresses assigned to the devices are unique.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW DC | Display the device and unit configuration |
| SHOW DC CHANNEL | Display the channel address |

### Unit Options

Unit options that may be specified for individual CS/80 drives are:

| Option | Action |
| --- | --- |
| 7911A | Set the drive type to 7911A with Option 140 |
| 7912A | Set the drive type to 7912A with Option 140 |
| 7914A | Set the drive type to 7914A with Option 140 |
| 7933A | Set the drive type to 7933A; default |
| 7935A | Set the drive type to 7935A |
| 7936H | Set the drive type to 7936H |
| 7937H | Set the drive type to 7937H |
| 7945A | Set the drive type to 7945A |
| 7957B | Set the drive type to 7957B |
| 7958B | Set the drive type to 7958B |
| 7959B | Set the drive type to 7959B |
| 9144A | Set the drive type to 9144A |
| 9145A | Set the drive type to 9145A |
| BUS=<n> | Set the drive’s bus address; default is the unit number |
| ENABLED | Enable the unit; default for unit 0 |
| DISABLED | Disable the unit; default for units 1-7 |

The 7911A, 7912A, 7914A, 7933A, 7935A, 7936H, 7937H, 7945A, 7957B, 7958B, and 7959B options select the disc drive model.  The 9144A and 9145A options select standalone cartridge tape drive models.  Option 140 deletes the integrated cartridge tape drive from the HP 7911A, 7912A, and 7914A.

The BUS option sets the HP-IB address of the drive.  Bus addresses range from 0-7, and each address must be unique before program execution begins.  For example, assuming that both DC0 and DC1 are enabled, a 7912A and 7933A might be configured as follows:

sim> SET DC0 7912A,BUS=3
sim> SET DC1 7933A,BUS=5

The assigned DC<n> unit numbers need not be successive, nor must the CS/80 bus addresses be ascending; any enabled SIMH unit can be assigned to any CS/80 device address.

All units but the first are initially disabled.  A disabled unit is removed from the bus and does not communicate with the interface. Enabled drives that are unattached still communicate with the GIC and continue to receive bus transactions.  It is therefore more efficient to disable all unused units.  Note that a unit may be enabled within the configuration command, e.g., SET DC1 ENABLED,7912A,BUS=3.

Unit configuration may be displayed with the following command:

| Command | Action |
| --- | --- |
| SHOW DC<n> | Display the selected unit’s configuration |

In response to a START, LOAD, DUMP, BOOT, RUN, GO, CONTINUE, or STEP command to begin instruction execution, a check is made to ensure that GIC channel address and CS/80 bus address and model number conflicts are not present.  If the check detects an inconsistent configuration, the simulator will report the error to the simulation console, and execution will be inhibited.  For example:

sim> SHOW DC
DC, fast timing, channel=11, 8 units
  DC0, 65MB, not attached, 7912A, bus=0, unit=0
  DC1, 28MB, not attached, 7911A, bus=0, unit=0
sim> RUN
DC bus device 0 has multiple assignments for unit 0

Simulation stopped, P: 000000 (NOP,NOP)
sim>

The simulator will not begin program execution until all conflicts are resolved.

### BOOT Command

For IMB systems, the BOOT command may be used as an alternative to START or LOAD to initiate the system cold load procedure for the specified unit.  The BOOT DC<n> command is the simulation equivalent to setting the START thumbwheel switches to the appropriate channel and device numbers and then pressing the START system control panel button to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Channel |  |  |  | Device |  |  |

Cold loading a system from the disc or cartridge tape is supported.

For Starfish systems, the BOOT command may be used an alternative to LOAD.  The BOOT DC<n> command is equivalent in hardware to setting the SWCH register to configure the device numbers, and pressing the LOAD and ENABLE buttons to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC device number |  |  |  |  |  |  |  | IMBA device number |  |  |  |  |  |  |  |

The upper byte defaults to the DRT number of the specified disc unit, and the lower byte defaults to the DRT number of the IMB Adapter.

### Tracing

The DC device supports configuration of the trigger, store, and window specifications.  The trigger source for the TRACE TRIGGER DC command is:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| COMMAND | opcode | BUS | range | CS/80 command execution |
|  |  | BLOCK | block |  |

The COMMAND source triggers when the CS/80 command designated by the opcode executes.  The opcode is entered as a numeric value in the device data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

If the BUS qualifier is added, the command must be executed by a device at the bus address specified in the range value.  The value is a list of individual bus addresses separated by commas and ranges of the form low-high.  For example, BUS=0-2,7 would trigger only the command was executed by a device at bus addresses 0, 1, 2, or 7.

The BLOCK qualifier is allowed only if the opcode is 0 (Locate and Read) or 2 (Locate and Write).  If it is added, the command must read or write the media block number specified by the block value.

Bus range and block values are entered in decimal unless overridden by a switch on the command line.

Triggering occurs only if the condition and all of the specified qualifiers are satisfied.

The storage states that may be configured for the TRACE STORE DC command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Controller commands executed |
| INCO |  | Controller command initiations and completions |
| STATE |  | Controller state changes executed |
| SERV |  | Controller and drive unit service events |

The CMD state traces the commands executed by the drives.  The INCO state traces the beginning and ending of commands, including command termination status.  The STATE state traces the changes between the internal controller execution states.  The SERV state traces controller and drive service scheduling and entries.

Omitting the list of storage states in a TRACE STORE DC command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW DC command is:

| Source | Value | Source Description |
| --- | --- | --- |
| BUS | range | Device addressing |

The BUS source restricts the storage of all states to those associated with the range of bus addresses designated by the range.  The value is a list of individual addresses separated by commas and ranges of the form low-high.  For example, BUS=3,5-7 would include in the trace list only those states pertaining to devices at bus addresses 3, 5, 6, and 7.

The storage window is enabled when a bus device in the specified range is addressed and is disabled when a different device is addressed.  This source allows detailed tracing of specified device addresses while eliminating states for devices that are not of interest.

The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Examples of the trace formats follow:

**DC   cmd: Device 0 Set Unit 0 executed
**DC   cmd: Device 0 Set Length 65024 executed
**DC   cmd: Device 0 Set Address 2 executed
**DC   cmd: Device 0 Locate and Read executing

**DC  inco: Device 0 unit 0 Locate and Read started
**DC  inco: Device 0 unit 0 read block 1
**DC  inco: Device 0 unit 0 Locate and Read completed with qstat 0

**DC state: Device 0 transitioned from Reporting Wait state to Command Wait state
**DC state: Device 0 parallel poll response disabled
**DC state: Device 0 transitioned from Command Wait state to Execution Wait state
**DC state: Device 0 transitioned from Execution Wait state to Read state
**DC state: Device 0 parallel poll response enabled
**DC state: Device 0 transitioned from Read state to Reporting Wait state

**DC  serv: Device 0 state Execution Wait positioning delay 16 service scheduled
**DC  serv: Device 0 state Execution Send data delay 1 service scheduled
**DC  serv: Device 0 unit 0 state Reporting Wait service entered

### Registers

The CS/80 device state contains these registers:

| Name | Size | Radix | Read-Only | Description |
| --- | --- | --- | --- | --- |
| PP | 1 | 2 |  | Parallel poll is active |
| PPR | 8 | 2 |  | Set of parallel poll responses |
| OPCODE [0:7] | 6 | 10 |  | Controller opcode, devices 0-7 |
| UNIT [0:7] | 4 | 10 |  | Current unit selected, devices 0-7 |
| CSTATE [0:7] | 6 | 10 |  | Controller state, devices 0-7 |
| CTIME | 24 | 10 |  | Fast controller overhead time |
| STIME | 24 | 10 |  | Fast controller per-byte status transfer time |
| KTIME | 24 | 10 |  | Fast unit one-step seek time |
| TTIME | 24 | 10 |  | Fast unit additional step traverse time |
| PTIME | 24 | 10 |  | Fast unit on-track positioning time |
| BTIME | 24 | 10 |  | Fast unit per-block traverse time |
| DTIME | 24 | 10 |  | Fast unit per-byte data transfer time |
| RTIME | 24 | 10 |  | Fast unit block residue traverse time |

## 7970E HP-IB Tape Controller with four Magnetic Tape Drives

The HP-IB interface for the HP 7970E 1600-bpi PE ½-inch magnetic tape drive connects one to four drives to the HP 31262A General I/O Channel.  This is the required connection method when using the drive with IMB-based HP 3000 computer systems.  The interface installs in the chassis of the master drive and replaces the HP 30215 tape controller and parallel interface that is used with IOP-based 3000s such as the Series II and III.  The master drive could be ordered with an HP-IB interface by specifying Option 436, or a parallel-interface drive could be retrofitted with an HP-IB interface by ordering HP part number 26072A.

Designed originally for connection to the HP 300 ("Amigo") computer system, the command set was expanded to include several related to HP-IB operation. They include the Amigo Identify and Amigo Clear sequences, Read and Write Loopback channel tests, and Device Specified Jump commands.  The interface also includes a 128-byte FIFO buffer between the drives and the bus to enable more efficient use of the GIC to which it is attached.  The interface occupies a single bus address, with each drive having a user-selectable unit number from 0-3.

The simulation provides up to four tape drives.  Attaching a tape image file to a unit simulates mounting a tape reel on a drive:

ATTACH [-R] [-F] MAn [<format>] <image-filename>

Adding the –R (read-only) switch is equivalent to mounting the tape without a write ring in place.  Adding the –F switch and a format identifier declares the tape image format to be used.  Supported formats are SIMH, E11, TPC, and P7B.  If the –F switch and a format identifier are not supplied, then SIMH tape image format is used.  Note that erase gaps embedded in the tape image file are supported only in SIMH image format mode.

Attached tape image files may be viewed or changed with an EXAMINE or DEPOSIT command that specifies a unit name and a decimal linear address.  These commands display or enter values as 8-bit bytes by default.  However, the –C, –M, and –W command-line switches may be included to interpret pairs of successive bytes as 16-bit words, as described in the Symbolic Display and Entry section above.

If the host operating system returns an error when reading or writing a tape image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator tape library I/O error: No space left on device

If this or another tape library error occurs, e.g., due to an illegal or damaged tape format, the simulator stops with an appropriate error message.  Resuming the simulation will fail the tape operation with Multiple Track Error status.  The target operating system will then react to this error as though a real drive had encountered a bad tape record.

A drive’s unit number is not set explicitly.  Instead, the drive unit number is derived from the simulation unit number.  For example, unit MA0 responds to tape unit select number 0.  Pressing a different unit select button on a mounted drive is equivalent to detaching and reattaching the tape image file to the corresponding simulation unit.  Pressing the OFF button is equivalent to setting the drive offline.

Device and unit options include configuring the timing, bus address, tape reel size, tape image format, and the ability to set drives offline or online.  The command forms are:

SET MA <device-option>
SET Man <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| COUNT=<n> | Set the number of MA instances; default is 1 |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| CHANNEL=<n> | Set the channel to which the drives are connected; default is 9 |
| BUS=<n> | Set the bus address to which the drives are connected; default is 0 |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

The COUNT option determines the number of separate instances of the MA drive set, with each instance supporting four drive units.  If additional MA instances are desired, the total number of drive sets may be specified with the COUNT option.  For example, issuing the SET MA COUNT=3 command will create two additional MA instances, while issuing SET MA COUNT=1 will remove all instances except the original one.  When more than one MA instance exists, the original MA is renamed MAA, while the additional instances are named MAB, MAC, etc.  The original MA will return to its original name if the count is reduced to 1.

When realistic timing is enabled, the simulation accurately models the tape movement times (in machine instructions).  For example, rewinding takes longer if the tape is positioned farther from the load point.  Realistic timing is necessary to pass the magnetic tape diagnostic timing tests.

Optimized timing reduces the timing to the minimum necessary to operate correctly; this is much faster than a real tape drive would operate.  In addition, enabling optimized timing also omits the initial erase gap normally written after the BOT marker.  This does not affect gaps written with the Write Gap command.

The delay times used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the unusually rapid tape operation.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized times.

The CHANNEL option connects the MA drive set to the GIC with the specified IMB channel address from 2-15.  The address determines the specific GIC to which the drives will respond.  The BUS option sets the HP-IB address of the drive set controller.  Bus addresses range from 0-7, and each address must be unique before program execution begins.  More than one MA instance can be connected to the same GIC, as long as the bus addresses assigned to the drive sets are unique.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW MA | Display the device and unit configuration |
| SHOW MA CHANNEL | Display the channel assignment |
| SHOW MA BUS | Display the bus address assignment |

### Unit Options

Unit options that may be specified for individual tape drives are:

| Option | Action |
| --- | --- |
| REEL=<n> | Set the reel size in feet; default is unlimited |
| CAPACITY=<n> | Set the reel capacity in megabytes; default is unlimited |
| OFFLINE | Set the unit offline; default when detached |
| ONLINE | Set the unit online; default when attached |
| FORMAT=<fmt> | Set the tape image format; default is SIMH format |
| ENABLED | Enable the unit; default |
| DISABLED | Disable the unit |

The reel size may be set to 600-, 1200-, or 2400-foot capacity.  Setting the capacity or reel size to 0 specifies unlimited capacity; in this configuration, the controller never returns an end-of-tape indication.

The OFFLINE and ONLINE options place a drive offline and online, respectively.  The former provides a convenient method of setting a drive "down" without detaching the associated tape image file.

The default tape image format for future ATTACH commands may be set to one of the format identifiers listed previously.  The unit must be detached when the format is set.  In addition, the tape image format may be specified temporarily in each ATTACH command.

Drive configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW MA<n> | Display the selected drive’s configuration |
| SHOW MA<n> REEL | Display the selected drive’s reel size or capacity |
| SHOW MA<n> CAPACITY | Display the selected drive’s reel size or capacity |
| SHOW MA<n> FORMAT | Display the selected drive’s tape image format |

### BOOT Command

For IMB systems, the BOOT command may be used as an alternative to LOAD to initiate the system cold load procedure for the tape drive.  The BOOT MA0 command is the simulation equivalent to setting the LOAD thumbwheel switches to the appropriate channel and device numbers and then pressing the LOAD system control panel button to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Channel |  |  |  | Device |  |  |

For Starfish systems, the BOOT command may be used an alternative to LOAD.  The BOOT MA0 command is equivalent in hardware to setting the SWCH register to configure the device numbers, and pressing the LOAD and ENABLE buttons to begin the cold load process for unit 0.  The SWCH register is set as follows:

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MA device number |  |  |  |  |  |  |  | IMBA device number |  |  |  |  |  |  |  |

The upper byte defaults to the DRT number of the specified disc unit, and the lower byte defaults to the DRT number of the IMB Adapter.

The system microcode supports cold loading only from unit 0.  Attempting to boot from another unit will result in a Command not allowed error.

### Tracing

The MA device supports configuration of the trigger, store, and window specifications.  The trigger sources for the TRACE TRIGGER MA command are:

| Source | Value | Qualifier | Value | Source Description |
| --- | --- | --- | --- | --- |
| TAPE | opcode | UNIT | range | Tape command execution |
| INTERFACE | opcode | PARAMETER | value | Bus command execution |
|  |  | UNIT | range |  |

The TAPE source triggers when the tape command designated by the opcode executes.  The INTERFACE source triggers when the interface command designated by the opcode executes.  If the PARAMETER qualifier is added, the interface command must be issued with the parameter value specified.  If the UNIT qualifier is added, the interface command must be executed for a device at the unit address specified in the range value.  The value is a list of individual unit numbers separated by commas and ranges of the form low-high.  For example, UNIT=0,2-3 would trigger only the command was executed by a device at unit address 0, 2, or 3.

Interface and tape opcodes and parameter values are entered as numeric values in the device data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Triggering occurs only if the condition and all of the specified qualifiers are satisfied.

The valid tape and interface command opcodes are:

| Opcode | Tape Command Description | Interface Command Description | Qualified |
| --- | --- | --- | --- |
| 1 | Select Unit 0 | Write Data |  |
| 2 | Select Unit 1 | Tape Command |  |
| 3 | Select Unit 2 | End | — |
| 4 | Select Unit 3 | Amigo Clear | — |
| 5 | Write Record | Write Loopback | — |
| 6 | Write File Mark | Read Data |  |
| 7 | Write Gap | Read Status |  |
| 10 | Read Record | Read Byte count |  |
| 11 | Forward Space Record | Device Specified Jump |  |
| 12 | Backspace Record | Read Loopback | — |
| 13 | Forward Space File | Amigo Identify | — |
| 14 | Backspace File | — |  |
| 15 | Rewind | — |  |
| 16 | Rewind Offline | — |  |
| 17 | Read Record Backward | — |  |

Certain interface commands cannot be qualified by the UNIT range because they affect only the bus controller.  If an unqualified interface command is specified with a UNIT qualifier, an Invalid argument error is displayed.

The PARAMETER qualifier may be specified only for interface opcodes Tape Command (2) and End (3); adding the qualifier to any other interface opcode will result in an Invalid argument error.  For Tape Command, the parameter value must be one of the tape command opcodes.  For End, the parameter value must be the logical OR of one or more of these values:

| Value | End Operation Description |
| --- | --- |
| 1 | Clear Poll Response |
| 2 | Inhibit Poll Response for Read Bursts |
| 4 | Enable Rewind Completion Response or Auto-Select |
| 20 | Clear DSJ |
| 40 | Abort Current Tape Operation |

For example, specifying INTERFACE=3;PARAMETER=21 will trigger only when an End command specifying both Clear Poll Response and Clear DSJ operations is executed.  Specifying INTERFACE=3 without a qualifier will trigger when any End command is executed..

The storage states that may be configured for the TRACE STORE MA command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Controller commands executed |
| INCO |  | Controller command initiations and completions |
| XFER |  | Data reads and writes |
| STATE |  | Controller command state changes executed |
| SERV |  | Tape unit service events |
| IOBUS |  | Tape controller bus signals and data words received and returned |

The CMD state traces the commands executed by the tape controller.  The INCO state traces the beginning and ending of commands, including the starting and ending tape position.  The XFER state traces the data bytes read from or written to the tape.  The STATE state traces the entries into each of the internal controller execution states.  The SERV state traces tape unit event service scheduling and entries.  The IOBUS state traces the signals and data received and returned via the data, flag, and function buses between the bus interface and the tape controller.

Omitting the list of storage states in a TRACE STORE MA command uses the indicated set of implicit states.

The window source that may be configured for the TRACE WINDOW MA command is:

| Source | Value | Source Description |
| --- | --- | --- |
| UNIT | range | Unit addressing |

The UNIT source restricts storage to those states associated with a device at the unit address specified in the range value.  The value is a list of individual unit numbers separated by commas and ranges of the form low-high.  For example, UNIT=0,3 would include in the trace list only those states pertaining to a device at unit address 0 or 3.

The storage window is enabled when a unit number in the specified range is addressed and is disabled when a different unit is addressed.  This source allows detailed tracing of specified unit activity while eliminating states for units that are not of interest.

Some states, such as the IOBUS states and certain CMD, STATE, and SERV states, are not associated with any tape unit.  Storage of these states is not inhibited by the window setting.

The radix for entering range values defaults to decimal but may be overridden by a switch on the command line.  See the Numeric Display and Entry section for details.

Examples of the trace formats follow:

**MA    cmd: Device 1 Tape Command Write Record executing
**MA    cmd: Device 1 Device Specified Jump 0 executing
**MA    cmd: Device 1 Write Data executing
**MA    cmd: Tape unit 0 Write Record of 200-byte record succeeded

**MA   inco: Device 1 Read Status command started
**MA   inco: Device 1 status is load point | online
**MA   inco: Device 1 Read Status command completed with DSJ 0
**MA   inco: Tape unit 3 Write Record started at position 0
**MA   inco: Tape unit 3 write gap call of 2.9-inch erase gap succeeded
**MA   inco: Tape unit 3 write call of 100-word record succeeded
**MA   inco: Tape unit 3 Write Record completed at position 5008

**MA   xfer: Tape controller Write Record FIFO 90 data 141 byte 99 loaded
**MA   xfer: Tape unit 3 Write Record FIFO 93 data 010 byte 10 unloaded

**MA  state: Device 1 transitioned from Command Ready state to Command Send state
**MA  state: Tape unit 3 Write Record start phase event entry
**MA  state: Device 1 parallel poll response enabled

**MA   serv: Tape unit 1 Read Record data phase delay 5 service scheduled
**MA   serv: Device 1 Tape unit 1 service entered
**MA   serv: Device 1 Bus controller state Command Send service entered




**MA  iobus: Tape controller idle state received data 000007 flags CMXEQ | INTOK 
**MA  i0bus: Tape controller busy state returned data 000001 functions IFGTC

### Registers

The MA device state contains these registers:

| Name | Size | Radix | Read-Only | Description |
| --- | --- | --- | --- | --- |
| DEVS | 8 | 2 |  | Bus address bitmap of devices present on the bus |
| XFER | 1 | 2 |  | Tape data transfer across channel is active |
| PP | 1 | 2 |  | Parallel poll is in progress |
| PPR | 8 | 2 |  | Bus address bitmap of parallel poll responses |
| ISTATE | 4 | 10 |  | Interface state |
| ICMD | 4 | 10 |  | Interface command |
| COUNT | 16 | 10 |  | Count of bytes read or written |
| DSJ | 2 | 10 |  | Device Specified Jump result |
| CSTATE | 4 | 10 |  | Controller execution state |
| STATUS | 16 | 8 |  | Controller status |
| USEL | 4 | 10 |  | Unit number currently selected |
| UATTN | 4 | 2 |  | Bitmap of units requesting attention |
| RECBUF [0:65537] | 8 | 8 |  | Record buffer |
| LIBSTA | 16 | 10 |  | Status from last tape support library call |
| LENGTH | 24 | 10 |  | Data buffer valid length |
| INDEX | 24 | 10 |  | Data buffer current index |
| GAPLEN | 32 | 10 |  | Length of erase gap preceding the current record |
| INPOS | 32 | 10 |  | Initial tape position |
| RSTART | 24 | 10 |  | Fast rewind start time |
| RRATE | 24 | 10 |  | Fast rewind rate |
| RSTOP | 24 | 10 |  | Fast rewind stop time |
| BTIME | 24 | 10 |  | Fast start time if positioned at the BOT |
| ITIME | 24 | 10 |  | Fast start time if positioned at an inter-record gap |
| DTIME | 24 | 10 |  | Fast data transfer time per byte |
| OTIME | 24 | 10 |  | Fast controller overhead time |
| UPROP [0:3] | 16 | 8 |  | Drive properties, drives 0-3 |
| USTATUS [0:3] | 16 | 2 |  | Unit status, drives 0-3 |
| UOPCODE [0:3] | 6 | 10 |  | Current operation code, drives 0-3 |
| USTATE [0:3] | 4 | 10 |  | Current command state, drives 0-3 |
| UPOS [0:3] | 32 | 10 |  | Current byte position, drives 0-3 |

The RECBUF register may be examined or deposited using any of the modes described in the Symbolic Display and Entry section above.

## 26069A HP-IB to Differential Translator with One 2611/13/17/18/19 Line Printer

The 26069A HPIB-to-Differential Translator allows the HP 2611, 2613, 2617, 2618, and 2619 line printers with differential interfaces to be connected to the HP 3000 General I/O Channel (GIC).  The interface translates the HP-IB signals supplied by the GIC to the parallel differential signals required by the printers.  Each translator supports a single line printer.  Up to four translators are supported on the Series 58.

Designed originally for connection to the HP 300 ("Amigo") computer system, the line printer command set includes several related to HP-IB operation. They include the Amigo Identify and Amigo Clear sequences, Read and Write Loopback channel tests, and Device Specified Jump commands.  The translator also includes eight 128-byte buffers to hold print data received from the bus to enable more efficient use of the GIC to which it is attached.  The interface occupies a single bus address.

The simulation provides one printer unit.  The 2617 is selected by default.  Attaching a text file to the unit simulates loading paper into the printer:

ATTACH [-N] LPA <image-filename>

Adding the –N (new file) switch clears the contents of the image file if present.  Without the –N switch, printer output will be appended to any preexisting image file content.

The printer unit may also be attached to a FIFO (pipe) file to direct printer output to a receiving program.  The receiving end of the file must be connected before the ATTACH is given to avoid erroneous console operation.  The –N switch is not relevant when a FIFO file is attached.

Printer output written to the image file is typically buffered by the host operating system’s underlying stream I/O routines.  While it is running, the simulator flushes the file after each printer top-of-form request to permit convenient inspection of the image file.  Stopping the simulator also flushes the file.

If the host operating system returns an error when writing to the printer image file, the simulator will report the error to the simulation console, e.g.:

HP 3000 simulator printer I/O error: No space left on device

The printer goes offline with an alarm condition, and the simulator stops.  Simulation may then be resumed, either with the printer set back online if the problem is fixed, or with the printer remaining offline if the problem is uncorrectable.

Detaching the text file from the unit with the DETACH LPA command simulates running out of paper.  If the command is entered while there are characters in the print buffer and the print location is not at the top of the form, Command not completed is displayed on the simulation console, and the file remains attached until the required conditions are true.  Once simulation is resumed and the print operations complete, the printer is set offline and detached automatically.

Detaching may also be forced with the DETACH –F LPA command.  This simulates physically removing the paper and takes effect immediately, regardless of any printing operations in progress.  A forced detach is performed automatically if the DETACH LPA command is entered while the CPU is halted.

Device and unit options include configuring the printer type and timing, output format, vertical format unit (VFU), and the ability to set the printer offline or online.  The command forms are:

SET LPA <device-option>
SET LPA <unit-option>

### Device Options

Device options that may be specified are:

| Option | Action |
| --- | --- |
| COUNT=<n> | Set the number of LPA instances; default is 1 |
| FASTTIME | Use optimized timing; default |
| REALTIME | Use realistic timing |
| CHANNEL=<n> | Set the channel to which the translator is connected; default is 11 |
| BUS=<n> | Set the bus address to which the translator is connected; default is 4 |
| ENABLED | Enable the device; default |
| DISABLED | Disable the device |

The COUNT option determines the number of separate instances of the LPA translator, where each instance controls one line printer.  Up to four instances may be created.  If additional LPA instances are desired, the total number may be specified with the COUNT option.  For example, issuing the SET LPA COUNT=2 command will create one additional LPA instance that may be connected to a GIC by assigning the same channel number to both.  Issuing SET LPA COUNT=1 will remove all instances except the original one.  When more than one LPA instance exists, the original LPA is renamed LPAA, while the additional instances are named LPAB, LPAC, etc.  The original LPA will return to its original name if the count is reduced to 1.

The printer supports realistic and optimized timing modes.  Realistic timing attempts to model the print buffer load and print-and-space operation delays inherent in the physical hardware.  For example, output of lines with more characters takes longer than output of lines with fewer characters, and spacing six lines takes approximately six times longer than spacing one line.

Optimized timing reduces operation delays to the minimums necessary to operate correctly; this is much faster than a real line printer would operate.

The delays used by the simulation in FASTTIME mode may be set via the register interface.  The values may be adjusted as necessary to work around any HP 3000 software problems that are triggered by the unusually rapid print operations.  Resetting the device with the RESET –P (power-on reset) command restores the original optimized times.

The CHANNEL option connects the LPA translator to the GIC with the specified IMB channel address from 2-15.  The address determines the specific GIC to which the translator will respond.  The BUS option sets the HP-IB address of the translator.  Bus addresses range from 0-7, and each address must be unique before program execution begins.  More than one LPA instance can be connected to the same GIC, provided the bus addresses assigned to the translators are unique.

Device configuration may be displayed with the following commands:

| Command | Action |
| --- | --- |
| SHOW LPA | Display the device and unit configuration |
| SHOW LPA CHANNEL | Display the channel assignment |
| SHOW LPA BUS | Display the bus address assignment |

### Unit Options

Unit options that may be specified are:

| Option | Action |
| --- | --- |
| 2611 | Set the printer model to 2611 |
| 2613 | Set the printer model to 2613 |
| 2617 | Set the printer model to 2617; default |
| 2618 | Set the printer model to 2618 |
| 2619 | Set the printer model to 2619 |
| VFU | Install the HP standard 66-line VFU tape; default |
| VFU=<filename> | Install a custom VFU tape image |
| OFFLINE | Set the printer offline; default when detached |
| ONLINE | Set the printer online; default when attached |
| EXPAND | Write expanded output to the printer image file; default |
| COMPACT | Write compact output to the printer image file |

The 2611, 2613, 2617, 2618, and 2619 options select the printer model.  Each printer is configured with Option 001, which provides 96-character sets.  The 2611, 2618, and 2619 support 132-character print line lengths, while the 2613 and 2617 support 136-character lines.  Exceeding the line length on all printers discards the excess characters.

The VFU option configures the printer’s vertical format unit, as described in the Vertical Format Unit section below.

The OFFLINE and ONLINE options place the printer offline and online, respectively.  The former provides a convenient method of setting the printer "down" without detaching the associated output file.

The printer will not go offline if there are characters in the print buffer.  Instead, the offline condition is held off until the line is printed and paper movement is complete.  The SET LPA OFFLINE command checks for data in the print buffer or a print operation in progress.  If either condition is true, the action is deferred, and Command not completed is displayed on the simulation console.  A SHOW LPA command will show that the device is still online.  Once simulation is resumed and the print operation completes, the printer is set offline.  No console message reports this, although a subsequent SHOW LPA command will indicate the new status.

Entering a SET LPA ONLINE command while an offline or detach action is deferred will cancel the action without triggering any programmed online-to-offline or offline-to-online status transition interrupt.  A RESET LPA command also cancels any deferred offline or detach action.  Additionally, it clears the print buffer and terminates any print action in progress, so a SET LPA OFFLINE or DETACH LPA will succeed if issued afterward.

The EXPAND and COMPACT options control the format of lines written to the printer image file.  In compact mode, a carriage-return/line-feed character pair terminates a printed line, but subsequent line spacing is performed by line-feed characters alone.  A top-of-form request will emit a form-feed character instead of the number of line-feeds required to reach the top of the next form.  This mode is suitable for sending the printer output to a physical printer connected to the host.

In expanded mode, paper advance is handled by emitting the correct number of carriage-return/line-feed pairs.  This mode is suitable for retaining printer output as a text file.

All of the printers are capable of overprinting.  If the printer is sent a format code to suppress spacing, the characters in the buffer are printed and the buffer is emptied, but the paper is not advanced.  The next print operation will print its characters over those already present on the paper.  In simulation, overprinting is performed in one of two ways, depending on the print mode.

In compact mode, overprinting is simulated by emitting a carriage-return character at the end of the line.  In expanded mode, overprinting is simulated by merging characters in the buffer before writing them as a single line in the printer image file.  As the second and subsequent lines are output, each new character is compared with its corresponding character in the buffer.  If the character in the buffer is a space, the new character replaces it.  If the new character is a space or is the same as the character in the buffer, the character in the buffer is retained.  Otherwise, the character in the buffer is replaced with a special character representing an overprinted combination.  This character defaults to DEL (octal 177) but may be changed by altering the OVPCHR register value.

### Vertical Format Unit

All printers support 12-channel VFUs.  A continuous punched paper tape that controls paper spacing is installed in the VFU reader.  The printer may be commanded to advance the paper until a punched hole is detected in a specified VFU channel.  The length of the tape establishes the length of the forms loaded into the printer.

Initially, the standard VFU tape (part number 02613-80001) is installed.  This tape associates channels 1-8 with the following printer actions:

| Channel | Printer Advances to |
| --- | --- |
| 1 | Top of form |
| 2 | Bottom of form |
| 3 | Next single space |
| 4 | Next double space |
| 5 | Next triple space |
| 6 | Next half-page |
| 7 | Next quarter-page |
| 8 | Next sixth-page |

Channel 9 is punched the same as channel 2, and channels 10-12 are uncommitted.

The simulator supports the use of custom VFU tape images.  A custom tape may be installed in place of the standard tape by issuing the SET LPA VFU=<filename> command.  Custom tapes must reserve channel 1 for the top-of-form location, but the other channels may be defined as desired.

A custom tape image is a plain-text file that starts with a VFU definition line and then contains one channel-definition line for each line of the form.  The number of channel-definition lines establishes the form length.

A semicolon appearing anywhere on a line begins a comment, and the semicolon and all following characters are ignored.  Zero-length lines, including lines beginning with a semicolon, are ignored.  Note that a line containing one or more blanks is not a zero-length line, so, for example, the line “ ; a comment starting in column 2” is not ignored.

The first (non-ignored) line in the file must be a VFU definition line of this exact form:

VFU=<punch characters>,<no-punch character>[,<title>]

...where:

- punch characters is a set of one or more characters used interchangeably to represent a punched location.

- no-punch character is a single character representing a non-punched location.

- title is an optional description that is printed by the SHOW LPA VFU command; the description “Custom VFU” is used if the title is omitted.

If the VFU line is missing or not of the correct form, then Format error is displayed on the simulation console, and the VFU tape is not changed.

The remaining lines define the channels punched for each line of the printed form.  The line format consists of a sequence of punch, no-punch, and “other” characters, in channel order.  Each punch or no-punch character defines a channel state, starting with channel 1 and proceeding left-to-right until all channels for the VFU are defined; any the extra channel states on the line are ignored.  If the line terminates before all channels are defined, the remaining channels are set to the no-punch state.  Any "other" characters, i.e., neither punch characters nor no-punch characters, are ignored and may be used freely to delineate the tape channels.

For a standard 66-line form, the first printable line (form line 1) is paper line 4, and the last printable line (form line 60) is paper line 63.  The first channel-definition line in the file specifies form line 1, i.e., the top of the form, and must have a punch in channel 1.  Channel line 60 defines form line 60, i.e., the bottom of the form.  Six more channel lines follow: three for the bottom margin, and three for the top margin of the next form.

An example custom VFU file using a 66-line tape definition for eight of the twelve VFU channels is:

; the VFU definition
;
; the set of punch characters is “1” and “X”
; the no-punch character is “0”
; all other characters are ignored

VFU=1X,0,A binary tape image

; the channel definitions

1 0 1 1 1 1 1 1   ; top of form (line 1; must have a punch in channel 1)
0-0-X-0-0-0-0-0   ; single space (line 2)
0011              ; channels 5-8 default to no-punch (line 3)
[...]
0 1 1 0 0 0 0 0   ; bottom of form (line 60)
0 0 0 0 0 0 0 0   ; bottom form margin (line 61)
0 0 0 0 0 0 0 0   ; bottom form margin (line 62)
0 0 0 0 0 0 0 0   ; bottom form margin (line 63)
0 0 0 0 0 0 0 0   ; top of form margin (line 64)
0 0 0 0 0 0 0 0   ; top of form margin (line 65)
0 0 0 0 0 0 0 0   ; top of form margin (line 66)

If a custom tape has been used, the standard tape may be reinstalled by issuing the SET LPA VFU command.

Attempting to command an advance to a channel that is not punched will cause a tape fault, and the printer will go offline.  Setting the printer back online will clear the fault.

The current VFU definition may be displayed with the following command:

| Command | Action |
| --- | --- |
| SHOW LPA VFU | Display the currently loaded VFU tape definition |

This command displays the current VFU tape title and then the channel definitions for each form line.  By default, a punched channel is indicated by an “O” character, and an unpunched channel is indicated by a period (“.”).  These characters may be changed by depositing new values into the PUNCHR and UNPCHR registers, respectively.

### Tracing

The LPA device supports configuration of the trigger and store specifications.  The trigger sources for the TRACE TRIGGER LPA command are:

| Source | Value | Source Description |
| --- | --- | --- |
| OUTPUT | quoted-string | Characters printed |
| FORMAT | format-code | Paper format command issued |

The OUTPUT source triggers when the quoted-string characters are sent to the printer.  Upper and lower case characters are distinct.  The syntax of quoted strings is described in the Quoted String section of the SIMH Users' Guide Supplement.

The FORMAT source triggers when the paper format code is sent to the printer.  The format code is entered as a numeric value in the device data radix, which defaults to octal but may be set to a different radix or overridden by a switch on the command line.  See the Numeric Display and Entry section for details.  The valid format codes in octal are:

| Code | Meaning |
| --- | --- |
| 000 | Suppress spacing after printing |
| 001-017 | Slew 1-15 lines after printing |
| 100-113 | Slew to VFU channel 1-12 after printing |

The storage states that may be configured for the TRACE STORE LPA command are:

| State | Implicit | State Description |
| --- | --- | --- |
| CMD |  | Printer and bus controller commands executed |
| INCO |  | Printer and bus controller command initiations and completions |
| XFER |  | Characters and format commands sent to the printer |
| BUF |  | Bus data buffer filling and emptying |
| STATE |  | Bus controller state changes executed |
| SERV |  | Printer and bus controller unit service events |

The CMD state traces the commands executed by the printer and bus controller.  The INCO state traces the beginning and ending of commands, including the resulting DSJ status.  The XFER state traces the characters and format commands sent to the printer.  The BUF state traces the filling and emptying of the bus data buffers.  The STATE state traces the changes between the internal bus controller execution states.  The SERV state traces printer and bus controller unit event scheduling and service entries.

Omitting the list of storage states in a TRACE STORE LPA command uses the indicated set of implicit states.

Examples of the trace formats follow:

**LPA   cmd: Printer paper loaded
**LPA   cmd: Device 0 Amigo Identify 200AH executing
**LPA   cmd: Device 0 Device Specified Jump 0 executing
**LPA   cmd: Device 0 Print Data executing
**LPA   cmd: Printed 132 characters on line 8
**LPA   cmd: Printer commanded to slew to VFU channel 1 from line 9
**LPA   cmd: Printer advanced 58 lines to line 1

**LPA  inco: Device 0 Set Escape started
**LPA  inco: Device 0 Set Escape completed with DSJ 0

**LPA  xfer: Character 'M' sent to printer
**LPA  xfer: Format code 100 sent to printer

**LPA   buf: Buffer 1 filled (62/128 bytes)
**LPA   buf: Buffer 2 filled (1/128 bytes)
**LPA   buf: Buffer 1 emptied
**LPA   buf: Buffer 2 emptied

**LPA state: Device 0 parallel poll response enabled
**LPA state: Device 0 transitioned from Command Ready to Parameter Wait state
**LPA state: Device 0 transitioned from Parameter Wait to Completion Wait state
**LPA state: Device 0 transitioned from Completion Wait to Command Ready state

**LPA  serv: Device 0 bus unit state Command Send delay 2 service scheduled
**LPA  serv: Device 0 bus unit state Command Send service entered
**LPA  serv: Device 0 printer unit print delay 1643 service scheduled
**LPA  serv: Device 0 printer unit service entered

### Registers

The Line Printer Controller state contains these registers:

| Name | Size | Radix | Symbolic | Read-Only | Description |
| --- | --- | --- | --- | --- | --- |
| DEVS | 8 | 2 |  |  | Present devices address bitmap |
| XFER | 1 | 2 |  |  | Data transfer across channel is active |
| PP | 1 | 2 |  |  | Parallel poll is in progress |
| PPR | 8 | 2 |  |  | Parallel poll responses address bitmap |
| CSTATE | 4 | 10 |  |  | Bus controller execution state |
| CMD | 4 | 10 |  |  | Bus controller command |
| DSJ | 2 | 10 |  |  | Device Specified Jump result |
| COUNT [0:7] | 8 | 10 |  |  | Bus data buffer counts |
| PARSER | 3 | 10 |  |  | Escape parser execution state |
| STATUS | 8 | 2 |  |  | Printer status |
| PFAULT | 1 | 2 |  |  | Paper fault exists |
| TFAULT | 1 | 2 |  |  | Tape fault exists |
| OLPEND | 1 | 2 |  |  | Offline transition is pending |
| PRLINE | 8 | 10 |  |  | Current print line number |
| PRTBUF [0:423] | 8 | 8 |  |  | Print buffer |
| BUFIDX | 8 | 10 |  |  | Current print buffer index |
| OVPCHR | 8 | 8 |  |  | Overprint character |
| FORMLN | 8 | 10 |  |  | Form length in lines |
| PUNCHR | 8 | 8 |  |  | Punched channel character |
| UNPCHR | 8 | 8 |  |  | Unpunched channel character |
| BTIME | 24 | 10 |  |  | Fast printer buffer load delay time |
| PTIME | 24 | 10 |  |  | Fast printing delay time |
| STIME | 24 | 10 |  |  | Fast paper slew per-line delay time |

The PRTBUF, OVPCHR, PUNCHR, and UNPCHR registers default to single-character format display and entry but may be overridden with numeric-format switches, if desired.